3GPP TSG-WG SA2 Meeting #166	S2-2412319

Orlando, USA, 18th – 22nd Nov, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Analyses on the architecture and functional split

Document for:	Discussion

Agenda Item:	19.4

Work Item / Release:	EnergySys / Rel-19

Abstract: This paper discusses and evaluates the architecture and functional split for the energy related information collection.

# 1. Introduction

In SA2#165 meeting, the basic architecture for enabling for energy related information collection and energy saving was approved. There are several issues to be further discussed regarding the details of the architecture, namely the services/styles used for information collection and calculation, and whether the distributed architecture can be used.

This document addresses the issues mentioned above.

# 2. Discussion

## 2.1	Centralized and hierarchical approach for energy evaluation

S2-2410425 explains the two possible architectural candidates for energy evaluation, namely:

-	Hierarchical approach which assumes that the SMF is enhanced to handle the finer granularity, i.e., PDU Session and QoS Flow, for the calculation of the energy consumption. The new Network Function EECF is in charge of UE and application granularities and to forward the relevant OAM information to the SMFs. The SMF reports the results of the energy consumption calculation for the PDU Session to the EECF.

-	Centralized approach which assumes the EECF acts as a centralized function that calculates the energy consumption for all different granularities. The SMF reports all data volume measurements for the PDU Session and the identifiers of the corresponding RAN nodes, I-UPFs and UPFs to the EECF.

As analysed in S2-2410425, it can be seen that:

-	There will be a tight integration in the PCC framework and the corresponding SMF and UPF functionality;

-	It is necessary to keep track of the data volume that has been transferred by each of the involved RAN nodes, I-UPFs (if in use) and UPFs;

-	There are also a couple of more complicated user plane scenarios/configurations that would be difficult to report from SMF;

-	And while the usage of energy consumption information for charging purposes may not be achievable in Rel-19, it can be expected that this will come in Rel-20 and the SMF would then need the energy consumption information for the PDU Session in order to forward it to the CHF;

-	Scalability can be an issue if energy evaluation is done for many UEs at the same time. It would then be beneficial to keep the measured data volumes locally at SMF and to expose only the calculation result.

-	Other enforcement actions that are based on energy consumption information, like energy credit or maximum energy consumption threshold, will also require that energy consumption information for the PDU Session is available at the SMF.

As such, there are several good reasons for involving the SMF in the energy consumption calculation process and to give the SMF the responsibility for the calculation of the PDU Session (and smaller) granularities.

Proposal 1: Therefore, the hierarchical approach (i.e., SMF calculates/evaluates the EnC or other energy related information for PDU Session (and smaller) granularities) shows more flexibility and future-proofness, and therefore it is suggested to use this approach for the architecture.

## 2.2	Options of the design principle on Architecture

As discussed in SA2#165 meeting, besides designing the brand-new functionalities (new design) for energy information collection and distribution, there is also one other possibility: to combine the energy related new functionality to the existing NWDAF, or following design principle of NWDAF (NWDAF-based), since the data collection functionality of NWDAF can be re-used.

Generally speaking, both options works properly but it would be better to clarify the following issues before determining any of the option.

-	Details of the two options. While the new design option is more or less starting from scratch, the NWDAF-based option is proposing to design the services, framework and functionalities following the NWDAF with some enhancements. But currently it is not very clear for the NWDAF-based option: whether it is fully reuse, or just borrowing the idea. In other words, the relationship between the newly-introduced EIF and NWDAF for the NWDAF-based option is TBC. Note that having EIF implemented within NWDAF means NWDAF is mandatory when deploying the energy related feature.

-	Whether the energy related control should be supported in this release. The main functionalities of EIF in this release contains information collection, calculation, and exposure. NWDAF related functionality already covers those. However, as highlighted in the TR conclusion for KI#2, the PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions, and such information is largely from EIF at the end. Therefore, the EIF should also assume the work of notification related to the energy related information. Therefore, it is important to know whether and how the control could be supported by the EIF. Note that the control information other than EnC itself is beyond the functionalities of NWDAF, and it might not be very useful of simply provide EnC to PCF from EIF, since the PCF may not be able to know the EnC is high/low/exceptional.

-	Future proofness. Considering there might be continuous work in the future releases, it is crucial to consider whether the design in this release would limit the future design. In this regard, using NWDAF-based option is less persuasive, since combining the EIF and NWDAF would limit the design of energy related functionality in the future, e.g., functionalities beyond collection/exposure like control, charging related.

Proposal 2: By considering the issues above, the new design is preferred, while the author also thinks it is possible to borrow the idea of the services for data collection/exposure already defined for NWDAF.

In addition, it is also beneficial to consider the following details as well to accelerate the discussion during the meeting.

-	Data collection. There is one opinion of reusing the function defined in TS 23.288 clause 5A on Data Collection Coordination and Delivery, as the DCCF is already there and designed for efficient data collection/coordination for multiple requests from NF consumers.

However, mandating DCCF's role in EnC estimation is not that necessary since:

-	For calculating EnC, the information required by EIF is mainly from OAM (i.e., DV, EnC of RAN/UPFs), and the Data Volume UL/DL per PSA UPF and per RAN node from SMF. There is not much that DCCF can do, since the interaction between EIF and OAM is more likely to be node-level, and the DV from SMF is better to be collected by directly interact with SMF.

But we can still borrow the idea of DCCF service when designing the EIF to save the discussion time.

-	Data storage. The interface of ADRF is designed as service-based, and technically any 5GC NF can invoke the provided service, therefore the storage is not an issue. However, one needs to consider what information to be stored before going to the ADRF option. If EnC info is relevant for network internal/operator logic for energy saving, the EIF is temporarily storing/aggregating until the next decision is taken. If EnC info is relevant for exposure to AF, the EIF should temporarily store until the next exposure event. Simply using EIF local storage would be enough.

In addition, no matter what option to be selected for the architecture, it is still preferable to have the data collection/estimation/exposure in TS 23.501/502 instead of TS 23.288. Since the functionalities related to energy are not only for data related processing. Note that distributing sub-features to multiple specifications will lead to a fragmented description, and it would be also hard for the subsequent maintenance.

# 3. Conclusion and proposal(s)

Based on the analyses in clause 2, we have the following proposal:

-	It is suggested to use the hierarchical approach for the architecture (i.e., SMF calculates/evaluates the EnC or other energy related information for PDU Session (and smaller) granularities) .

-	It is suggested to design the functionality of EIF independently from NWDAF, while it can be further discussed whether or not to borrow the idea of the services for data collection/exposure already defined for NWDAF.