3GPP TSG-SA WG2 Meeting #166	S2-2412343

18 - 22 November 2024, Orlando, USA                                              (revision of S2-2407469)

Title:	[Draft] Reply to LS on local BDC setup on terminating side in case INVITE does not contain DC description

Response to:	-

Release:	Rel-18

Work Item:	NG_RTC

Source:	Samsung [will be: SA2]

To:	CT1

Cc:	CT4

Contact Person:

Name:	DongYeon Kim

Tel. Number:

E-mail Address:	dy522.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2412345 [will be: CR 1407R1 (TS 23.228)]

1. Overall Description:

SA2 thanks CT1 for questions for clarification on local bootstrap DC setup on terminating side in case INVITE does not contain DC media description.

SA2 would like to answer to the CT1 questions as below.

CT1 Q1: Is the presence of DC media description in the SDP offer of an initial INVITE request from the originating network mandatory condition for the terminating IMS AS to notify the DCSF?

SA2 Answer on Q1: Yes it is mandatory. The presence of DC media description in the SDP offer of an initial INVITE request is mandatory for the IMS AS to notify the DCSF in both originating and terminating network.

CT1 Q2: If the answer on Q1 is No, the SDP offer in an initial INVITE request from the originating network does not contain DC media description and the terminating UE has IMS DC capability and subscription, can the terminating IMS AS notify the DCSF based on the operator policy?

SA2 Answer on Q2: No. If the DC media description is not contained in the SDP offer of the initial INVITE request, the terminating IMS AS does not notify to the DCSF regardless of the operator policy.

CT1 Q3: If the answer on Q1 is No and Q2 is Yes, after the terminating IMS AS notifies the DCSF, is the DCSF allowed to instruct the IMS AS to set up bootstrap data channel between the terminating network and terminating UE if the SDP offer of an initial INVITE request from the originating network does not contain DC media description?

SA2 Answer on Q3: Based on answers of Q1 and Q2, The bootstrap data channel is set up between the terminating network and terminating UE when the DC media description is contained in the initial INVITE request from the originating network, and the DCSF is allowed to instruct the IMS AS during this bootstrap data channel setup procedure.

SA2 has discussed that the P2A DC Sessions initiated by the originating UE and anchoring MF in terminating network is supported based on TS 23.228 in Rel-18 with DC media description contained in SDP offer in INVITE request from the originating UE. This is clarified with the attached TS 23.228 CR agreed in SA2.

The DC Sessions initiated by the terminating network without INVITE requests from the originating side (no DC description in INVITE) will be defined in SA2 Rel-19 NG_RTC_Ph2 normative phase.

2. Actions:

To CT1

ACTION: 	SA2 kindly asks CT1 to take into account the above answers.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #165		14-18 October 2024		Hyderabad , IN

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, US