SA WG2 Meeting #166	S2-2412357

November 18 – 22, 2024, Orlando, FL, USA

Source:	Ericsson

Title:	Way Forward based on IETF MASQUE: Reply LS on XRM metadata exchange between 3GPP Core and an Application server

Document for:	Agreement

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract of the contribution: This contribution provides input to discussion on the LS in S2-2412356.

# Background

The IETF Masque working group replied to the LS on XRM metadata exchange between 3GPP Core and an Application server in S2-2409443 sent from the SA2#164 meeting with a set of comments from individuals as per RFC 4053, section 3.2.2.2.

# Discussion

In summary, one of the comments states that the solution will not work, but that seems to stem from a misunderstanding on the involvement of the AS and the ability to control the maximum UDP payload size. Other solutions comment on the perceived efficiency issues and the lack of full endpoint support. None of the other comments state that the solution does not work. Several comments mention the IETF SCONE working group, but it should be noted that this WG is setting out to solve a different problem (communicating network bitrates from on-path network elements to endpoints). While the solutions defined in SCONE could be generalized to fit the use-case of XRM they do not provide encryption and integrity protection of the data.

Several comments are likely stemming from an incomplete understanding of the entire solution. Specifically, it is missing that an API between the server and network is used.

It is very important to note that this entire proposal is designed for a 3GPP context where constraints and requirements can be set that do not normally apply to protocols and solutions designed for the general Internet. This includes restrictions on MTU sizes and communication between application endpoints and the network via APIs.

The comments on endpoints driving the solution are well received and can be addressed by the fact that the AS is in fact involved. For instance, it is possible to define a more active role for the AS in the CID registration, to fully remove any potential CID size ambiguities.

Observations for the comments in LS Response follow:

Comment 1:

I don't think the proposed architecture will work, nor would it be consistent with IETF standards.  It would also be less efficient than other options available to 3GPP.

When processing QUIC packets without acting as a QUIC endpoint, standards-compliant processing is limited to the invariants documented in RFC 8999.  Nothing in that RFC promises that Connection ID lengths will be fixed for the duration of a connection.

Observation: The solution does not require connection Ids to have a single length throughout the duration of a connection. New connection IDs that are longer than original connection IDs are going to be registered successfully. New connection IDs that are shorter than the original connection ID can lead to a fallback to tunnelled mode.

In the proposed design, the UPF decides whether each packet is tunneled or forwarded, and the UE is not aware of which path was taken.  This prevents the UE from correctly implementing Path MTU detection, as these different paths are likely to have distinct MTUs.

Observation:

A 3GPP network can enforce an MTU size that will accommodate for any overhead, therefore there will be no distinct path MTUs. See TS 23.501 clause 5.6.10.4.

The server is an active participant in the metadata exchange and can therefore control the maximum QUIC packet size for the end-to-end connection to accommodate for the expected overhead. The server should send the max_udp_payload_size transport parameter with an appropriate value to the client in the end-to-end connection.

The proposal mentions defining a new Packet Transform that adds metadata to each packet.  This indicates that "zero overhead" forwarding is not a requirement.

If only UDP flows are considered relevant, I would recommend implementing the forwarding function using a trivial UDP forwarder such as SOCKS5-UDP (RFC 1928).  This avoids any reliance on the details of QUIC, removes the need to double-encrypt long-header packets, and prevents the issues I've mentioned above.

Observation:  Using a trivial UDP forwarder does not solve the issue with protecting the metadata, also SOCKS does not support per-packet metadata.

--

Comment 2:

I tend to agree...  Anything like this is only likely to add overheads, in the form of additional encapsulation bytes, additional encryption effort, and additional delays.  Establishing a case for that additional would require fairly extraordinary justification and should come with a comparative analysis against alternatives (anything driven by endpoints would be strong candidates).

Observation: This solution is endpoint driven, the Masque proxy is located is tightly integrated with the Application Server (AS). Forwarding mode reduces drastically any additional bytes, basically to those needed to convey the actual metadata in a protected way.

There also appears to be an overlap here between the ideas that seem to be driving the request and the work in the newly-formed SCONE working group.

Observation: There is a requirement on protecting the metadata since the metadata exposes quite detailed application state. The main difference between this work and the work in the SCONE WG is that SCONE is used to expose limited network information to endpoints, which is a different use case with a different set of requirements. Furthermore, SCONE has not yet produced any specifications.

--

Comment 3:

I would propose a reply somehow like this:

Thanks for reaching out to the MASQUE working group.

We would like to note that technical questions as contained in your liaison statement are best addressed directly to the MASQUE mailing list [1]. All IETF mailing are open and anybody can post questions like this. This will ensure the most rapid reply to you matters and will provide you more detailed expert input. Further, note that inline with RFC4052 [2] these kind of “informal channels” are the preferred way of communication for the IETF generally.

With respect to your questions, RFC9000 [3] and RFC9298 [4] specify the following: Connection IDs in the QUIC header are not encrypted. A client that is aware of QUIC connection IDs can register them using REGISTER_CLIENT_CID and REGISTER_TARGET_CID capsules as defined in draft-ietf-masque-quic-proxy [5]. If a received UDP datagram does not contain a registered QUIC connection ID, the UDP payload is sent in tunnelled mode. We don’t have any further comments on the mechanism under specification by 3GPP.

As noted above, please feel free to reach out for any further questions directly to the masque mailing list [1].

Observation: This comment confirms several aspects of the solution.

--

Comment 4:

Agreed that a solution driven by the endpoints would be preferable here, and that this seems like it needs to coordinate with what SCONE will work on.

Observation: This solution is endpoint driven; the Masque proxy is tightly integrated with the Application Server (AS). See above comments about SCONE.

# Proposal

Based on the response LS and the observations provided above, we propose to introduce support for QUIC aware proxying in the solution to KI#2 as documented in the study conclusions. We suggest adding support for sending end-to-end connection ID information in capsules from AS to UPF. The AS can update the UPF with this information whenever it becomes available. The UPF can decide to use these values for CID registration. This enhancement reduces the risk of fallback to tunnelled mode when connection IDs of different sizes are used throughout the duration of an end-to-end connection. Hence improving efficiency in these conditions.