3GPP TSG SA WG2 #166	            S2-2411332

18 - 22 November 2024, Orlando, USA 	(revision of S2-2411010)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1480 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Architecture for avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, [ZTE, vivo, Huawei, HiSilicon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#8 conclusion in TR 23.700-77, the architecture for avatar communications needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of IMS DC based avatar communication is introduced. [R04: SA2#166] - Editor’s Notes in clause AC.X.1 General is cleared. - Note regarding DC AS URL is clarified. - Function description of IMS AS is added in clause AC.X.2. - Reference points are added in clause AC.X.2. - Procedures are newly added as clause AC.X.3. - Service operation parameter is added in clause AA.2.4.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS DC based avatar communication cannot be supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.2.2, AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list, Indication of avatar communication and BAR access allowed.

Calling ID is the public identity of the calling IMS subscriber.

Called ID is the public identity of the called IMS Subscriber.

Session case indicates if this is an originating or terminating IMS session.

Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'.

Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Indication of avatar communication and BAR access allowed indicates that the network is allowed to access to BAR for the user. This indication may be sent with Calling ID. See clause AC.X.3.1.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list, Calling ID, Called ID, Session case |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |

# * * * * Second change (all text is new) * * * *

AC.X	Support of Avatar Communication

AC.X.1	General

This clause describes the enhancements to the IMS architecture supporting data channel services to additionally support Avatar communication services.

For Avatar communication over IMS data channel, the list of Avatar ID(s) may be locally configured in the UE or fetched from the Base Avatar Repository (BAR) by DCAS, and is transferred to the UE through bootstrap data channel or application data channel.

Avatar ID List contains the list of Avatar IDs that are identifying the Avatar Representation allowed for the user, the metadata associated with the Avatar (e.g. the data required for the user to select one Avatar to use in a call such as thumbnails, location, time, country, etc), and the asset information (e.g. the personal assets such as accessesories, guarmets).

When the Avatar ID List is downloaded through bootstrap data channel, the DCSF gets the DCAS address to access to the BAR from DC service specific data stored in the HSS or locally configured.

After selection of Avatar ID to use in the avatar call by the user of the UE, the Avatar Representation identified with the selected Avatar IDs is provided from the BAR to MF, and it is transferred to the local UE (if not locally available) and/or to the remote UE according to the rendering mode via application data  .

The avatar media is rendered by processing the Avatar Representation with the animation data.

-	The avatar media rendering process may be performed by the local UE, the remote UE, and/or the network according to the rendering mode.

-	The animation data may be generated by the local UE, the remote UE, or the network (e.g. from audio, video, and/or metadata such as action and facial expression data).

The details of enhanced IMS data channel architecture and procedures are described in following sub-clauses.

AC.X.2	Architecture

Figure AC.X.2-1: Architecture to support Avatar communication

To support Avatar communication, the data channel architecture and functions described in clause AC.2 is enhanced as below:

Base Avatar Repository (BAR):

-	The BAR stores Avatar Represenation and its associated Avatar ID. One or more Avatar Representation(s) is stored for a user, and each Avatar Representation is identified with Avatar ID.

-	The BAR may provide Avatar ID List to the DC Application Server, and the DC Application Server may further provide to DCSF.

-	The BAR sends Avatar Representation to the MF directly.

Editor's note:	The interface(s) between the BAR and the DC Application Server is FFS.

DCSF:

-	The DCSF may request the Avatar ID List to DCAS to fetch it from the BAR, and sends it to MF via MDC1.

MF:

-	The MF sends the Avatar ID List to the UE through bootstrap data channel or application data channel.

-	The MF receives the Avatar Representation from the BAR by using MDC4.

-	The MF supports Avatar media processing and transcoding (e.g. ASR/TTS).

DC AS:

-	The DC AS retrieves the Avatar ID List from BAR and may send it to DCSF via DC3 or DC4 during bootstrap data channel setup procedure for Avatar communication.

-	The DC AS supports Avatar media negotiation and provides the instruction for Avatar media processing.

IMS AS:

-	The IMS AS determines whether the user of the UE is allowed to use the data channel based avatar communication and whether it is allowed to access to the BAR for this user of the UE. If allowed, the IMS AS indicates to the DCSF about the BAR access allowance with the user identity.

The following reference points are used and enhanced to support Avatar communication service over IMS data channel:

-	DC2: Reference point between the IMS AS and the DCSF for checking whether the use of avatar call and BAR access are allowed for the user.

-	DC3/N33: Reference point between the DCSF and the NEF, between the NEF and the DCAS for Avatar ID List download.

-	DC4: Reference point between the DCSF and the DCAS for Avatar ID List download.

-	MDC1: Reference point between the DCSF and the MF for Avatar ID List transfer over bootstrap data channel.

-	MDC3: Reference point between the DCAS and the DCSF for Avatar ID List transfer over bootstrap data channel.

-	MDC2: Reference point between the DCAS and the MF for Avatar ID List transfer over application data channel.

-	MDC4: Reference point between the BAR and the MF for Avatar Representation transfer.

-	Mb: Reference point used for Avatar ID List and Avatar Representation transfer.

AC.X.3	Procedures

AC.X.3.1	Bootstrap DC setup for Avatar communication

Figure AC.X.3.1-1 depicts the Boostrap DC setup signalling procedure which includes the downloading of list of Avatar IDs that are stored in the BAR and allowed to use for the user of the UE.

Figure AC.X.3.1-1: Bootstrap DC setup signalling procedure for Avatar communication

The steps in the call flow are as follows:

- UE#1 sends the SIP INVITE request as described in the step 1 of clause AC.7.1

- IMS AS validates user subscription data to determine whether the data channel call request needs to be notified to DCSF as described in the step 2 of clause AC.7.1. Based on the subscription data, the IMS AS determines whether the user of the UE#1 is allowed to use DC based avatar call and the network is allowed to access to the BAR for this user.

- IMS AS notifies the DCSF of the DC call event to the DCSF as described in the step 3 of clause AC.7.1. If it is determined as allowed in step 2, the IMS AS includes the Indication of avatar communication and BAR access allowed, and Calling ID in Nimsas_SessionEventControl_Notify message.

Based on the Indication of avatar communication and BAR access allowed, and Calling ID, the DCSF stores in the context that the Avatar ID List request through bootstrap data channel is allowed for the user of the UE#1. This context is used in step 7.

- Step 4 to 20 of Bootstrap Data Channel Setup Signalling procedure described in clause AC.7.1 applies.

- Bootstrap DC Establishment, DC Application list download, dedicated DC Application request, and download of DC Application are performed based on the result of step 4. The UE#1 downloads the avatar call DC application.

- The UE#1 requests the Avatar ID List for the user of the avatar call DC application through bootstrap data channel. The request is sent to DCSF via MF.

- The DCSF identifies the session which is associated with the boostrap channel that the Avatar ID List is requested and verifies whether the avatar call and the BAR access is allowed for the user of the UE#1 based on the context stored in the step 3.

- If it is verified in step 7, the DCSF checks the DCAS address for BAR access with the user identity of UE#1 (e.g. Calling ID) and the DC application ID which were included in the Avatar ID List request. The DCSF gets the DCAS address from the DC service specific data stored in HSS or locally configured.

- The DCSF requests to the DCAS to fetch the Avatar ID List for the user of the avatar call DC application. In the request, the DCSF includes the user identitiy (e.g. Calling ID, GPSI) of the UE#1. If the DCAS is in the untrusted domain, the request is sent via NEF.

- The DCAS requests to the BAR the Avatar ID List that is allowed for the user of the avatar call DC application. In the request, the DCAS includes the user identity (e.g. Calling ID, GPSI) of the UE#1.

- The BAR responses to the DCAS the Avatar ID List that is allowed for the user of the avatar call DC application.

- The DCAS responses to the DCSF the Avatar ID List for the user of the avatar call DC application. If the DCAS is in the untrusted domain, the request is sent via NEF.

- The Avatar ID List for the user of the avatar call DC application is delivered to the UE#1 through bootstrap data channel. The response is sent from DCSF via MF.

The user of the UE#1 selects an Avatar to use in the avatar call with the UE#2, based on the metadata associated with each Avatar ID within the Avatar ID List. The selection of an Avatar to use is conducted before and/or after application data channel setup for an application.

When it is selected before application data channel setup, the UE#1 requests the Avatar Representation of the selected Avatar to the MF with the Avatar ID and the destination of the Avatar Representation downloading (i.e. local UE, remote UE, or both) via bootstrap data channel, then the MF requests to the BAR. The BAR provides the Avatar Representation to the UE#1 and/or to the UE#2 based on the destination indicated in the requests.

The Avatar to be used in the avatar call with the UE#2 may be selected or re-selected after an application data channel setup for an avatar communication application, e.g. based on each avatar communication application specific configurations such as the limited rendering modes, the limited Avatars, the limited usage in certain location/country/time. In this case, the UE#1 sends the requests for downloading Avatar Representation through the application data channel.

# * * * * End of changes * * * *