SA WG2 Meeting #166	S2-2411342

# 14 - 18 November 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 10 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinements of NWDAF initiated VFL training procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile, CATT], ZTE, [OPPO, China Telecom, ETRI, Huawei, HiSilicon, Nokia, Futurewei, Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As outlined in S2-2411194, we have reached an agreement on the VFL training process procedures, this CR proposes to update and clean up some related ENs. Remove the following ENs: Editor’s Note: It is FFS if the term “backward local ML model training information” needs to be changed into “intermediate results”. Since the backward local ML Model training information contains gradient information or loss information, which is same as intermediate result, we prefer to keep backward local ML Model information naming. Editor’s Note: Whether weight of feature is computed by VFL server is FFS. Editor’s Note: Whether VFL server and VFL clients share feature information is FFS. The features are vendor-specific information and we prefer not to share it with another vendor in any VFL related procedure. Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS The VFL server determines to stop the VFL process not VFL clients. So VFL clients do not need to know the convergence report. Editor’s Note: It is FFS which service is used between VFL server and VFL clients for VFL training. The EN above is covered by the third EN, so we propose to remove it to avoid duplication. Editor’s note: The report may include ID(s) of sample(s) corresponding to the intermediate training result. We propose to add sample ID along with intermediate result that to be reported by client to server. This will help VFL server identify the sample used by VFL Client in the current iteration, compute loss per sample and send back this information to client for local model update. The EN above is deleted correspondingly. Propose to add the maximum response time in the initial request sent by VFL server to VFL client. To avoid the VFL training process taking long time. Editor’s Note: Whether VFL Training termination Flag in the termination message is required is FFS. This EN is modified to below: Editor’s Note: Whether VFL Training termination Flag in the termination message is required will be determined after we decide the service operation used in VFL training process. Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS. The latest ML Model information and VFL correlation ID are stored by VFL server and VFL client to perform joint inference later; The VFL client and VFL server can also optionally store the ML Model to ADRF, so the EN above is deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Formatting corrections and editorial modifications. EN clean ups. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2H.2.3 Training Procedure for Vertical Federated Learning

6.2H.2.3.1 	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1 Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server.

Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical re-selection, addition, or removal of VFL Client NWDAF(s) is FFS.

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS. The details of services used in the VFL training procedure is FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided in each training iteration.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE: VFL Server may determine to perform VFL training based on the internal trigger or local configuration.

The steps 2-6 are repeated until the training termination condition is reached.

2.	To start VFL training, the VFL server allocates VFL correlation ID. The VFL server sends a request to start the VFL training to each of the selected VFL clients. The VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF clients(s) and Naf_VFLTraining_Subscribe or Naf_VFLTrainingInfo_Request to the selected AF clients(s). The request includes VFL correlation ID, VFL capability type (VFL Client). The request may also include maximum response time(i.e. the maximum time between VFL clients receive backward local model information and send back intermediate training result).

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the backward local ML model training information to each of the VFL clients for next round of VFL training.

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and based on possible backward local ML model training information distributed by the VFL server in the previous training iteration, and computes and reports the client intermediate training result of the local ML model to the VFL server. The report may include sample ID(e.g. UE ID) corresponding to each intermediate training result..

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the backward local ML model training information (e.g. gradient information or loss information) based on the client intermediate training result received in step 4, its own local intermediate results and the label. The backward local ML model training information is used for updating the models of VFL clients. Different backward local ML model training information may be computed for different VFL clients, respectively.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g., based on the convergence of a loss function or loss value, the pre-set iteration number is reached) whether VFL Training process converged. If not, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If yes, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor’s Note: The content of the VFL status report is FFS.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

Editor’s Note: The following is FFS: whether and how VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed.

7.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process. The VFL Server and each VFL Client, stores the VFL correlation ID and latest information related to their locally trained Models.The VFL Sever(i.e. NWDAF) and VFL Client (i.e. NWDAF and trusted AF) may optionally store the latest ML Model information to ADRF.

NOTE X: The VFL correlation ID may be used later for inference.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required will be determined 	after we decide the service operation used in VFL training process.

NOTE Y: The VFL server and VFL client need to locally maintain the mapping relationship between ML Model ID 	and VFL correlation ID locally if they stores the ML Model information to ADRF after VFL training process.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

* * * End of changes* * *