SA WG2 Meeting #166	S2-2411399

# Orlando, USA, 18 – 22 November, 2024                        (revision of S2-24xxxxx)

Source: 	NEC

Title: 	AIoT conclusion for Inventory QoS on Key issue#3

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes the AIoT conclusion for Inventory QoS.

1	Discussion

It is necessary to provide QoS control for the AIoT inventory service to address the application layer and network layer requirements.

AIoT QoS parameters can be provided by the service layer from AF and the network layer from the core network.

AF provides QoS parameters for AIoT inventory operation: Max time duration for inventory, guaranteed Reading-device successful rate of inventorying an AIoT device group, the AIoT device priority in the inventory group.

The core network may have AIoT device subscription for QoS: AIoT device priority in the AIoT service.

The core network makes the final decision of the AIoT device priority during the inventory, and applies the QoS control in the core network. The core network sends the QoS parameters to Reader.

The Reader applies the QoS control during the inventory.

2	Proposal

It is proposed to update TR 23.700-13 as following.

# * * * * Start of Change* * * *

## 8.3	Conclusion on Key Issue #3

### 8.3.x	Inventory QoS

AIoT QoS parameters can be provided by the service layer from AF and the network layer from the core network.

AF provides QoS parameters for AIoT inventory operation: Max time duration for inventory, guaranteed Reading-device successful rate of inventorying an AIoT device group, the AIoT device priority in the inventory group.

- Max time duration for inventory: after Reader receives the Inventory Request, the Reader is expected to send Inventory response before the max time duration time out. It restricts the max time duration for inventory to avoid too long duration e.g., when the AIoT device cannot be read.

- Guaranteed reading successful rate of inventorying an AIoT device group: it defines the lowest successful rate of reading AIoT devices in the group, when the RAN Reader receives the Inventory Request including multiple AIoT devices in the group. It enforces the Reader to guarantee the successful rate of inventory.

- AIoT device priority in the inventory group: when there are multiple devices in the group for inventory, AF may indicate the priority for different device, when Reader receives such information, it shall prioritize the high-priority device for inventory.

The core network may have AIoT device subscription for QoS: AIoT device priority.

The core network makes the final decision of the AIoT device priority during the inventory, and applies the QoS control in the core network. The core network sends the QoS parameters to Reader.

- AIoT device priority indicates the priority in the AIoT network and is considered in the Reader for inventory with different AIoT devices or groups.

The Reader applies the QoS control during the inventory.

# * * * * End of Changes * * * *