3GPP TSG-SA2 Meeting #166	S2-2411467

# 18 - 22 November 2024, Orlando , USA

Source:	China Telecom

Title:	Discussion on FS_IMS_RES outcome and future work plan

Document for:	Discussion

Agenda Item:	4.1

Work Item / Release:	FS_IMS_RES (1010003) / Rel-19

Abstract of the contribution: This contribution discusses the potential SA2 impact based on the CT4 LS S2-2409600/(C4-244485) and provides suggestion of the reply LS.

1. Background

In the LS S2-240960/C4-244485 and the attached 3GPP TR 29.866 v1.2.0, CT4 introduces outcome of the study on IMS disaster prevention and restoration enhancement (FS_IMS_RES) and future work plan as bellow:

- For KI#1: Continue service for registered users in case of HSS/UDM failure or overload, Sol#1, Sol#3 and Sol#4 are proposed to be standardized.

- For KI#2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users, Sol#5 is proposed to be standardized.

- For KI#3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails, Sol#6, Sol#7, Sol#8 and Sol#9 are proposed to be standardized.

- For KI#4: Preventing the IMS disaster in the HSS overload scenario, Sol#2 and Sol#10 are proposed to be standardized.

Table 1 in the LS S2-240960/C4-244485 depicts impacts of these FS_IMS_RES solutions, and Sol#1, Sol#2, Sol#3 and Sol#4 are evaluated to have SA2 impacts, and CT4 would kindly asks SA2 to evaluate these solutions from the side of stage 2.

Besides, CT4 also asks SA2 for view on future work plan of IMS_RES, i.e. how CT4 coordinate with SA2 on harmonizing stage 2 work in CT4-lead 3GPP TS 23.380 and SA2-lead 3GPP TS 23.228.

This paper briefly evaluates the mentioned SA2 impacted solutions, i.e. Sol#1, Sol#2, Sol#3 and Sol#4, in the incoming LS, and provides suggestion for the reply LS.

2. Discussion

2.1 	Solution #1, Solution #3 and Solution #4 for KI#1 in TR 29.866

Solution #1, as described in Clause 6.1.2 of 3GPP TR 29.866, proposes to re-use information that already captured in P-CSCF/I-CSCF/S-CSCF and optimize some signal flows, to ensures successful IMS re-registration of a registered UE in the IMS network when HSS fails or overloads:

- in case of the re-registration process without I-CSCF, P-CSCF forwards the REGISTER request directly to the S-CSCF to enable re-registration bypassing the HSS while minimally altering the existing network functionality.

- in case of the re-registration procedure for routing with I-CSCF, P-CSCF adds the S-CSCF address stored in P-CSCF to a new header field of the REGISTER request. Then, the I-CSCF forwards the REGISTER request to the S-CSCF bypassing the HSS by parsing the S-CSCF address of the new header field.

Observation 1: Solution #1 reduces querying HSS in re-registration process in case of HSS/UDM failure or overhead. SA2 needs to update Clause 4.6.1, Clause 4.6.2 and Clause 4.24 in TS 23.228 to reflect the P-CSCF/I-CSCF/S-CSCF enhancement for Solution #1.

Solution #3, as described in Clause 6.3.2 of 3GPP TR 29.866, introduces a new AS to guarantee the IMS initial registration and MT services continue for UEs registered to the IMS network in case of HSS failure or overload, which supplements Solution #1. UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.) of the new AS are synchronously derived from the ISC interface (S-CSCF-AS) directly or indirectly. During the IMS (re-)registration and MT services procedures, to ensure successful S-CSCF address retrieval in case of HSS/UDM failure, the S-CSCF address is stored in a new AS during the registration process.

Observation 2: In Solution #3 S-CSCF/I-CSCF need to be enhanced, and a new AS dedicated for HSS bypass need to be deployed. SA 2 SA2 needs to update Clause 4.24 in TS 23.228 to reflect the enhancement for Solution #3.

Solution #4, described in Clause 6.4.2 of 3GPP TR 29.866, proposes to utilize the OPTIONS mechanism to find the S-CSCF serving the subscriber for bypassing HSS, requiring the S-CSCF to delay removing all subscription information related to this specific Public User Identity in the de-registration procedure.

Observation 3: In Solution #4 S-CSCF/I-CSCF interactions need to be enhanced for HSS/UDM failure. SA2 need to update Clause 4.24 in 3GPP TS 23.228 to support Solution #4.

2.2 	Solution #2 for KI#4 in TR 29.866

Solution #2, as described in Clause 6.2.2 of 3GPP TR 29.866, proposes to reduce the burden on the HSS during the IMS initial registration process and the IMS third-party registration process by utilizing available AVs and user profile stored on the S-CSCF in case of HSS overload. This helps reduce the HSS load further decrease the probability of signalling storm and accelerate failure restoration for the IMS network while making minimal changes to the network function.

Observation 4: Solution #2 optimizes signal flows and reduce HSS load. S-CSCF need to be enhanced. SA2 need to update Clause 4.6.3 and Clause 4.24 in 3GPP TS 23.228 to support Solution #2.

Observation 5: The above impact to SA2 TS 23.228 is well identified by CT4 and reflected in LS S2-240960/C4-244485.

3. Conclusion and Proposals

Conclusion: CT4 has do well studied on the IMS disaster prevention and restoration enhancement (in FS_IMS_RES) and reached conclusion, including identified impacts to SA2 TS 23.228. Considering the workload in current SA2 and previous similar working procedure for SID/WID led by CT WGs, e.g., FS_SMS_SIB/SMS_SIB, in which CT4 also led the SI and WI with SA2 coordination, it is suggested to ask CT4 start the normative work and synchronized the update in TS 23.228 subsequently.

Proposal: To include the above conclusion in LS-out (S2-2411433) replying to LS-in (S2-240960/C4-244485).