3GPP TSG-SA2 Meeting #166	S2-2411562

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024				   (revision of S2-2411012)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5087 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NEF services to support the conclusion of KI#1 and KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon, Qualcomm, ZTE, Samsung], NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8 in 3GPP TS 23.700-77 concludes KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" and KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the NEF service to support IMS event subscription/notification and data channel establishment, update and termination. Rev-3: Add Nnef_ImsSessionManagement_Notify service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#1 and KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.1, 5.2.6.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ECSAddress | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_TrafficInfluenceData | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF, V-NEF |
|  | UeIdMappingGet | Request/Response | GMLC |
|  | UeIdMappingCreate | Request/Response | AF |
|  | UeIdMappingUpdate | Request/Response | AF |
|  | UeIdMappingDelete | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_MemberUESelectionAssistance | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_AF_request_for_QoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_DNAIMapping | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | UpdateNotify |  | AF, NWDAF |
| Nnef_UEAddress | Get | Request/Response | AF |
| Nnef_ImsSessionManagement (ImsSM) | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

# * * * * Second change * * * *

### 5.2.6.Y	Nnef_ImsSessionManagement service

##### 5.2.6.Y.1	General

The service provides the capability to create and release an IMS session with standalone data channel media and modify data channel media in a specific IMS session.

NOTE: In this release only session management of IMS data channel is specified.

##### 5.2.6.Y.2	Nnef_ImsSessionManagement_Create operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with standalone data channel media.

Inputs, Required: Calling ID, Called ID, Media information set.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

Media inforamaiton set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Type: it indicates the media type used in the created session, and can be Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC).

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P), App Binding Information, and if P2A ADC is selected, the party which this request is targeted,

-  	If PA2 or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When BDC is selected, it indicates the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Subscribed Event ID(s) as described in Table AA.2.4.2.2-1 in TS 23.228 [55], Notification Target Address and Notification Correlation ID.

Outputs, Required: Result indication and when the operation is successful: Session ID.

Outputs, Optional: None.

##### 5.2.6.Y.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the data channel media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Operation Type: it indicates adding, updating or removing the corresponding media.

-	Media Type: it indicates the operated media is Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC).

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P), App Binding Information, and if P2A ADC is selected, the party which this request is targeted

-	If P2A or P2A2P is selected, the MDC2 media endpoint address of the application layer is indicated.

-	When BDC is selected, it indicates the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Calling ID, Called ID, Subscribed Event ID(s) as described in Table AA.2.4.2.2-1 in TS 23.228 [55], Notification Target Address and Notification Correlation ID.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.6.Y.4	Nnef_ImsSessionManagement_Delete service operation

Service operation name: Nnef_ImsSessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.6.Y.5	Nnef_ImsSessionManagement_Notify service operation

Service operation name: Nnef_ImsSessionManagement_Notify

Description: Provides the subscribed event information to the NF Consumer.

Inputs, Required: Event ID as described in Table AA.2.4.2.2-1 in TS 23.228 [55], Notification Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *