3GPP SA WG2 Meeting #166	S2-2411621

Orlando, FL, USA, 18-22 November 2024

# Title:	[Draft] Reply LS on multi-modality awareness

# Response to:	Reply to SA2 LS on multi-modality awareness (R2-2409272/S2-2411300)

# Release:	Release 19

# Work Item:	NR_XR_Ph3-Core

Source:	SA2

To:	RAN2

Cc:	RAN3, SA4

Contact Person:

Name:	Sebastian Speicher

E-mail Address:	speicher AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank RAN2 for the Reply to SA2 LS on multi-modality awareness and would like to provide the following feedback.

SA2 understands that RAN2 foresees that MMSID could be used by NG-RAN to enable joint admission control and QoS flow to DRB mapping.

SA2 has decided to not provide MMSID to NG-RAN for joint admission control because SA2 already studied joint admission control for multiple multi-modal flows in Rel-18 and decided against joint admission control at RAN because the application layer is in a better position to handle joint admission control for multiple flows for the following reasons:

- Only the application layer can determine how to handle the case that the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established but it may not be acceptable if the QoS flow for audio cannot be established.

- The conditions which QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS.

- Existing PCF/NEF procedures already enable the application layer to determine whether all required QoS flows were successfully established and to take application-specific actions based on this.

With respect to providing MMSID to NG-RAN for DRB mapping SA2 would like to understand better why there is a need to ensure a specific DRB mapping for XR services before SA2 can decide whether to provide MMSID to NG-RAN for DRB mapping. SA2 would also like to clarify that the PCF can already ensure mapping of different modalities to the same DRB by mapping those modalities to the same QoS flow.

With respect to traffic synchronization SA2 is still awaiting feedback from SA4 on the usefulness of RAN 2’s proposals from an XR service perspective before SA2 will be able to discuss RAN 2’s traffic synchronization proposals.

2. Actions:

To RAN2:

ACTION: 	RAN2 is kindly requested to take the above into account and to provide more details on the need to ensure a specific DRB mapping for XR services.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #AH		24-28 January 2025	Electronic meeting

TSG-SA2 Meeting #167		17-21 February 2025	Athens, Greece