3GPP TSG-WG2 Meeting #166	S2-2411632

# Orlando, USA, 2024-11-18 – 2024-11-22										(revision of S2-2411058)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5004 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on subscription to UPF event exposure service via SMF during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Mobile], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Modifications needed in relocation procedures: - Subscribing to target UPF should be optional), Whether a new subscription should be sent or not: - If the UPF event consumer indicates the certain UPF id in subscription request or trigger direct subscription to certain UPF that UPF id indicates, after UPF relocation, the UPF event consumer may not trigger new subscription request to new UPF. - If the UPF event consumer indicates the certain DNAI in subscription request, and after UPF relocation the DNAI is changes, for example relocates from one DNAI to another DNAI, the UPF event consumer doesn’t not trigger new subscription request to new UPF in new DNAI. - Removal of Editor Note. - Incomplete procedures: Events subscribed over N4, AnyUE and Group_ID cases were missing in 4.15.4.5.6. - EN and missing cases also when direct subscription to UPF in 4.15.4.5.7 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set the SMF to subscribe the target UPF as optional and add clarification about why not to subscribe any more. - Procedure updated for the case SMF subscribes over N4. - Clarification added for how the procedures apply for Any_UE or Group_ID in 4.15.4.5.6 and 4.15.4.5.7. - Added PDU Session update with N4 session release in Source UPF. Such release makes Nupf_EE unsubscribe unnecessary. - UPF always sends any data collected not yet reported including a cause code to justify why the report is sent. UPF indicates as cause for the report that the PDU Session is not going to be served by this UPF anymore, which is the cause from UPF point of view, and understandable by cNF. - The cNF Subscription is not terminated e.g. if target has been Any_UE. - The EN is solved as described above and therefore, removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that the SMF always subscribes the UPF event. Unspecified procedure for all events and cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6, 4.15.4.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

##### 4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation

This information flow is used in the scenario of UPF relocation, e.g. the source UPF is I-UPF which is relocated in SSC mode 1, or the source UPF is PSA UPF which is relocated in SSC mode 3, or the source UPF is local PSA UPF which is relocated due to UE mobility.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation or via N4 Session Modification, depending on the event.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the UPF needs to be relocated.

5.	The SMF inititates a PDU Session update to relocate UPF (for example procedure in clause 4.3.5.3.) including N4 session release in source UPF.

6.	When source UPF terminates the subscription upon release of the UE related N4 Session (step 5)., the source UPF sends the collected data not yet sent to the UPF event consumer if any. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet. UPF includes in the notification as cause for this report that the PDU Session is not going to be served by this UPF anymore.

7-8.	To continue the subscription of the UPF event consumer, if the UPF related information is matched with the subscription request in step 1, the SMF may select a target UPF and may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. Depending on the event, SMF may have sent the subscription to target UPF in step 5 instead. The target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

Nsmf_EventExposure_Subscribe service operation can include "Any UE" or Group ID as Target of Event Reporting. For such subscriptions, steps 2-8 above would apply specifically to the event exposure subscription in UPF affected by the relocation.

##### 4.15.4.5.7	Information flow for subscription directly to UPF event exposure service during UPF relocation

Figure 4.15.4.5.7-1: Subscription directly to UPF event exposure service during UPF relocation

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer (e.g. AF/NEF) sends a request to the NWDAF for analytics, and indicates a single UE (e.g. UE IP address) in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, Application ID, DNAI, etc.

2.	If the UPF event meets the criteria for direct subscription to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address, and optional DNN, S-NSSAI, DNAI, etc.

3.	The NRF provides Nnrf_NFDiscovery_Response that may refer to an UPF.

4.	Assuming UPF has been discovered,  the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE.

5.	The UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

6.	When the PFCP session corresponding to the UE is released, e.g. due to UPF relocation, the UPF determines that the subscription to the UPF event has been terminated for that UE.

7.	If step 6 took place, the UPF may send a notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a report, and as cause code that the PDU Session is not going to be served by this UPF anymore. The UPF shall report to the consumer any data that has been collected but not sent to the consumer yet.

Nupf_EventExposure_Subscribe can be sent directly to UPF with Target of Event Reporting "Any UE". For such subscriptions, steps 5-7 above would apply specifically to the event exposure subscription affected by the relocation.

* * * End of Change * * *