3GPP TSG-SA2 Meeting #166	S2-2411645

# 18th – 22nd November 2024, Orlando USA                               (revision of S2-2411192)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1208 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the general inference procedure for vertical federated learning to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [OPPO, ZTE, Samsung, Nokia, Lenovo, Toyota, ICS, Huawei, HiSilicon], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Editor's note: How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS. is removed since clause 6.2H.2.1 describes the VFL discovery. The UE ID(s) is added in the request send from the VFL server to the VFL client to indicate the VFL client which UE will be used as the sample to let VFL client collect the data and perform the VFL inference. The Editor's note: Whether the VFL model correlation ID is needed or using the model ID is FFS. is removed. The local model in each client can be different. Model ID is designed to identiy the sepcfic model. The model ID is same means the model should have the same topology. However, in the VFL case, the topology for each client local model can be different. It is not suitable to use the same model ID for every client. The VFL model correlation ID is perferred to indicate those local model in each client should used for the same VFL task e.g. for the same analytcis ID. The VFL model correlation ID is added when the VFL client send the VFL inference result to the VFL server. VFL model correlation ID is used to let VFL server know which inference result from which client need to be combined to generate the VFL inference result. Changes in Rev. 5: Resolves EN: Editor's note: Whether the case that the AnLF can not generathe the analytics output but can determines the VFL server for the requested analytics is exist is FFS. Whether and how such NWDAF is capable to determine the NWDAF as VFL server is FFS. Adds untrusted AF as server in new procedure Changed the client intermediate local inference results to client intermediate results, to show that it is the same intermediate results as in training Since the service names will be discussed in coming meeting, uses the generic service name Nnf_VFLOperation while adding a new EN: #### Editor’s note: The service operation names used for VFL operations in the procedures below are FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Several ENs are resolved for the VFL inference procedure. Parameter from Analytics subscription such as analytics target and analytics filer is added in the request send from the VFL server to the VFL client and in related replies. The VFL model correlation ID is added when then VFL client send the VFL inference result to the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference sucessully if some open issue can not be resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G, 6.2H.1, 6.2H.2.4(new), 6.2H.2.4.1 (new), 6.2H.2.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2G	Void

### 6.2G.1	Void

### 6.2G.2	Void

#### 6.2G.2.1	Void

# * * * * Next change * * * *

### 6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAFs and/or AFs via NRF

Both the VFL server and VFL client store the VFL model after finishing the VFL training process and use the same VFL local model to perform the VFL inference later based on the VFL correlation ID. The differences between the VFL training and inference are that for the inference there is no check of the labels, nor any server intermediate results are sent and as such only client intermediate results are sent to the server.

Editor's note:	This clause needs to be updated to include the description of the subclauses for NWDAF or AF as the server case.

# * * * * Next change * * * *

#### 6.2H.2.4	Inference procedure for vertical federated learning

Editor’s note: The service operation names used for VFL operations in the procedures below are FFS.

#### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g., UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server for the requested analytics, sends a subscription request to NWDAF VFL server using NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g., UE IDs.

Editor’s note: the following NOTE is to solve the below EN. But since it is FFS in which clause to include this case the whole NOTE is done as an Editor’s note: NOTE: The AnLF has during Model subscription request received information from NWDAF containing MTLF that no model will be possible to retrieve and if AF is VFL server, the AF is known.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g., depending on their contribution to the training result and the current status of the client.

If the VFL server is NWDAF and the VFL client(s) are NWDAF(s), VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

When the VFL server is NWDAF and the VFL client(s) are AF(s), VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes the UE ID(s), the VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate results to the VFL server including the VFL correlation ID.

The intermediate results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server combines all the intermediate results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

7.	 Depending on request and when NWDAF is VFL server it sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

#### 6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Editor's note:	If any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	same as step 0 in 6.2H.2.4.1.

1.	same as step 1 in 6.2H.2.3.1, to NEF using Nnef_VFLOperation_subscribe.

NEF forwards the subscription request to AF using Naf_VFLOperation_subscribe.

NOTE: The AnLF has during Model subscription request received information from NWDAF containing MTLF that no model will be possible to retrieve and if AF is VFL server, the AF is known.

2.	Same as step 2 in 6.2H.2.3.1, to NEF using Nnef_VFLOperation_subscribe.

NEF forwards the subscription request to NWDAF using Nnwdaf_VFLOperation_subscribe.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

3.	Same as step 3 in 6.2H.2.3.1.

4.	Same as step 4 in 6.2H.2.3.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in 6.2H.2.3.1, to NEF using Nnwdaf_VFLOperation_Notify.

NEF forwards the subscription request to AF using Nnef_VFLOperation_Notify.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in 6.2H.2.3.1.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

7.	Same as step 7 in 6.2H.2.3.1, to NEF using Naf_VFLOperation_Notify.

NEF forwards the subscription request to NWDAF using Nnef_VFLOperation_Notify.

8.	Same as step 8 in 6.2H.2.3.1.

# * * * * End of changes * * * *