3GPP TSG-WG2 Meeting #166	S2-2411658

# Orlando, USA, November 18-22, 2024		(revision of S2-2410857)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0149 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of procedure for UAV Triggered Network-assisted DAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon, CATT], Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause “5.14.2 Procedure for UAV Triggered Network-assisted DAA” was introduced in SA2#164 meeting. The FFS remains in the call flow is for the case step 3 and step 4 are both received by NEF, and there was a concern that handlings behind step 3 and step 4 towards UE are basically the same, i.e. to measure the UE signal and calculate the UE location. Multiple UE locations can be derived and further determine the relative distance between the UEs. It is observed LCS/Ranging service can only determine the relative distance of the UEs, while the NWDAF analytics may output more parameters in addition to relative distance. Therefore, the step 3 and step 4 request are not requesting the same information, and NEF should handle the two requests separately. Another issue identified in the procedure is the USS cannot be a LCS client but only AF. LCS client implies to support Le interface defined by OMA spec and it is not suitable for USS. Reason for change to rev 2: GMLC receives concurrent location requests from NEF and NWDAF between steps 3 and 5, there are errors in NOTE X. The CR did not use the correct baseline approved spec, needs to be redone with accurate baseline. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to addres the editor’s note: there is a possibility that after step 6, GMLC receives concurrent location requests from NEF and NWDAF, and GMLC may decide to combine the concurrent location requests by fully executing one of the requests and using the ensuing location estimate result(s) to satisfy the other request(s) without fully executing the latter, as defiend in TS 23.273. In rev 2, the changes include: - Updated the figure and step description. - Updated the NOTE X to “There is a possibility that between steps 3 and 4b, …”. - Corrected the baseline used in the CR which used draft spec version. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note in procedure is not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implement CR (S2-2409998, CR#0143) to TS 23.256 first, before implementing this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.14.2	Procedure for UAV Triggered Network-assisted DAA

Figure 5.14.2-1: Procedure for UAV/UAV-C Triggered Network-assisted DAA

1.	The UAV establishes a PDU Session for communication with the USS as described in clause 5.2.3.

2.	The DAA service may be triggered by UAV(s). The UAV(s) (via its paired UAV-C) requests DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides to subscribe/request to 5GC for GMLC service on Ranging/Sidelink Positioning location and/or Relative Proximity predictions on collision from NWDAF.

NOTE 1:	The other content of DAA service information derived at USS is out of scope.

3.	The USS may request GMLC service via NEF.

- The USS may request GMLC service via NEF for Ranging/Sidelink Positioning location results, i.e. the relative positioning between UAV UEs as described in clause 6.20.3 of TS 23.273 [8] for one notification, or clause 6.20.4 of TS 23.273 [8] for notifications (steps 1-20 for initiation monitoring, steps 21-31 for monitoring periodic).

- USS as AF may send the absolute positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI) as described in clauses 6.2.1 and 6.3.1. GMLC replies to USS of the absolute location of UAV 1 and UAV 2. USS calculates the relative positioning result based on the UAV UE1 absolute location and UAV UE2 absolute location.

NOTE 2:	USS can request same or different GMLC for the UAV1 and UAV 2 location, e.g. depending on which PLMN the UAV registers.

4.	The USS may request NWDAF service via NEF.

4a.	The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [20] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [20]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)). The other parameters included in the request are described in clause 6.5.3.3.

4b.	After NEF receives the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [20]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

Editor's note:	If both step 3 and step 4 to step 6 happen, it is FFS how to reduce possible positioning signalling to same UEs.

4c.	If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [20] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [20].

Steps 3 and 4 can be performed in any order (e.g. simultaneously, sequentially, or alternately).

NOTE X: There is a possibility that between steps 3 and 4b, GMLC receives concurrent location requests from NEF and NWDAF, and GMLC can decide to combine the concurrent location requests by fully executing one of the requests and using the ensuing location estimate result(s) to satisfy the other request(s) without fully executing the latter, as defiend in TS 23.273 [8].

5.	If receiving the location of UAV1 and UAV2, USS calculates the relative positioning result for UAV UE1 location and UAV UE2 location from step 1 and step 2. The USS estimates the potential collision based on the information received in step 3 and/or the predictions from step 4c. The USS informs the UAV-C(s) the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

USS may be replaced by TPAE.

6.	The UAV-C(s) informs its paired UAV(s) the potential collision, and information received from the USS, include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision, and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

7.	UAVs performs operations to avoid collision.

7a.	If both the two UAVs which may collision (e.g. UAV 1 and UAV 2) been informed by their paired UAV-C(s), the UAVs can be steered to avoid collision in accordance with the received information (e.g. trajectory correction information to avoid collision) and using mechanisms that are out of scope for 3GPP.

7b.	If only one of the two UAVs which may collision (e.g. UAV 1) been informed by its paired UAV-C, the UAV 1 triggers conflict resolution procedure with UAV 2 as described in steps 4-7 in clause 5.6.1.

# * * * * End of changes * * * *