SA WG2 Meeting #166	S2-2411710

November 18 – 22, 2024, Orlando, FL, USA

Source:	Ericsson

Title:	Solving KI#4 with N6 metadata using KI#2 mechanisms

Document for:	Discussion/Agreement

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2/ Rel-19

Abstract of the contribution: This contribution discusses whether SA2 should specify a solution for KI#4 using KI#2 mechanisms.

# Background

TR 23.7000-70 includes conclusions for XRM Phase 2 study phase.  KI#4 has studied the Traffic detection and QoS flow mapping for multiplexed data flows, and KI#2 the support of PDU Set information for end-to-end encrypted traffic.

For KI#4, the enhancements related to traffic detection that have been agreed in the conclusions can be summarized as extensions to the Packet Filters. Those consist of (S)RTP header information. Several RFCs are listed that describe how to differentiate the (S)RTP media streams from other protocols which may be multiplexed and share the baseline packet filter.

The conclusions for KI#2 promote to normative phase three mechanisms for 5GC to receive Media Related Information. The conclusions indicate the information that can be received with each of the 3 mechanisms. On this aspect, the conclusions mention that information is “XRM metadata…i.e. related information defined in R18 may also correspond to XRM metadata defined as output of other Key Issues of this TR such as KI#5.”

Tdoc S2-2411182 (SA2 #165 agreed CR to TS 23.501) introduces a new clause to, as per KI#2 conclusions, specify the functionality for Support of transferring media related information over N6 with an EN that needs to be addressed:

“Editor’s Note: Other parameters related to KI#4 are FFS.

# Discussion

Discussion follows in relation to the EN Tdoc S2-2411182 that says:

“Editor’s Note: Other parameters related to KI#4 are FFS.

That is, any parameters related to KI#4 that could be in the media related information that 5GC can receive using the agreed mechanisms for KI#2 are FFS. As already mentioned, KI#2 conclusions reflected that other KIs could contribute to this information, e.g. KI#5.

As in KI#5 conclusions, “The filter information between AF and UPF (via PCF and SMF) is enhanced to be able to detect the "expedited transfer indication" in N6 metadata for QoS flow mapping”. The “Expedited Transfer Indication” is already listed in the media related information in the agreed CR (Tdoc S2-2411182).

For KI#4, the TR includes many solutions and some of these solutions propose that 5GC receives media related data on N6 e.g. reusing the mechanisms discussed for KI#2. But none was included in the agreed conclusions (TR has no evaluation). Using N6 metadata is not mentioned in the agreed conclusions.

When in alignment with KI#2 conclusions, it is agreed to include additional info as N6 metadata for another KI, that is said explicitly in the conclusions (e.g. KI#5).

Observation 1: Receiving KI#4 related parameters as XRM metadata over N6 was not a conclusion of the study phase for KI#4. There was no agreement to promote it to normative phase.

The reasons why this was not agreed are not described. There was no time for evaluations and the TR does not include evaluation text. There were papers submitted (e.g. Ericsson S2-2406230) with reasons why KI#4 solutions based on XRM metadata over N6 (there are several solutions of this type in the TR) cannot be used in the UE for UL. This type of solution applies if (S)RTP headers are not sent in clear e.g. if media is sent with IETF draft for RTP over QUIC (RoQ). But it can be only used to steer multiplexed media streams into QoS Flows in DL (reflective QoS cannot be used to solve the steering in UL when the media streams share the baseline filters).

Observation 2: it may be possible to have a partial (only DL) solution for KI#4 by adding KI#4 related parameters in the Media related information of the KI#2 mechanisms. The relevance of the additional UCs that would be solved this way has not been analyzed.

At the same time, this information (i.e. the KI#4 related parameters) is not Rel-18 XRM metadata, but new data. Whether information should be sent, what information that should be sent, those questions have not been discussed, evaluated, or agreed properly. Just considering the solutions in the TR, the following alternatives are included: Track ID (as in Solution#9), stream info (as in Solution #14) Connection or Stream ID (as in Solution #15), QSC-ID (as in Solution #27), sub-flow ID (as in Solution #29) or Obfuscated codes (as in Solution #12). They are proposed to be provided by AF to match metadata provided by the AS over N6. They are likely to impact the 5GC APIs, and not just the protocols on N6. One solution which is not in the TR would be replicating the (S)RTP Headers defined for the Packet Filters in the metadata. In another solution metadata could be used to label the media streams, for example, certain Label is included in the metadata of all the packets of multiplexed streams with same QoS requirements All these solutions differ in several aspects and the topic deserves proper analysis.

Observation 3: it is unclear which parameters related to KI#4 should be added to the N6 metadata. TR includes some alternatives and there may be other solutions. They would need to be evaluated and compared.

Based on the observations above, considering the study phase did not handle evaluations due to lack of time and considering that the remaining planned TUs are for the pending agreed normative work, adding parameters for KI#4 should not be a priority and a reason for additional TUs and/or delays.

Therefore, the following Proposal is included.

# Proposal

Adding parameters related to KI#4 in the N6 metadata was not agreed in the study phase, this type of solution would have important limitations, and it is unclear how the solution should look like (which parameters); the alternatives were not evaluated due to lack of time, and such analysis would be required now. As a result, the following is proposed:

Remove the EN. Do not further discuss KI#4 related parameters for the Media related Information in Release’19.

Consider KI#4 for E2E encrypted traffic (e.g. RoQ) a candidate for a future release.