3GPP TSG-WG SA2 Meeting #166	S2-2411822

# Orlando, USA, 18 – 22 November 2024										(revision of S2-2411078)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5713 | rev | 6 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: Enhancement for AF influence on traffic routing with Energy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [KDDI, TOYOTA MOTOR CORPORATION, China Mobile, CATT, ZTE, InterDigital, Nokia, ETRI, Deutsche Telekom, NEC], vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-66 v19.0.0 has documented the UP path adjustment in.the conclusion for key issue#3: 5GS enhancements for network energy saving and efficiency. 2) For enhancements on existing operations and procedures for energy saving and energy efficiency: - UP path of PDU session may be adjusted. This contribution provides to UP path adjustment considering energy related information. This paper is merge version of S2-2410258, S2-2410306, S2-2410369 and S2-2410346. New revision: How to achieve the UP path of PDU session adjustment is not clear, e.g., which entity can triger or perform the UP path adjustment based on the energy related information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UP path adjustment considering energy related information. New revision: Clarify how the AF, PCF and SMF trigger and perform the UP path of PDU session adjustment based on energy related information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for UP path adjustment considering energy related information in R19 Energy Efficiency study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The clause 5.X.Y follows the structure in CR5628 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***1st Changes ***

5.X.Y	UP path selection and adjustment based on network energy related information

SMFs may consider energy related information in the NF profile to select suitable UPF(s) for a PDU session.

Editor’s note: Considering energy related information from EIF and/or OAM for UP path adjustment of PDU session is FFS.

AF may subscribe the energy related information of a list of DNAI(s) from the EIF, and EIF collects NF/Node-level Energy Consumption information related to the DNAI and generates Energy Consumption report exposed to AF. AF may take the energy consumption report into account for the DNAI(s) adjustment and provides the adjusted DNAI(s) for the UP path adjustment using the application function influence on traffic routing as described in clause 5.6.7. AF may also subscribe a Maximum allowed Energy Consumption for a DNAI to the EIF, when exceeding the maximum allowed energy consumption, EIF notifies the AF, then the AF may adjust the DNAI for UP path adjustment.

PCF may subscribe the energy related information of a list of DNAI(s) from the EIF, and EIF collects NF/Node-level Energy Consumption information related to the DNAI and generates Energy Consumption report exposed to PCF, based on the Energy Consumption report, the PCF may provide an updated list of DNAI(s) to SMF for UP path adjustment as described in clause 4.3.5 of TS23.502[3].

SMF may subscribe to the EIF about NF/Node-level Energy Consumption of UPF, based on the Energy Consumption report of the UPF, the SMF may re-select the UPF for the UP path adjustment as e.g., in the clause 4.3.5 of TS23.502[3].

# *** End of Changes ***