3GPP TSG-WG SA2 Meeting #166	S2-2412048

Orland, Nov 18 – 22, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#1 on topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#1 on topology 2 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#1 on topology 2.

Main rationales for this proposal:

- The UE NAS-based solution is less efficient than the RRC-based option, as the UE still needs to contact the gNB for resource allocation.

- The same applies to the UP-based option. In RAN3, the same issue with the UP option is being discussed: the UP solution cannot function without a resource allocation process, which must involve the CP path, creating a challenge for UP transparency. In contrast, the RRC-based solution is a natural fit and aligns well with topology 1, where the CN (AMF or AIOTF) communicates with the AIoT RAN using NGAP. And it is observed that RAN3 is very likely to support the RRC option.

- The RRC based option A, where the AIOTF has a direct interface with the gNB, has the following drawbacks:

  - This architecture cannot support the main case where the AF provides UE reader information already known, as AMF involvement is anyway necessary to locate these UE readers and translate core-level IDs (SUPIs) into NGAP IDs.

  - A new interface would need to be defined between the gNB and AIOTF even if it is a lightweight version of NGAP, including new multivendor interworking testing.

  - This would impact the gNB, requiring a new NG Setup* procedure with different IEs and different behavior when connected to the AIOTF. The AIOTF cannot be treated as an AMF.

  - It would allow having two separate termination of RAN CP interface (i.e., the AMF and the AIoTF) from the gNB to the CN.

  - The AIoT request with area information might not be fulfilled if no UE readers are connected in the location, as the AMF is required to put UEs into connected mode. This limitation occurs when the gNB fully selects the UE readers.

  - This option would not resolve the issue of per-application authorization to function as a reader.

Thus, it is proposed to keep only the RRC based option with AMF involvement as the conclusion.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.1.0.

FIRST CHANGE

### 8.1.3	Architecture to Support Topology 2

The following principles apply:

-	Messages between the UE Reader and the AIOTF are delivered using RRC and NGAP between gNB/AIoT RAN and AMF, and SBI between AMF and AIOTF.

Note 1:	Details of the AMF Service toward AIOTF (e.g. whether it is a new service or the existing Namf service is enhanced) will be determined during the normative phase.

Note 2:	Details of the required NGAP and RRC enhancement will be determined by RAN3 and RAN2.

- Figure 8.1.3.3-1 below shews the aspects related to Topology 2 reference architecture with other NFs removed.

Figure 8.1.3.3-1: Non-Roaming 5G system architecture for topology 2

- Figure 8.1.3.3-2 below shows the aspects related to Topology 2 protocol stack between the UE reader and AIOTF.

Figure 8.1.3.3-2: Protocol Stack for the RRC option

END OF CHANGES