3GPP SA WG2 Meeting #166 	S2-2412065

Orlando, US, Nov 18 – 22, 2024	(revision of xx-yyxxxx)

# Source:	ZTE

# Title:	TEI20 small enhancement on Network Slicing

# Document for:	Information

# Agenda Item:	30.8

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	TEI20 small enhancement on Network Slicing

Acronym:	TEI20-TBD

Unique identifier:	TBD

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

3	Justification

Currently the UE-Slice-MBR is statically subscribed in UDM. The network slice consumer may want to upgrade the UE-Slice-MBR for short time to meet new requirements and fall back to subscribed UE-Slice-MBR after the short time. NSSAA procedures enhancement to support dynamical Slice MBR provisioning from the network slice consumer needs to be specified.

The network slice mapping information between VPLMN and HPLMN is only provided from NSSF to AMF in the Nnssf_NSSelection_Get service operation. When the mapping information is updated in the NSSF(e.g. the VPLMN network slice is not available), it is not possible to notify the AMF in time about the change. A mechanism to support slice mapping information notification from NSSF needs to be specified.

4	Objective

The following aspects will be specified in this WID:

- Support dynamical Slice MBR provisioning from the network slice consumer.

- Support slice mapping information notification from NSSF

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |

Total TU estimates for the study phase:

Total TU estimates for the normative phase: 0.5

Total TU estimates: 0.5

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Enhancement of system architecture of 5GC | SA#103 (March 2024) | SA#104 (June 2024) |
| TS 23.502 | Enhancement of procedure of 5GC | SA#103 (March 2024) | SA#104 (June 2024) |

6	Work item Rapporteur(s)

name, company, company dot com

7	Work item leadership

SA2

8	Aspects that involve other WGs

RAN impacts covered by RAN WGs (RAN2, RAN3).

Security aspects covered by SA3.

OAM and charging aspects covered by SA5.

9	Supporting Individual Members

Supporting IM name

ZTE