3GPP TSG-WG SA2 Meeting #165	S2-2412081

# Oct 14 – 18, 2024, Hyderabad, IN

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5861 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on same PLMN scenario of MWAB operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IIT Bombay |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding clarifications to support ‘BH PLMN and MWAB Broadcasted PLMN being same’, based on the conclusions in TR 23.700-06. In same PLMN scenario there is a need to specify N6 interface between BH-UPF and UE-AMF/UE-UPF/NG-RAN, which is not indicated in TS 23.501 or shown in any of the figures in the Annex S clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are added: - Adding details of N2 and N3 comunication from BH PDU session to UE AMF and UE UPF when both BH PLMN and MWAB Broadcasted PLMN are same. - Adding details of Xn comunication from BH PDU session to another NG-RAN node when both BH PLMN and MWAB Broadcasted PLMN are same. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Details on single PLMN scenario handling is missing, which may result in lack of understanding during deployment. It would also deem the normative work on MWAB to be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex S.2 (informative) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex S (informative):
Architecture examples and protocol stacks for MWAB support

# S.1	General

This Annex provides system architecture examples for MWAB operations in PLMN cases and NPN cases. The example protocol stacks for the MWAB support is also provided.

# S.2	System architecture for MWAB operation in PLMNs

Figure S.2-1 presents an example architecture for the MWAB operation when no roaming was involved for the MWAB-UE. In this case, there may be two PLMNs involved, i.e. the BH PLMN that serves the MWAB-UE, and the MWAB Broadcasted PLMN that serves the UE connected to the MWAB. In some cases, the BH PLMN and the MWAB Broadcasted PLMN can be the same PLMN.

The MWAB-gNB logically belongs to MWAB Broadcasted PLMN and announces the PLMN IDs of the MWAB Broadcasted PLMN. The MWAB-gNB establishes N2 and N3 connection with the UE AMF and UE UPF in the MWAB Broadcasted PLMN via the PDU session of the MWAB-UE established with BH PLMN.

The BH UPF in the BH PLMN serves the MWAB-UE and provides the connection via a N6 interface towards the MWAB Broadcasted PLMN, to carry the N2 and N3 traffic from MWAB-gNB. The BH UPF also supports the access to the OAM servers in the MWAB Broadcasted PLMN by the MWAB-gNB.

NOTE:	Depending on deployment requirement, a security gateway may be required between the BH UPF and the MWAB Broadcasted PLMN network. In that case, the MWAB-gNB need to connect to the security gateway based on pre-configured security credentials. The traffic between MWAB-gNB and the MWAB Broadcasted PLMN goes inside the security tunnel established via the security gateway.

UE connected to the MWAB-gNB can access the 5GS services offered by MWAB Broadcasted PLMN. No MWAB specific enhancement to the UE is required. The UE connected to the MWAB-gNB is not aware of the BH PLMN, and thus does not need any roaming agreement between its HPLMN and the PLMN 1.

Figure S.2-1: Architecture for MWAB operation support - non-roaming

Figure S.2-2 presents an example architecture for the MWAB operation when MWAB-UE is roaming with a Local Breakout PDU Session for its operation. In this case, there may be three PLMNs involved, i.e. the BH PLMN that serves the MWAB-UE, and the MWAB Broadcasted PLMN that serves the UE connected to the MWAB, and the HPLMN of the MWAB-UE. The use of the Local Breakout PDU Session by the MWAB-UE can be configured by its HPLMN, e.g. with some VPLMN specific URSP rules. In some cases, the BH PLMN and MWAB Broadcasted PLMN can be the same PLMN.

The BH PLMN can access the MWAB-UE's HPLMN UDM for the subscription information. The rest of the operations are similar to that shown in figure S.2-1.

Figure S.2-2: Architecture for MWAB operation support - MWAB-UE roaming with Local Breakout

Figure S.2-3 presents an example architecture for the MWAB operation when MWAB-UE is roaming with a Home Routed PDU session for its operation. In this case, PDU session of the MWAB-UE is routed by BH PLMN to the HPLMN of the MWAB-UE. The rest of the operations are similar to that shown in figure S.2-2. In some cases, the BH PLMN and MWAB Broadcasted PLMN can be the same PLMN.

Figure S.2-3: Architecture for MWAB operation support - MWAB-UE roaming with Home Routed

In figure S.2-1, figure S.2-2 and figure S.2-3, if the UE served by the MWAB is roaming, there can be another PLMN (not shown in the figures), i.e. the HPLMN of the UE served by the MWAB, involved. The interaction of the HPLMN of the UE and the MWAB Broadcasted PLMN follows that described in clause 4.2.4 for the roaming case. The UE served by the MWAB-gNB is not aware of BH PLMN, and thus does not need any roaming agreement between its HPLMN and the BH PLMN.

NL1

UE LMF

Data Network

UE SMF

N4

N8

N7

N2

N4

N3

N6

N11

N6

NG-RAN

UE UPF

UE AMF

MWAB

MWAB-gNB

N2

N3

Xn

UE

NR-Uu

BH UPF

BH NG-RAN

MWAB-UE

N8

NR-Uu

N11

BH SMF

BH AMF

BH PDU session

N15

BH PCF

UDM

Figure X.2-4: Architecture for MWAB operation support – BH-PLMN and MWAB Broadcasted PLMN are same

Figure X.2-4 presents an example architecture for the MWAB operation when BH-PLMN and MWAB Broadcasted PLMN are same. N6 interface between BH-UPF and UE-AMF/UE-UPF shall be used to facilitate N2/N3 communication with MWAB-gNB. Similarly, N6 interface between BH-UPF and NG-RAN shall be used to facilitate Xn communication with MWAB-gNB.

# * * * * End of changes * * * *