3GPP TSG-SA2 Meeting #166	S2-2412177

# Orlando, USA, November 18 - 22, 2024          			                 		 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0631 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve FFS related to the interaction between the GMLCs in different PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposed to solve the following FFS: Editor's note: It is FFS if the GMLC in the PLMN serving the Target UE can trigger the (H)GMLC of the MWAB-UE to perform the location estimation of the MWAB-UE, i.e. the case when MWAB-UE H(PLMN) and the PLMN serving a MWAB-UE are different PLMNs than the PLMN serving the Target UE. Based on the following description in clause 4.3.3 in TS 23.273, A GMLC is the first node an external LCS client accesses in a PLMN (i.e. the Le reference point is supported by the GMLC). AFs and NFs may access GMLC directly or via NEF. The GMLC may request routing information and/or target UE privacy information from the UDM via the Nudm interface. If the GMLC in the PLMN serving the target UE and the (H)GMLC of the MWAB-UE are located in different PLMNs, the GMLC in the PLMN serving the target UE can access (H)GMLC of the MWAB-UE directly or via NEF. This is deployment dependent, as follows: - If there is interface between the two GMLCs, the GMLC in the PLMN serving the target UE can obtain the informat cion of GMLC in the (H)GMLC of the MWAB-UE via the NRF by invoking the procedure specifed in clause 4.17.5 in TS 23.502. In thisase, the GMLC in the PLMN serving the target UE can trigger the (H)GMLC of the MWAB-UE to perform the location estimation of the MWAB-UE. The interaction between the two GMLCs is the same as the interaction between H-GMLC and V-GMLC. - Otherwise, the GMLC in the PLMN serving the target UE can obtain the information of NEF in the (H)GMLC of the MWAB-UE via the NRF by invoking the procedure specifed in clause 4.17.5 in TS 23.502. In this case, the GMLC in the PLMN serving the target UE can trigger the (H)GMLC of the MWAB-UE to perform the location estimation of the MWAB-UE via NEF. Both options are clarified in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The interaction between two GMLCs in different PLMNs is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.19.2, 5.19.3, 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G enhanced positioning area: see TS 22.261 [3].

5G positioning service area: see TS 22.261 [3].

Backhaul PLMN (BH PLMN): see TS 23.501 [18].

LCS Client: entity that interacts with GMLC for the purpose of obtaining location information for one or more UEs. The LCS Client may reside in the UE.

For the purposes of the present document, the following terms and definitions given in TS 23.271 [4] apply:

Call Related: see TS 23.271 [4].

Codeword: see TS 23.271 [4].

Current Location: see TS 23.271 [4].

Deferred location request: see TS 23.271 [4].

DL Positioning: positioning of a target UE in which the target UE obtains downlink measurements for a 3GPP RAT.

GNSS Assistance Data: see clause 6.5.2.1 of TS 37.355 [20].

Immediate location request: see TS 23.271 [4].

Last Known Location: see TS 23.271 [4].

LCS (LoCation Services): see TS 23.271 [4].

LMF-based AI/ML Positioning: UE-assisted/LMF-based positioning with LMF-side model, and NG-RAN node assisted positioning with LMF-side model.

Local Co-ordinates: see TS 23.032 [8].

Local Location: location determined by Local Co-ordinate(s).

Located UE: see TS 23.586 [40].

Location Estimate: see TS 23.271 [4].

LOS/NLOS measurement indication: LOS-NLOS-Indicator as defined in TS 37.355 [20] or LOS/NLOS information as defined in TS 38.455 [15].

Positioning Reference Unit (PRU): see TS 38.305 [9].

Mobile Base Station Relay: see TS 23.501 [18].

Mobile gNB with Wireless Access Backhauling (MWAB): see TS 23.501 [18].

MWAB Broadcasted PLMN: see TS 23.501 [18].

PRU association: association of a PRU with an LMF by providing PRU related information to an LMF.

PRU dis-association: remove the PRU related information to dis-associate a PRU with an LMF.

Pseudonym: see TS 23.271 [4].

Pseudonym mediation device: Functionality that verifies pseudonyms to verinyms.

RAT Independent Positioning: positioning of a target UE in which the target UE obtains measurements not related to a 3GPP RAT.

Requestor: see TS 23.271 [4].

Requestor Identity: see TS 23.271 [4].

Response Method: for LCS Client using the OMA MLP protocol. Detail see TS 23.271 [4].

Scheduled Location Time: a future global time (e.g. UTC) at which a UE is to be located.

Service Type: see TS 23.271 [4].

Sidelink Positioning: see TS 23.586 [40]

Target UE: see TS 23.271 [4].

UL Positioning: positioning of a target UE in which NG-RAN obtains uplink measurements of the target UE for a 3GPP RAT.

UL+DL Positioning: positioning of a target UE using both DL Positioning and UL Positioning.

Velocity: see TS 23.271 [4].

Verinym: see TS 23.271 [4].

* * * Second Change * * *

5.19.2	Obtaining location information for the MWAB

There are multiple options for an LMF to obtain the location information and velocity of the MWAB(s):

-	The LMF can derive the location and velocity of the MWAB by using NRPPa or by requesting the GMLC in MWAB Broadcasted PLMN to derive the location of the MWAB-UE using the UE-ID of the MWAB-UE. The GMLC in MWAB Broadcased PLMN requests the location of the MWAB-UE by requesting the GMLC in MWAB HPLMN which is responsible to trigger MT-LR procedure as specified in clause 6.1.1 or 6.1.2 or requests the location of the MWAB-UE by requesting the NEF in the MWAB HPLMN which is responsible to trigger the MT-LR procedure as specified in clause 6.1.2.

-	As the timing of the location estimations for the Target UE and MWAB(s) is important for the quality of the location estimation of the Target UE, the LMF needs to reduce the timing offset of the positioning measurements, i.e. the positioning of the Target UE and MWAB can be scheduled with using the same scheduled location time and compensate for the potential time difference of the positioning measurements, e.g. taking velocity of MWAB into account.

* * * Third Change * * *

5.19.3	Privacy check for MWAB

If the positioning of the MWAB is performed for the location estimation of a Target UE (UE different from the MWAB), the privacy check in clause 5.4 is skipped for the MWAB. When the LMF requests the GMLC in MWAB Broadcasted PLMN to derive the location of the MWAB (UE), the LMF includes a MWAB indication indicating the location of MWAB is requested to determine the location of a Target UE in the location request. The GMLC in MWAB HPLMN obtains the subscription information of the MWAB-UE from the UDM. Based on the indication, and/or MWAB's subscription information, the GMLC in MWAB HPLMN skips the privacy check.

If the positioning of the MWAB is performed for the location estimation of the MWAB itself when it acts as a normal UE which is not authorized to operate as MWAB based on the subscription information, the UE privacy check procedure in clause 5.4 is performed.

* * * Fourth Change * * *

6.1.5	5GC-MT-LR procedure involving MWAB

Figure 6.1.5-1 illustrates the network positioning requested by the LCS clients when MWAB(s) is involved.

In this scenario, it is assumed that the target UE may be identified using an SUPI or GPSI. The procedure follows the functionality in clause 5.19 and the 5G-MT-LR procedures in clauses 6.1.1 and 6.1.2. It is further assumed that:

-	The NG-RAN in figure 6.1.5-1 is a MWAB-gNB.

-	When the MWAB-gNB is integrated to the 5GS, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a MWAB-gNB is mobile and its UE ID (GPSI) of the MWAB-UE via a TRP information exchange towards the MWAB-gNB with the Cell ID of the TRP. The LMF may also performs NRPPa TRP information exchange procedure before the positioning procedure for a target UE, e.g. to determine the position capability of NG-RAN.

-	The LMF that performs the location estimation of the MWAB (MWAB-UE) can be different (e.g. even in different PLMN) than the LMF that performs the location estimation of the target UE (not shown in figure 6.1.5-1).

Figure 6.1.5-1: 5GC-MT-LR procedure involving MWAB

1.	[Optional] The location services client sends a request to the (H)GMLC for a location for the target UE identified by an GPSI or an SUPI. The UE may or may not be served by a MWAB.

2.	The GMLC in MWAB Broadcasted PLMN invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF serving the target UE to request the current location of the target UE. The location procedure may be also triggered at the AMF for a 5GC-NI-LR as at step 1 in clause 6.10.1, or an event report sent by the UE for a deferred 5GC-MT-LR for periodic or triggered location as at step 25 in clause 6.3.1.

3.	The 5GC-MT-LR procedure as specified in clause 6.1.1 step 5-8 or clause 6.1.2 step 6-12 are performed with the difference that the LMF selection also consider LMF capability supporting MWAB involvement. If the AMF is aware that the target UE is served by a MWAB, it would select the LMF that can support the MWAB handling.

NOTE 1:	The step 4 to 11 may happen as part of step 3.

4.	LMF derives if any MWAB(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 3. The AMF serving the target UE may indicate that the serving cell is an MWAB (if applicable). When the MWAB-gNB was integrated as a TRP, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MWAB and/or the UE-ID (GPSI) associated with MWAB-UE. The LMF, when the OAM provides a new Cell Id to the LMF, performs a TRP information exchange as specified in TS 38.455 [15] for the new Cell ID. When the MWAB-gNB is integrated in the 5GS, the Cell ID(s) of the MWAB is (are)provided to the LMF. If the LMF selects any cell for UE positioning measurements, and the LMF determines it needs to perform a TRP information exchange with the cell, it shall perform TRP Information Exchange procedure as specified in TS 38.455 [15].As the MWAB can be mobile the LMF may need to determine an updated location of the MWAB by either performing step 5-7 (option 1) or performing step 8-10 (option 2) if option 1 is not feasible.

NOTE 2:	It is assumed that for the case of MWAB mobility, the OAM can timely update the Cell Id of the MWAB to the LMF so as to enable it to perform a proper TRP information exchange. Until the Cell Id of the MWAB is provided to LMF by OAM, the Cell of the MWAB is not used for location procedures.

5.	[Conditional] The LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MWAB(s) by directing the TRP information exchange messages to the MWAB-gNB associated with the Cell ID(s), for example, by setting the NRPPa TRP Information Type Item IE to "mobile TRP location info".

6.	[Conditional] The MWAB-UE may initiate a 5GC-MO-LR procedure (as defined in clause 6.2) to get its location information if requested by MWAB-gNB based on implementation. The MWAB-UE's location info is used by the co-located MWAB-gNB to determine the updated location for its TRP.

7.	[Conditional] The MWAB-gNB report its updated TRP location, e.g. TRP's geo-coordinate, velocity and the time for obtaining them, to the LMF.

8.	[Conditional] If the LMF invokes the Ngmlc_Location_ProvideLocation_Request service operation to the GMLC in MWAB Broadcasted PLMN by providing the identifier of MWAB-UE and a scheduled location time. The LMF determines the identifier of MWAB-UE based on the Cell ID of MWAB-gNB and the received the GPSI of the MWAB-UE in a TRP information exchange when the MWAB-gNB was integrated as a TRP. LMF may receive the scheduled location time in step 3. Otherwise, LMF may generate the scheduled location time, e.g. based on response time.

9.	[Conditional] The GMLC in BH PLMN determines the AMF serving the MWAB-UE and steps 4-10 in clause 6.1.1 or steps 4-23 in clause 6.1.2 is performed. The privacy check in clause 5.4 is skipped for the MWAB based on the subscription data of the MWAB-UE. The scheduled location time received in step 8 is also provided to the MWAB-UE when sending a location request to the MWAB-UE. The GMLC of the UE may further retrieve the location information of the MWAB-UE via the GMLC or NEF in MWAB HPLMN which is responsible to retrieve the location information of the MWAB-UE via the GMLC in BH PLMN, based on the information retrieved from UDM of the MWAB-UE (e.g. serving AMF and GMLC address serving the MWAB-UE in BH PLMN).

10.	[Conditional] The GMLC in MWAB Broadcasted PLMN sends the location service response including the MWAB-UE's location to the LMF when the location response is received from the GMLC or NEF in MWAB HPLMN.

11.	[Conditional] The LMF performs one of the positioning procedures with the target UE described in clauses 6.11.1 and 6.11.2. To reduce the timing offset of the positioning measurements, the UE positioning may be scheduled with the same scheduled location time as the MWAB-UE positioning in step 9. LMF sends the same time with the scheduled location time in step 8 to the target UE in clause 6.11.1. If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB-UE updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455 [15].

12.	The LMF uses the received location and velocity of the MWAB(s) when estimating the location of the target UE together with the measurements reports in step 3 or optionally the updated measurement reports in step 11.

13.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the target UE.

14.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC to return the current location of the target UE.

15.	The (H)GMLC of the target UE sends the location service response to the location services client.

* * * End of Changes * * *