3GPP TSG-WG SA2 Meeting #166	S2-2412211

# Orlando, USA, Nov 18 – Nov 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5114 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to UPF selection for additional capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Deutsche Telekom, SK Telecom, Rakuten Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The impacts on N4 Session establishment procedure are missing. The corresponding clause 6.3 in 23.501 only describes N4 Node level procedures (i.e. N4 Association procedures). The subscription data for a PDU Session can include a list of required/preferred functionalities for the PDU Session based on which SMF selects UPF for the PDU Session. Whereas, a UPF can be selected for a User’s PDU Session even if the functionalities are present in the subscription data. For the UPF to be able to be able to use this information, and apply the customized configurations to dedicated users who subscribed for those features (e.g. protection features), enhancements to N4 Session establishment messages are needed (as described in Solution#4 of the TR 23.700-63). This information is not provided in the form of N4 rules (i.e. PDR/FAR etc.) but is applicable for the whole PDU Session (similar to how SMF provides ATSSS control information to UPF) No impacts on N4 rules are expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the impacts on N4 Session Establishment procedure for supporting Required/Preferred UPF functionalities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPF functionalities present in subscription data do not work as desired. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Rev3 : Clarified that the Functionalities are to be applied for the whole PDU Session and not for particular SDFs. Clarified How UPF uses the information regarding capabilities is out of scope. |  |  |  |  |  |  |  |  |

# * * * * First change * * *

#### 4.4.1.2	N4 Session Establishment procedure

The N4 Session Establishment procedure is used to create the initial N4 session context for a PDU Session at the UPF. The SMF assigns a new N4 Session ID and provides it to the UPF. The N4 Session ID is stored by both entities and used to identify the N4 session context during their interaction. The SMF also stores the relation between the N4 Session ID and PDU Session for a UE.

Figure 4.4.1.2-1: N4 Session Establishment procedure

1.	SMF receives the trigger to establish a new PDU Session or change the UPF for an established PDU Session.

2.	The SMF sends an N4 session establishment request message to the UPF that contains the structured control information which defines how the UPF needs to behave. If the SMF is a V-SMF and it supports HR-SBO for the PDU session, V-SMF includes SUPI, HPLMN DNN and S-NSSAI, and an indication that the UE PDU session is working in HR-SBO mode.

If the UPF supports Nupf_EventExposure service, the SMF should include DNN and S-NSSAI in the N4 Session Establishment procedure.

NOTE 1:	If SMF does not provide DNN and S-NSSAI to UPF it could result in rejections for the Nupf_EventExposure_Subscribe service operations, unless UPF is configured with a DNN and S-NSSAI for a specific IP address range.

If the UPF supports operator configurable UPF capabilities (as defined in clause 5.8.2.21 of TS 23.501 [2]), the SMF may provide to the UPF the information of the UPF functionalities that apply for the PDU Session associated with the N4 session.

NOTE 2:	SMF can determine the functionalities that apply for the PDU Session based on the required and/or preferred functionalities obtained from the subscription data and/or based on the functionalities supported by the UPF. How UPF uses this information is out of scope.

EN : [UPEAS_Ph2] Whether SMF can provide Packet Inspection (or more specifically, inspection based on higher layers/DPI) as one of the functionalities that apply for the PDU Session to the UPF is FFS.

3.	The UPF responds with an N4 session establishment response message containing any information that the UPF has to provide to the SMF in response to the control information received.

If the UPF (by configuration or other means) utilizes an NWDAF, UPF adds the NWDAF serving the UE identified by the NWDAF instance ID. Per NWDAF service instance the Analytics ID(s) are also included.

NOTE 2:	The SMF can use this NWDAF related information and can forward it to the PCF using the SMF initiated SM Policy Modification procedure, as a result of a Policy Control Request Trigger.

4.	The SMF interacts with the network function which triggered this procedure (e.g. AMF or PCF).

# * * * * End of change * * *