3GPP TSG-WG SA2 Meeting #166	S2-2412241

# Orlando, USA, 18 – 22 November 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1529 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS-AGW relocation for UE-Sat-UE communication at change of satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, UE-satellite-UE communication via IMS-AGW deployed on satellite should be supported. When satellite changes, the IMS-AGW on satellite may be relocated to a target satellite or on the ground. This can be triggerd by a re-INVITE message. This contribution propose the IMS network to send re-invite to UE and IMS network at peer side. Two approaches are proposed: - P-CSCF to send re-invite, which means the P-CSCF needs to support B2BUA. This approach is straightforward and dose not have impact on other IMS function,e.g. S-CFCF, IMS AS. - S-CSCF to send re-invite, based on notification from P-CSCF. This approach is simple comparing to the IMS AS to send re-invite, considering P-CSCF has no knowledge of the capable IMS AS. To simplify the SIP procedures the P-CSCF can include SDP information in the notification, then the S-CSCF can include this SDP in the re-invite message. Which way or whether both ways should be adopted can be decided during the meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an IMS-AGW relocation procedure for UE-Sat-UE communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompleted specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Implementation notes: The new clause AX.5.x in the CR is after the clause AX.5.1 in the approved CR1492. The referenced TS23.501 clause 5.4.x is added by TS23.501 CR5583 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change(all new) * * * *

AX.5.x	At change of satellite

Based on the call setup procedure described in clause AX.5.1, both originating and terminating P-CSCF should subscribe to the SMF for the Notification of User Plane Management Events associated with UE-Satellite-UE communication media traffic, with Early notifications type based on clause 5.6.7.1 in TS 23.501 [93] and clause 4.3.6 in TS 23.502 [94].

Based on the description in clause 5.4.x of TS 23.501 [93], at change of satellite connected with UE, the SMF serving the originating or terminating UE may report a target satellite identifier and a candidate DNAI associated with UPF onboard the target satellite (if the target satellite is connected with source satellite) or associated with UPF on ground network (if the target satellite is not connected with source satellite). The procedure in this clause describes how media re-negotiation and IMS-AGW relocation are performed after receiving the report, in which the P-CSCF A or B can be any side of the originating or terminating P-CSCF. The P-CSCF performs different actions based on the candidate DNAI reported from SMF:

-	If the SMF reports a target satellite identifier and a candidate DNAI associated the UPF onboard the target satellite, the P-CSCF shall determine whether UE-satellite-UE communication can continue, e.g., by checking whether the target satellite is connected with the peer side satellite (if they are not the same).

-	If the SMF reports a candidate DNAI associated with the PSA UPF on ground network, the P-CSCF shall perform media re-negotiation as normal after selecting a ground IMS-AGW.

Figure AX.5.2-1:	Procedure for serving satellite change with P-CSCF/S-CSCF triggering re-invite

1a.	If 5GC detects the serving satellite ID of UE A change during handover, PCF notifies P-CSCF A that the serving satellite ID of UE A change has occurred and indicates the new serving satellite ID of UE A. If 5GC detects the UE A is not access via regenative based satellite access but using transparent satellite access or terrestrial network access, PCF notifies that to P-CSCF.

1b.	If 5GC decides to change the UP path status e.g., due to the serving satellite ID of UE A change, Early/Late Notification including a candidate DNAI will be sent to P-CSCF A, since P-CSCF A has subscribed to user plane management event at call set up step 12 or step 18 during Application Function influence on traffic routing procedures described in clause 4.3.6 of TS 23.502 [94].

Case 1:

2.	P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B can continue based on the new serving satellite ID of UE A and stored serving satellite ID of UE B.

3.	P-CSCF A and SMF negotiate to determine the target DNAI.

If receiving the Early Notification with a candidate DNAI associated with UPF onboard satellite from SMF, P-CSCF A shall map the canidate DNAI to the AGW and determine whether the candidate DNAI associated with the AGW can be used. If the candidate DNAI can be used, the positive response will be sent to the SMF as defined in TS 23.502 [94] to establish a path on the target satellite for optimized media routing. If the candidate DNAI cannot be used, the negative response is sent to SMF. SMF may re-selet a new DNAI and trigger the negotiate procedure again or terminate the relocation procedure.

If the Early Notification is not received from SMF, P-CSCF A determines to relocate the AGW to a new satellite, then P-CSCF A sends a request to SMF with the candidate DNAI associated with the target new AGW. If P-CSCF A received a positive response, then P-CSCF A is aware that the candidate DNAI can be used as the target DANI. If P-CSCF A received a negative response, then P-CSCF A will re-selet a new AGW and perform this step again.

4.	The P-CSCF A requests the new AGW to allocate resources for UE A. The P-CSCF A determines to send SIP re-INVITE to both sides, or the P-CSCF sends message to S-CSCF to trigger it to send re-invite to both sides. P-CSCF can include SDP information in the message to S-CSCF, then the S-CSCF can include this SDP in the re-invite in following steps.

5.	The SIP re-INVITE with modified SDP offer for the new address allocated by the new AGW and the new serving satellite ID of UE A is sent to the peer side. Upon receiving the new satellite ID, P-CSCF B is aware that the UE-satellite-UE communication for UE A and UE B can continue.

6.	The modified SDP offer for the new AGW is sent to UE A using the SIP re-INVITE procedure described in step 4.

Step 5 and 6 can be parallel.

Case 2:

2.	P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B can continue based on the new serving satellite ID of UE A and stored serving satellite ID of UE B. In this case, the Early/Late Notification is not received from SMF and P-CSCF A determines AGW does not need to be relocated e.g., not every satellite has a AGW on-board.

3a.	P-CSCF A determines to send SIP re-INVITE, or the P-CSCF sends message to S-CSCF to trigger it to send re-invite, or

3b.	P-CSCF A determines to send SIP message.

NOTE:	Using SIP message to send the new serving satellite ID of UE A to the peer side can be faster and have a smaller message size.

4a.	The SIP re-INVITE with new serving satellite ID of UE A and SDP offer is sent to the peer side, or

4b.	The SIP message with new serving satellite ID of UE A is sent to the peer side.

Case 3:

2.	P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B cannot continue based on the new serving satellite ID of UE A and stored serving satellite ID of UE B. In this case, the access type of UE A does not change.

3.	P-CSCF A and SMF negotiate to determine the target DNAI.

IIf receiving the Early Notification with the DNAI associated with the UPF onboard satellite from SMF, P-CSCF A sends a negative response with the DNAI associated with the target new AGW on the ground to the SMF.

If the Early Notification is not received from SMF, P-CSCF A selects the AGW on the ground and sends a request with the DNAI associated with the target new AGW on the ground to the SMF.

4.	The P-CSCF allocate resources on the ground AGW. The P-CSCF A determines to send SIP re-INVITE to both sides, or the P-CSCF sends message to S-CSCF to trigger it to send re-invite to both sides. P-CSCF can include SDP information in the message to S-CSCF, then the S-CSCF can include this SDP in the re-invite in following steps.

5.	The SIP re-INVITE with modified SDP offer for the ground AGWa nd the new serving satellite ID of UE A is sent to the peer side. Upon receiving the new satellite ID, P-CSCF B is aware that the UE-satellite-UE communication for UE A and UE B cannot continue.

6.	The modified SDP offer is sent to UE A using the SIP re-INVITE procedure.

Step 5 and 6 can be parallel.

Case 4:

2.	If receicing the notification that UE A is not using regenerative-based satellite access but transparent satellite access or terrestrial network access from 5GC, P-CSCF A determines that the UE-satellite-UE communication for UE A and UE B cannot continue.

3.	P-CSCF A and SMF decide to fall back to the ground.

If receiving the Late Notification with the DNAI associated with the PSA UPF on the ground from SMF, P-CSCF A relocates the AGW on the ground and sends positive response to SMF.

4. 	The P-CSCF allocate resources on the ground AGW. The P-CSCF A determines to send SIP re-INVITE to both sides, or the P-CSCF sends message to S-CSCF to trigger it to send re-invite to both sides. P-CSCF can include SDP information in the message to S-CSCF, then the S-CSCF can include this SDP in the re-invite in following steps.

5.	The SIP re-INVITE with modified SDP offer for the ground AGW is sent to the peer side. Upon receiving the SIP re-INVITE without a satellite ID, P-CSCF B is aware that the UE-satellite-UE communication for UE A and UE B cannot continue because the access type of UE A is not regenerative-based satellite access.

6.	The modified SDP offer is sent to UE A using the SIP re-INVITE procedure described in step 4.

Step 5 and 6 can be parallel.

# * * * * End of changes * * * *