3GPP TSG SA WG2 Meeting #166	S2-2412295

Orlando, US 18th Nov 2024 - 22nd Nov 2024	(revision of S2-220xxxx)

Source:	Apple

Title:	VFL: Discussion on sample change during training

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: This discussion paper analyses how sample change during VFL training can be supported.

# 1. Discussion

In SA2#165, we had presented the scenario of change or loss of sample (e.g., UEs) during a VFL Training process in S2-2410078.

In this discussion paper, we analyse how the loss of samples during VFL training can be handled.

### Loss of sample(s) during VFL Training

The reasons for loss of samples, especially when samples are UEs could be UE mobility or change of user consent for participation in training. Loss of samples come with the risk of having to throw away all the training progress and start all over again. In some cases, the VFL server would be able to work with a reduced set of samples from an intermediate point in the training. These intermediate points in the training process can be seen as training checkpoints.

The key aspects for handling loss of samples during VFL training are

- Alignment on training checkpoint information (This may include information like client actions to store model status at training checkpoints, how many checkpoints can be stored etc)

- VFL client is able to notify loss of sample(s) to the VFL server

- VFL server is able to indicate start of a training round from a previous training checkpoint and the samples that are removed.

The following call flow shows how loss of sample during VFL training is handled

Figure 1 Sample change during VFL training

Step 1. VFL server performs the VFL client discovery and selection.

Step 2. VFL Server starts the preparation phase. The contents of the intermediate results are agreed at this stage. VFL server indicates how it would indicate training checkpoints.

Step 3. VFL Client confirms the preparation request

Step 4. VFL Server and client performs sample alignment.

NOTE: This could be combined with preparation phase.

Step 5. VFL server starts the training process

Step 6. VFL client sends the intermediate results

Step 7. VFL server processes the intermediate results and decides on the backward propagation local model training parameters. VFL server also determines whether this can be considered a training checkpoint.

Step 8. VFL server starts another round with the update of local model parameters. VFL server also indicates whether this is a training checkpoint.

Step 9. VFL client updates local model with the provided parameters. VFL client stores the model status at the checkpoint.

Step 10. VFL client detects loss of samples.

Step 11. VFL Client notifies the loss of samples to the server

Step 12. VFL Server decides to restart training from a previous checkpoint

Step 13. VFL Server indicates the start of another round of training from a previous checkpoint.

If there are more than one VFL client in the training process, the VFL server ensures that all VFL clients are aware of the training checkpoints and indicates the lost samples at the start of training round from a training checkpoint.

If the VFL server determines that continuing the training is not possible due to the loss of sample(s), it may decide to terminate the training process.

The CR 1134 to 23.288 is revised to describe the handling of loss of samples during VFL training process in S2-2411346.

Proposal 1: It is proposed that SA2 discuss the revision of CR1134 describing the handling of loss of samples for VFL training.

# 2. Conclusion

Proposal 1: It is proposed that SA2 discuss the revision of CR1134 describing the handling of loss of samples for VFL training.

# 3. References

- S2-2410078 - VFL: Sample Alignment and Sample Update , SA2#165

- S2-2411346 – General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s)