3GPP TSG-WG SA2 Meeting #166	S2-2412300

Orlando, USA, Nov 18 – 22, 2024

Source:	China Mobile

Title:	Conclusions for KI#2 – Temp ID for privacy protection

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Introducing a new Temp ID based privacy protection solution without writing operations and registration procedure. Temp ID related conclusion for KI#2 are proposed.

# 1. Introduction/Discussion

Permanent Device ID will be used to carry sensitive goods information and as a username during the authentication procedure. This is necessary to realize privacy protection in the solution.

The current solution to realize privacy protection is based on a temp ID assigned by 5GC. However, an additional registration procedure and frequent sync-up procedures (writing operations) between 5GC and Ambient IoT Device are needed.

Regarding LS from RAN, frequent writing operations shall be avoided in the solution. Besides, additional registration procedures will increase the cost of the whole solution.

Actually, there is another temp ID solution in solution 1: AIoT Temporary Identifier Control. The temp ID is generated by Ambient IoT device itself, then 5GC obtains binding relationship between temp ID and permanent device ID through standard inventory procedure (read operation).

In our view, the solution #1 can be enhanced to support the privacy protection without frequent writing operation, the main pointes are as follows:

1).  The ambient IoT device assigns the temp ID, not 5GC.

2).  The Ambient IoT temp ID is synced between the ambient IoT device and 5GC by standard inventory procedure.  Thus, there is no writing operation between 5GC and the Ambient IoT Device.

3).  5GC will obtain and maintain the binding relationship between temp ID and permanent device ID via standard inventory procedure.

4).  5GC will add an “ID response indication” parameter in the standard inventory message to instruct ambient IoT devices to respond with only Temp ID or both Temp ID and permanent device ID to realize uplink privacy protection

5). 5GC can use Temp ID to replace permanent device ID as the target ambient device filter info in inventory message from 5GC to ambient IoT devices to realize downlink privacy protection

Temp ID generation and sync up procedure between device and 5GC

Downlink and Uplink Privacy protection based on Temp ID

Temp ID update and sync up procedure

Highlights on Solution:

  - Don’t need additional registration procedure to trigger Temp ID allocation

  - Don’t need to write operations to sync up Temp ID between 5GC and ambient IoT Devices

  - Temp ID is synced up between ambient IoT device and 5GC by standard inventory procedure (read operation)

  - Improve data transportation efficiency by using Temp ID (maybe 20-30 bits) to replace Permanent device ID (average ID length 96bits – 496bits)

  - Low-cost solution to realize privacy protection based on Temp ID.

  - Temp ID solution can be enabled based on AF ID

Overhead on the solution:

  - We are adding an additional parameter, ID response indication, to instruct the ambient IoT device how to respond to the inventory.

Proposal:  The temporary ID in the AIoT NAS layer is required for the privacy protection. The detailed Temporary ID solution can be determined during the normative phase.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13.

FIRST CHANGE

## 8.2	Interim Conclusion on Key Issue #2

### 8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier and Identification Management.

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, which is used to indicate the identifier is assigned by an operator or by a third party.

-	The operational entity, which is the entity managing and allocating the Identifier:

-	For operator allocated ID, it is a network identifier (i.e. MCC+MNC and/or NID).

-	For third party allocated ID, it is information used to identify a 3rd party.

Editor's note:	Whether the operational entity is always needed is FFS.

2)	Part2 information:

-	The information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

NOTE 2:	If the information used to identify the 3rd party is the scheme and the company prefix of the EPC, Part2 information is not necessary to be the whole EPC but can be the leftover part of the EPC.

NOTE 3:	The coding for the above information is left to stage 3.

Figure 8.2.1-1: The Operator allocated ID

For operator allocated Identifier, the network identifier is mandatory and can be used to index the authentication server or not.

Editor's note:	For operator allocated Identifier, it is FFS whether further information (e.g. the credential holder ID) in Part 1 is needed.

Figure 8.2.1-2 The third party allocated ID

For third party allocated Identifier, the network identifier is not needed. The third party may be the credential holder or not. If it is not the credential holder, the network should be provided with third party related context including the information used to locate the authentication server.

Editor's note:	The length of Identifier is fixed or dynamical is FFS.

Editor's note:	The uniqueness of Identifier is FFS.

The temporary ID in the AIoT NAS layer is required for the privacy protection. The detailed Temporary ID solution can be determined during the normative phase.

END OF CHANGES