3GPP TSG-WG SA2 Meeting #166	S2-2412327

Orlando,US, 18-22 November 2024	(revision of S2-24xxxxx)

Source:	Apple

Title:	Conclusions update for Key Issue #1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes update for conclusions for Key Issue #1

# 1. Introduction

This contribution proposes update for conclusions for Key Issues #1 in TR 23.700-13.

AIoT devices have significant limitations in terms of their baseband capabilities, storage, form factor, cost, complexity etc. A NAS based solution for managing and conducting information transfer offers significant advantage in terms of reduced overhead for these devices. There is no PDU session created for session management and data transfer is done efficiently using NAS messages. The device communicates with the network directly by sending data (Ethernet, Unstructured frames) using its AIoT Device ID.

AIoT devices can be deployed using either Topology 1 or Topology 2. In case of Topology 2 the BS readers are likely not deployed and the CN reaches out to the AIoT device using the UE reader. Further in such scenarios, the AIoT-enabled gNB is not necessarily a reader. For Topology 2 deployment scenarios, the CN can select a UE reader based on application requirements from the AF, CN-level information, and possibly other details reported by the RAN. The CN can also work in conjunction with RAN to determine radio conditions and UE reader reachability, and thereafter page the UE reader to bring it into connected mode. AIoT device management and subsequent information transfer for AIoT use cases (inventory, commands, etc.) can then be conducted using a AIoT Device NAS protocol.

As such, it is proposed to use an AIoT NAS protocol between the AIoT Device and the AIoTF which can be used by the AIoTF to manage AIoT device related information. The NAS based solution is also specified for Topology 2.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * Start of changes * * * *

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

Editor's note:	Information needed for radio resource allocation to readers is FFS and requires cooperation with RAN WG2/RAN WG3.

Editor's note:	Which architecture options described in the following clauses will be concluded is FFS.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF manages the device related information.

Editor's note:	Whether and what AIoT Device related information (e.g. AIoT Device last known Reader ID, optionally the result of AIoT device validation result, etc.) is stored in the AIOTF, are FFS.

Editor's note:	Whether the UDM should store the device related information is FFS.

b.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE readers to perform AIoT service operations towards the AIoT Devices (s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

2.	A Permanent AIoT Device IDis stored in the AIoT Device and the UDM or a Credential Holder's AAA server.

Editor's note:	Whether and how the validation of the Ambient IoT Device ID is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT NAS protocol is supported between the AIoT Device and the AIoTF and is used by the AIoTF to manage AIoT device related information. AIoT use cases such as inventory and commands are supported by the AIoT NAS protocol.

NOTE:	Security aspects for the AIoT device will be specified by SA WG3.

### 8.1.2	Architecture to Support Topology 1

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Communicate with BS Reader (i.e. AIoT RAN), either directly or via an AMF, to trigger e.g. inventory request and command request).

NOTE 1:	There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has gNB functionality for NR-Uu or not.

Editor's note:	The relationship between the AIoT RAN and (other) gNB functionality is FFS.

NOTE 2:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

NOTE 3:	It is not expected a deployment will use both direct communication between a BS Reader and an AIoTF and indirect communication between a BS Reader and an AIoTF via an AMF.

When a BS Reader and the AIoTF communicate directly:

-	The AIOTF communicates with a BS Reader via a direct interface Nx.

-	Figure 8.1.2-1 below shows the aspects related to Topology 1 (direct path) reference architecture with other NFs removed.

Figure 8.1.2-1: Non-Roaming 5G System Architecture (Direct Path)

-	Figure 8.1.2-2 below shows the aspects related to Topology 1 (direct path) in reference point representation with other NFs removed.

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (Direct Path)

NOTE 4:	The protocol used over Nx will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

NOTE 5:	The protocol stack used between the AIoTF and the BS Reader will be concluded by RAN WG3.

-	Figure 8.1.2-3 below shows the aspects related to Topology 1 (direct path) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-3: Example Protocol Stack between AIoTF and AIoT Device for Topology 1 (Direct Path)

Editor's note:	The details of the protocol stack are FFS.

When a BS Reader and the AIoTF communicate indirectly via an AMF:

-	The AIoTF connects with a BS Reader via an AMF. The N2 interface between the BS Reader and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 6:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader.

-	NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services.

NOTE 7:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support Services which are used by an AIoTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIoTF) are introduced in 5GC.

-	The AMF passes information between BS Readers (e.g. operation requests and responses) and the AIOTF.

Editor's note:	The AMF enhancements are FFS.

Editor's note:	Details of the Service (e.g. whether it is a new service, whether the existing Namf service is enhanced is FFS).

-	Figure 8.1.2-4 below shows the aspects related to Topology 1 (indirect path via AMF) reference architecture with other NFs removed.

Figure 8.1.2-4: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2-5 below shows the aspects related to Topology 1 (indirect  Path via AMF) in reference point representation with other NFs removed.

Figure 8.1.2-5: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2-6 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-6: Example Protocol Between AIoTF and AIoT Device for Topology 1 (indirect Path via AMF)

Editor's note:	The details of the protocol stack are FFS.

Editor's note:	How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF is FFS.

### 8.1.3	Architecture to Support Topology 2

#### 8.1.3.1	General

The following options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

-	NAS-based option as defined in clause 8.1.3.4.

#### 8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-1, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-2. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN WG2, RAN WG3 and SA WG2.

Figure 8.1.3.2-1: User-plane architecture for Topology 2

Figure 8.1.3.2-2: Protocol stack for the user-plane architecture for Topology 2

NOTE 1:	Which transport protocol to use for AIoT-AP can be decided by CT WG1 in coordination with RAN WG3.

NOTE 2:	Security for AIoT-AP is assumed to be defined by SA WG3.

Editor's note:	Further details are FFS.

Editor's note:	The terminology, e.g. AIoT RAN is FFS.

#### 8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-1 and Figure 8.1.3.3-2.

Editor's note:	Which option to use, i.e. whether gNB/AIoT RAN connect to AIoTF directly (Option A in Figure 8.1.3.3-1) or via AMF (Option B in Figure 8.1.3.3-2) is FFS.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option (Option A)

Figure 8.1.3.3-2: Protocol Stack for the RRC option (Option B)

#### 8.1.3.4	NAS option

The following principles apply:

-	As depicted in Figure 8.1.3.4-1, the AIoT Device connects to the AIoTF based on the AIoT NAS Protocol (AIoT NAS). The related protocol stack is shown in Figure 8.1.3.4-1. The AIoT NAS protocol will support procedures and information to be exchanged as specified by CT WG1 and SA WG2.

Editor's note:	Further details are FFS.

Figure 8.1.3.4-1: Protocol Stack for the NAS option

# * * * * End of changes * * * *