3GPP TSG SA WG2 Meeting #166                                                        	S2-2412332

Nov 18-22, 2024, Orlando, US

Source:	Meta USA

Title:	Discussion on MMSID and traffic sync info to RAN

Document for:	Discussion

Agenda item:	19.3.2

# 1. Introduction

This paper discusses the information requested by RAN w.r.t “multi-modality awareness”, see (S2-2411300/ R2-2409272)

- Multi-modal Service ID to gNB

- Traffic synchronization with packet level synchronization information.

# 2. Discussion

## 2.1 Multi-modal Service ID

Multi-modal is supported in Rel-18 as described in TS 23.501, section 5.37.2. In general, data flows that are closely related to each other are corelated with an “Multi-modal Service ID”, which is given by AF via Nnef_AFsessionWithQoS service for each related data flow. With this information, PCF does the “magic” to assign the “right” PCC rules for those related data flows. In summary, the “Multi-modal Service ID” (MMSID) and QoS flow relationship is known within the PCF. To send this information to RAN, PCC rules need to be enhanced to pass MMSID to UPF (via SMF and N4). UPF then needs to include this information with a new GTP-U extension header (via N3) for those related data flows.

Observation: Including MMSID to gNb via N3 with GTP-U extension header is technically feasible and viable for Rel-19.

## 2.2 Packet level synchronization information

In high level, media synchronizing with RTP (RFC 3550) requires the following 3 data points:

- Synchronization source (SSRC): The source of a stream of RTP packets, identified by a 32-bit numeric SSRC identifier carried in the RTP header so as not to be dependent upon the network address.

- Timestamp: The timestamp reflects the sampling instant of the first octet in the RTP data packet.  This information in carried in the RTP header. Time stamp is media-specific RTP timestamp (i.e, stamp-rate is negotiated via SDP).

- Relative time between “wall clock” and RTP’s timestamp. This information is derived from RTCP (SR) sender report.

For 5GS, the encoder/sender (AS/AF) would also need to include additional information related to “synchronization threshold” and “media-specific RTP timestamp sampling rate information” between related data flows in order for 5GS to determine the “Packet level synchronization information”.

Observation: There are at least 5 pieces of information required by 5G Core Network to deduce “Packet level synchronization information”.

Although SSRC and timestamp can be observed by UPF, the “relative time” in (c), “synchronization threshold”, and “RTP timestamp sampling rate information” will have to be discussed in SA2 on how that information can be sent from AS/AF, and what type of information will be carried as part of the “Packet level synchronization information” to RAN. This additional task may require a study which means beyond Rel-19 timeframe [to be decided in SA2].

# 3. Conclusion & proposal

Agree to send MMSID to gNb via N3 with new GTP-U extension header.

To discuss whether Packet level synchronization information is achievable for Rel-19.