3GPP TSG-SA2 Meeting #166	S2-2411626

# 18th – 22nd November, 2024, Orlando, US                            (Revision of S2-2411194)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | 14 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE, OPPO, China Telecom, ETRI, Huawei, HiSilicon, Nokia, Futurewei, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper adding procedure of Vertical Federated Learning when NWDAF is acting as VFL server. ////////////////////////////////////////////////////////////////////////////////////////////////////// 1. The following two ENs are suggested to be removed. Because if dynamical reselection, addition, or removal of VFL client occurs, it means the training procedure will stop and the VFL model will have to be re-trained after performing sample and feature alignment with the new members of VFL, which obviously introduces much complex considering R19 time limitation and could be left to the next release. Editor’s Note: Whether and how to maintain a Vertical Federation Learning process including dynamical reselection, addition, or removal of VFL Client NWDAF(s) is FFS. Editor’s Note: the following is FFS. The VFL server determines with which VFL Client(s) to continue the VFL. The VFL server may provide to the VFL clients any sample ID(s) if changed. 2. The EN below can be removed by replacing “backward local ML model training information” with “intermediate model training information” based on the offline discussion. Editor’s Note: It is FFS if the term “backward local ML model training information” needs to be changed into “intermediate results”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause for procedure of Vertical Federated Learning when NWDAF is acting as VFL server Removed 3 ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3 (new), 6.2H.2.3.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CRs shall be implemented in the following order: CR 1198, CR 1246, CR1235, CR1208, CR1134 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

6.2H.2.3 Training Procedure for Vertical Federated Learning

6.2H.2.3.1 Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor's Note:  How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor’s Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor’s Note: It is FFS whether sample/feature information is required to be provided or updated in each training.

Editor´s Note: Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor’s Note:	Terminology will be aligned in the future.

0.	[OPTIONAL] If the VFL server containing both AnLF and MTLF, then step 0 is skipped.

An analytic consumer sends a subscription request to the NWDAF including Analytics ID, optionally Target of Analytics Reporting = e.g. UE IDs.

Either based on the information received, operator policy or internal configuration, VFL server decides to initiate VFL inference procedure with VFL clients.

Editor’s note: the following case (steps 0a and 0b) is FFS and shall be removed if trusted AF is to not be included in this clause. If NWDAF containing MTLF does not have a model and realizes it cannot be VFL server, the NWDAF sends a subscription request for a model to VFL server AF using Naf_VFLOperation_Subscribe including Analytics ID, optionally Notification target set to the first NWDAF, optionally Target of Analytics Reporting = e.g. UE IDs.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE 1: VFL Server may determine to perform VFL training based on the internal trigger or local configuration.

The steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to each of the selected NWDAF clients(s) and Naf_VFLTraining_Subscribe or Naf_VFLTrainingInfo_Request to the selected AF clients(s). The request includes VFL correlation ID, Analytics ID. Optionally, the VFL Server includes: VFL Interpretability Indicator, , Analytic filter information, , maximum response time(i.e. the maximum time between VFL clients receive backward local model information and send back intermediate training result).

NOTE 2: VFL Server determines the parameters Analytics ID, VFL Interpretability Indicator, based on local configuration comprising the possible values for such parameters agreed among vendors and/or application providers participating in the same VFL Group for specific VFL task(s).

If a VFL client is an untrusted AF, NWDAF as VFL server sends the VFL training request to the AF via NEF, the NEF may translate the analytic filter information if needed, e.g. TAIs into geographical area.

Editor´s Note: Analytics ID and Vendor-specific Feature ID might already have been indicated in the preparation phase and are then no longer required

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the VFL correlation ID, and intermediate model training information to each of the VFL clients for next round of VFL training.

NOTE: 	The initial local models for VFL clients may be provisioned by the VFL server by methods outside the scope of 3GPP.

NOTE 3:	In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference?

Editor´s Note: Additional Parameters to be provided in the request are FFS.

Editor´s Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

Editor’s Note:	It is FFS which service is used between VFL server and VFL clients for VFL training.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g., after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server.

Editor´s note: The following is FFS and may depend on the service design: When the clients reports the client intermediate training result, it also includes the corresponding VFL correlation ID.

5.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the intermediate model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor’s Note: Whether weight of the VFL Client is computed by VFL server is FFS.

Editor’s Note: Whether VFL server and VFL clients share feature information is FFS.

6.	[Optional] The NWDAF acting as VFL server evaluates (e.g. based on the convergence of a loss function or loss value, and/or if the pre-set iteration number is reached) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include model metric (e.g. ML model accuracy).

Editor’s Note: The content of the VFL status report is FFS.

Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

7. The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process, the termination message contains VFL Correlation ID.

The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Models. Additionally, the VFL server stores the VFL client information, which may be used to determine associated VFL client in the VFL inference.

In case of VFL client, it stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Models

NOTE X: The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

Editor’s Note: Whether VFL Training termination Flag in the termination message is required	 is determined after settling down the service operation.

NOTE 1:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as 	VFL Server and the untrusted AF is via NEF.

6.2H.2.3.2  Training Procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

The figure 6.2H.2.3.2-1 below shows the training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server.

Figure 6.2H.2.3.1-1Training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

Editor’s Note: The ENs listed in clause 6.2H.2.3.1 are also applied to this clause.

1.	Same as step 1 in 6.2H.2.3.1.

2.	To start VFL training, the VFL server do same as in step 1 in 6.2H.2.3.1, using Nnef_VFLOperation_Subcribe

The steps 3-7 are repeated until the training termination condition is reached.

3.	[Optional] Same as step 3 in 6.2H.2.3.1.

4.	Same as step 4 in 6.2H.2.3.1 to NEF using Nnwdaf_VFLOperation_Notify.

NEF forwards the subscription request to AF using Nnef_VFLOperation_Notify.

5.	Same as step 5 in 6.2H.2.3.1.

6.	[Optional] Same as step 6 in 6.2H.2.3.1, to NEF using Naf_VFLOperation_Notify.

NEF forwards the subscription request to NWDAF using Nnef_VFLOperation_Notify.

7.	Same as step 6 in 6.2H.2.3.1, to NEF using Nnef_VFLOperation_subscribe.

NEF forwards the subscription request to AF using Naf_VFLOperation_subscribe.

# >>>>END OF CHANGES<<<<