3GPP TSG SA WG2 Meeting #166	S2-2412394

# 18 - 22 November, 2024, Orlando , US				    						(Revision of S2-2411608)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1511 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of multiplexing multiple DC applications over single SCTP connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution adds the functionality requirements and procedures to support multiplexing multiple DC applications over single SCTP connection based on the KI#7 conclusion in TR 23.700-77. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add functionality requirements and procedure to support multiplexing multiple DC applications over single SCTP connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change  (all new texts)****

## AC.7.X	Multiplexing multiple DC streams over single SCTP connection

### AC.7.X.1			General

When multiple data channels are used in an IMS session, the UE and IMS network may support SDP negotiation to multiplex multiple data channel streams into a single data channel SDP media description m line and transport streams of multiple applications in the same SCTP connection, to save the number of m lines used by the UE and to improve the efficient usage of media resources.

The UE and IMS network also need to support data channel de-multiplexing when supporting data channel multipelxing. If the UE receives streams targeting to different applications over a single SCTP connection, it identifies the applications and delivers the data of the streams to corresponding applications. If the IMS network receives multiplexed streams which have different remote endpoints, e.g. a local bootstrap data channel and a remote bootstrap data channel of a UE, the MF terminates the multiplexing and sends different streams to corresponding endpoints based on instructions of DCSF.

The following multiplexing scenarios are supported in this release:

-	Multiplexing local bootstrap data channel and remote bootstrap data channel between the UE and its home IMS network using a single SDP m line media description if both support data channel multiplexing capability;

-	Multiplexing different application data channels for applications with compatible QoS requirements between the UE and its IMS network using a single SDP m line media description if both support multiplexing capability;

-	Multiplexing different application data channels for applications with compatible QoS requirements between the originating IMS network and terminating IMS network using a single SDP m line media description if both support DC multiplexing capability;

Multiplexing the data channels using the same stream ID are not supported.

The following de-multiplexing scenarios are supported in this release:

-	The supporting IMS network de-multiplexes local bootstrap data channel and remote bootstrap data channel from a SDP media description towards a UE that does not support multiplexing;

-	The supporting IMS network de-multiplexes application data channels from a SDP media description, if the data channels have different endpoints e.g. a P2P application and a P2A application or towards a UE that does not support multiplexing ;

-	The supporting originating IMS network de-multiplex data channels towards remote network if the remote network or remote UE does not support DC multiplexing.

If the UE decides to use data channel multiplexing, when the UE generates SDP offer in INVITE or re-INVITE request, the DC stream IDs associated with the data channels to be multiplexed and the binding information combined with the data channels are included in the SDP offer for a single m line.

NOTE 1:	How the application id and the DC stream IDs are included in the SDP is specified in SA WG4.

The IMS AS notifies the event of data channel multipelxing along with the data channel media information and binding information to the DCSF if data channel multiplexing is allowed for the subscriber.

The DCSF determines whether and how the data channels are multiplexed. When the DCSF determines that some data channels are allowed to be multiplexed, it instructs the IMS AS to multiplex the specific data channel streams to the same m line on the MF. The IMS AS further instructs MF to reserve a single media termination for the streams to be multiplexed.

If the DCSF determines that a data channel needs to be de-multiplexed from a SCTP connection, it instructs the IMS AS to de-multiplex the specific data channel stream on the MF. Based on the instruction of the DCSF, the IMS AS further indicates the MF to reserve separate media termination for the de-multiplexed stream in stead of using the same media termination with other multiplexed streams.

If the IMS AS is instructed by DCSF to use data channel multiplexing, when IMS AS generates SDP offer in INVITE or re-INVITE request to remote network, the DC stream IDs associated with the data channels to be multiplexed and the binding information combined with the data channels are included in the SDP offer for a single m line.

### AC.7.X.2			Capability negotiation

#### AC.7.X.2.1	Capability negotiation between UE and IMS network

The UE and IMS network may support DC multiplexing capability negotiation as follows:

The UE and the IMS network mutually negotiate their capabilities of data channel multiplexing during registration procedure.

If the UE supports data channel multiplexing, when the UE registers on the IMS network, the UE shall include a media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the initial and any subsequent REGISTER request.

If the UE supports data channel multiplexing, when the UE initiates a IMS session using data channel multiplexing, the UE shall include the media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the INVITE request.

If the P-CSCF supports data channel multiplexing, when the P-CSCF receives the REGISTER request from the UE, the P-CSCF shall include a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the initial and any subsequent REGISTER request.

NOTE:	The P-CSCF and IMS-AGW need to understand the multiplexed SDP to avoid gating the multiplexed media streams.

If the home IMS network supports this feature, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the 200 OK response to the initial and any subsequent REGISTER request.

If the IMS network supports this feature, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the SIP request messages to the remote network.

#### AC.7.X.2.2	Capability negotiation between originating and terminating network

The DCSF in originating IMS network shall determine whether the terminating IMS network supports data channel multiplexing by local configuration or based on the SDP answer received from remote network setting port of multiplexed DC media to zero.

Editor’s note:	Setting port of multiplexed DC media to zero does not apply to standalone IMS DC session, in which case the session is rejected by 488. Whether and how the originating network handle this scenario is FFS.

NOTE:	The DCSF can be configured to only allow multiplexing data channels between the UE and the IMS network, i.e. on UNI interface.

### AC.7.X.3			Determination and handling of multiplexing and de-multiplexing

#### AC.7.X.3.1	Determination of multiplexing and de-multiplexing

When the IMS AS receives the INVITE request with data channel multiplexing, if data channel multiplexing is supported, it reports the event to the DCSF for data channel management. The DCSF determines whether the data channel multiplexing is allowed and whether serving network needs to de-multiplex the data channel traffic based on subscription data, binding information, the endpoints of data channels and capability of terminating network.

#### AC.7.X.3.2 	Handling multiplexing and de-multiplexing at the oroginating network

The IMS AS and DCSF in the originating network follows the following principles to handle data channel multiplexing and de-multiplexing:

-	If the data channels are kept multiplexed, the DCSF instructs originating IMS AS accordingly. The originating IMS AS further instructs the MF to reserve the same media termination for multiplexed data channels. This attempt may succeed and IMS session establishment may be successful.

-	If the data channels are kept multiplexed, the DCSF instructs originating IMS AS accordingly. The originating IMS AS further instructs the MF to reserve the same media termination for multiplexed data channels. If the terminating network rejects the SDP offer because the terminating network does not support multiplexing, then originating IMS AS and originating DCSF needs to revert to a new demultiplexed SDP offer towards the target network as described in next bullet.

-	If the originating network determines to demultiplex the data channels is determined, the DCSF in the originating network instructs originating IMS AS accoirndgly. The originating IMS AS further instructs the MF to reserve separate media terminations for the de-multiplexed streams. When the originating IMS AS sends the SDP offer to terminating IMS network, the data channels are not multiplexed in the SDP offer. IMS session establsihment continues.

Editor’s note:	When a IMS network not supporting data channel multiplexing receives an SDP offer with multiplexed data channels for a standalone IMS DC session it rejects the session with 488. How the originating IMS network handles this scenario is FFS.

#### AC.7.X.3.3 	Handling at the terminating network.

The IMS AS and DCSF in the terminating network follows the following principles to handle data channel multiplexing and de-multiplexing: -

-	If the data channels are kept multiplexed, the terminating DCSF instructs terminating IMS AS accordingly. The terminating IMS AS further instructs the MF to reserve the same media termination for multiplexed data channels;

-	If the data channels is determined to be de-multiplexed, the terminating DCSF instructs terminating IMS AS accordingly. The terminating IMS AS further instructs the MF to reserve separate media terminations for the de-multiplexed streams. When the terminating IMS AS sends the SDP offer to terminating UE, the data channels are not multiplexed in the SDP offer.

### AC.7.X.4			Procedures

#### AC.7.X.4.1	Bootstrap data channel establishment

##### AC.7.X.4.1.1	Bootstrap data channels multiplexing in originating network

Figure AC.7.X.4.1.1-1 shows the procedure when the local bootstrap data channel and remote bootstrap data channel is multiplexed by the originating UE. The originating UE initiates an IMS session with multiplexed SDP for the two bootstrap data channels. The originating IMS network de-multiplexes the bootstrap data channel due to the two data channels have different endpoints.

Figure AC.7.X.4.1.1-1: Multiplexing bootstrap data channels in originating network

1.	When the UE wants to multiplex its bootstrap data channels, i.e. local bootstrap data channel and remote bootstrap data channel, the UE includes multiplexed SDP media description in initial INVITE request. The media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing is included in reINVITE request.

2-3.	The IMS AS selects and report event to DCSF.

4.	The DCSF determines based on stream id in the SDP that local bootstrap and remote bootstrap data channels need to be de-multiplexed.

5.	The DCSF generates data channel media information for originating side and terminating side.

6.	The DCSF instructs the IMS AS to reserve resources on MF.

7.	The IMS AS selects MF supporting data channel multiplexing.

8.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

9-12.	The session continues after the media resource is reserved successfully. The SDP offer in the INVITE request may also be multiplexed if needed.

13-16.	The terminating network responds and the session is established successfully.

The streams of local data channel and remote data channel are transported in the same SCTP connection between UE#1 and MF and are routed to DCSF and remote network correspondingly.

##### AC.7.X.4.1.2	Bootstrap data channel multiplexing in terminating network

Figure AC.7.X.4.1.2-1 shows the procedure when the terminating IMS network multiplexes the received remote data channel and the local bootstrap data channel towards the terminating UE.

In this procedure, when the terminating IMS network receives the SDP offer of remote bootstrap DC for terminating UE, it multiplexes this bootstrap dat channel with the local bootstrap data channel for terminating UE. When the terminating UE receives the multiplexed SDP offer, it replies with the multiplexed SDP answer to allow multiplexing of the two bootstrap data channels between terminating UE and terminating IMS network.

Figure AC.7.X.4.1.2-1: Multiplexing bootstrap data channels in terminating network

1.	The originating network sends initial INVITE request with SDP offer for remote bootstrap data channel to UE#2.

2-3.	The IMS AS selects and reports event to DCSF.

4.	The DCSF determines that local bootstrap and remote bootstrap data channels to UE#2 need to be multiplexed.

5.	The DCSF generates data channel media information for originating side and terminating side.

6.	The DCSF instructs the IMS AS to reserve resources on MF.

7.	The IMS AS selects MF supporting data channel multiplexing.

8.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

9-12.	The session continues after the media resource is reserved successfully.

13-16.	The UE#2 responds and the session is established successfully.

The streams of local data channel and remote data channel are transported in the same SCTP connection to UE#2.

#### AC.7.X.4.2	Application data channel establishment

##### AC.7.X.4.2.1	Application data channel multiplexing when both originating and terminating networks support data channel multiplexing

Figure AC.7.X.4.2.1-1 shows the procedure of multiplexing application data channels when both originating and terminating networks support data channel multiplexing.

In this example procedure, the originating UE multiplexes 3 applications all with different endpoints. The originating IMS network and terminating IMS network eventually de-multiplexes the applications based on the different endpoints. Since the originating network and terminating network both support data channel multiplexing, application data channels are multiplexed between originating network and terminating network.

Figure AC.7.X.4.2.1-1: Application data channel multiplexing when both originating and terminating networks support data channel multiplexing

1.	When the UE#1 wants to multiplex its application data channels for app#1, #2 and #3, the UE includes multiplexed SDP media description when sending reINVITE request. The endpoints of the applications are different, that of app#1 is DC AS in originating network, that of app#2 is DC AS in terminating network and that of app#3 is terminating UE. The media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing is included in reINVITE request.

2.	The IMS AS report event to DCSF.

3.	The DCSF determines that data channel for app#1 needs to be de-multiplexed because its target endpoint is local DC AS. The DCSF also determine that the multiplexing for app#2 and #3 is kept unchanged towards their target terminating network as described in AC.7.X.3.

4.	The DCSF generates data channel media information for originating side and terminating side.

5.	The DCSF instructs the IMS AS to reserve resources on MF.

6.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

7-8.	The session continues after the media resource is reserved successfully.

9-10.	The IMS AS generates reINVITE request to terminating network. The reINVITE request includes capability of supporting data channel multiplexing and the SDP offer for multiplexed data channels of app#2 and #3.

11.	The terminating network supports data channel multiplexing. The IMS AS in terminating network reports event to DCSF.

12.	The DCSF in terminating network determines that data channel for app#2 and #3 needs to be de-multiplexed as described in AC.7.X.3 because they have different endpoints,

13.	The DCSF generates data channel media information for originating side and terminating side.

14.	The DCSF instructs the IMS AS to reserve resources on MF.

15.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

16-17.	The session continues after the media resource is reserved successfully.

18-19.	The IMS AS generates reINVITE request to UE#2.

20-21.	The UE#2 replies 18X response with SDP answer for app#3.

22-23.	The IMS AS replies 18X response with capability indication of supporting data channel multiplexing and SDP answer for multiplexed data channels for app#2 and #3 to originating network.

24.	The IMS AS replies 18X response with SDP answer for multiplexed data channels for app#1, #2 and #3 to UE#1.

25-26.	The IMS AS reports event to DCSF.

27.	The session is established successfully. The streams of app#1, #2 and #3 are transported in the same SCTP connection between UE#1 and originating MF. The stream of app#1 is routed to DC AS and streams of app#2 and #3 are transported in the same SCTP connection between MFs of originating and terminating network and are routed to DC AS and UE#2 accordingly.

##### AC.7.X.4.2.2	Application data channel multiplexing when terminating network does not support data channel multiplexing

Figure AC.7.X.4.2.2-1 shows the procedure of multiplexing application data channels when terminating network does not support data channel multiplexing.

In this example procedure, the originating UE initiates a IMS session with audio and DC media components. The UE multiplexes 3 applications all with different endpoints. The originating network de-multiplexes app#1 because app#1 is terminated in originating network. The originating network further de-multiplexes app#2 and app#3 because the terminating network does not support data channel multiplexing and rejects the multiplexed SDP offer, by updating the session towards terminating network with normal SDP offer.

Editoe’s note:	The procedure for standalone IMS DC session is FFS.

Figure AC.7.X.4.2.2-1: Application data channel multiplexing when terminating network does not support data channel multiplexing

1.	When the UE#1 wants to multiplex its application data channels for app#1, #2 and #3, the UE includes multiplexed SDP media description when sending reINVITE request. The endpoints of the applications are different, that of app#1 is DC AS in originating network, that of app#2 is DC AS in terminating network and that of app#3 is terminating UE. The media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing is included in reINVITE request.

2.	The IMS AS report event to DCSF.

3.	The DCSF determines that data channel for app#1 needs to be de-multiplexed because its target endpoint is local DC AS. The DCSF also determine that the multiplexing for app#2 and #3 is kept unchanged towards their target terminating network as described in AC.7.X.3.

NOTE:	The DCSF can determines that terminating network does not support data channel multiplexing based on local configuration, In this case, the DCSF will determine to de-multiplex app#2 and app#3 and instruct IMS AS to send normal SDP offer to terminating network without the following steps.

4.	The DCSF generates data channel media information for originating side and terminating side.

5.	The DCSF instructs the IMS AS to reserve resources on MF.

6.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

7-8.	The session continues after the media resource is reserved successfully.

9-10.	The IMS AS generates reINVITE request to terminating network. The reINVITE request includes capability of supporting data channel multiplexing and the  SDP offer for multiplexed data channels of app#2 and #3.

11-12.	The terminating network does not support data channel multiplexing. The terminating network replies 18X with the port of media component for multiplexed data channels set to zero in SDP answer and no capability indication of supporting data channel multiplexing is included.

13-14.	The IMS AS determines that the terminating network does not support data channel multiplexing. It sends UPDATE request with normal SDP offer for app#2 and #3 to re-negotiate SDP with terminating network.

15-16.	The terminating network replies 200 OK (UPDATE) with SDP answer for app#2 and #3.

17.	The IMS AS reports the event of terminating network not supporting data channel multiplexing to the DCSF along with the media information of SDP answer.

18.	The DCSF instructs the IMS AS and the IMS AS further requests the MF to modify media resources for streams of app#2 and #3.

19.	The IMS AS replies 18X response with SDP answer for multiplexed data channels for app#1, #2 and #3 to UE#1.

20.	The session is established successfully. The streams of app#1, #2 and #3 are transported in the same SCTP connection between UE#1 and originating MF. The stream of app#1 is routed to DC AS and streams of app#2 and #3 are transported to terminating network and are routed to DC AS and UE#2 accordingly.

**** End of Changes ****