3GPP TSG-WG2 Meeting #166	S2-2412436

18th – 22nd November 2024, Orlando USA	(was S2-2410683)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1246 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 – Update of VFL training and inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ETRI, InterDigital, ICS, OPPO, Lenovo, Vivo, Nokia, Apple, Samsung, Futurewei, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solving multiple Editor’s Notes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes in 5.4: 1) Added that Vertical Federated learning is a machine learning techinique working without exchanging/sharing of local data set nor ML models 2) Editor's note: Support for AF acting as VFL server selecting multiple NWDAFs is FFS. Is solved by introducing: - An AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s)) to participate in a VFL procedure. 3) Editor's note: Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS. Is solved in procedure procedure for preparation phase Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. are solved in procedure clauses, where NWDAFs being part of inference have earlier been part of training. 4) Adding where the procedures are described Changes in 6.2F.2: Clarifying that some parameters are needed for VFL. Changes in rev.4 Solving parts of follwing EN Editor's note: Whether new information for interoperatbility when using VFL, such as the VFL Interoperability information, convergence reports or whether inference needs new indicators or not is FFS. Changes in 6.2H Added Preparation procedure Alignments including Sample and feature alignment is done in this phase Changes in rev.4 Solving the EN Editor´s note: Clarifications which parameters are regarded as requirements and can be modified by the VFL client is required by adding that this info is given in 6.2H.2 The last part of Editor´s note: It is ffs whether the VFL client can only reduce the requirements (e.g. samples) or also add new requirements. It is also ffs whether step 1 needs to be repeated if requirements have been reduced by the VFL client. Is solved, by adding that the decision is up to server (which might deem it to be necessary since it would like to change for all clients after receiviing first possible agreed requirements. Introduce External NWDAF ID, which replaces the discussed Temporary NWDAF ID. How NEF comes up with the External NWADF ID is implementation specifics. The External NWDAF ID is kept over long time including training and inference. See no need to introduce new flag so remove Editor’s Note: Whether to extend the existing ML Preparation Flag for reuse or introduce a new flag dedicated for VFL preparation is FFS. Since the service names will be discussed in coming meeting, uses the generic service name Nnf_VFLOperation while adding a new EN: #### Editor’s note: The service operation names used for VFL operations in the procedures below are FFS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL not implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.4, 6.2C.1; 6.2F.2, 6.2H.1, 6.2H.2, 6.2H.2.1, 6.2H.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Merges CR 1246 R2, CR 1246 R3, CR 1246 R5 and CR 1246 R6 into CR 1246 R4. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AI/ML	Artificial Intelligence/Machine Learning

DCAF	Data Collection Application Function

FL	Federated Learning

RE-NWDAF	Roaming Exchange Network Data Analytics Function

VFL	Vertical Federated Learning

HFL	Horizontal Federated Learning

* * * Next Change * * *

## 5.3	Horizontal Federated Learning (FL) among multiple NWDAFs

This clause describes Horizontal Federated Learning.

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAF for short), the main functionality includes:

FL server NWDAF:

-	discovers and selects FL client NWDAFs to participant in an FL procedure

-	requests FL client NWDAFs to do local model training and to report local model information.

-	generates global ML Model by aggregating local model information from FL client NWDAFs.

-	sends the global ML Model back to FL client NWDAFs to perform an additional training iteration if needed.

FL client NWDAF:

-	locally trains ML Model as tasked by the FL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other FL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the FL server NWDAF.

-	receives the global ML Model from FL server NWDAF and perform an additional training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML Model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML Model training, then FL procedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model. The FL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating FL server to initiate FL procedure on the FL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as described in clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using its local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

* * * Next Change * * *

## 6.2C	Horizontal Federated Learning among Multiple NWDAFs

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Horizontal Federated Learning technique to train an ML Model.

* * Next Change * * *

#### 6.2H.2.2	Preparation procedure for Vertical Federated Learning

#### 6.2H.2.2.0	General

The preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) to enable interoperability, sample alignment and may include feature negotiation if the VFL Server did not learn the supported FeatureIDs from each VFL Client using the discovery phase, alternatively the VFL Server may know the supported FeatureIDs by a VFL Client based on configuration. The Vertical Federated Learning preparation procedure can be skipped if the VFL Server can decide which VFL Client(s) support the VFL procedure to be performed, e.g. based on local configuration or offline procedures.

#### 6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registered or not and whether the NWDAF as VFL server can check whether UEs are registered before interacting with VFL clients.

Editor’s Note: Whether the case of trusted AF as VFL Server is included in the clause of NWDAF as VFL server or in the clause of untrusted AF as VFL server is FFS.

1.	The NWDAF as VFL Server may send a Vertical Federated Learning preparation request including the Analytics ID to each of the NWDAF VFL Client(s), using Nnwdaf_VFLTraining_Request and to each of the AF VFL Clients(s), using Naf_VFLTraining_Request possibly via NEF when the VFL Client is an untrusted AF. The NWDAF as a VFL Server also provides, the suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training, the suggested list of sample IDs that will be used in training.

2.	Each VFL Client checks if it can meet the ML Model training requirement. Each VFL Client ID checks the list of sample IDs suggested by the VFL Server and then provides to the VFL Server the list of sample IDs that it can accept out of the sample IDs suggested by the VFL Server. Each VFL Client checks the VFL Interoperability Information and determines which VFL Interoperability information that the VFL Client accepts. The VFL Clients provides the list of supported Feature IDs to the VFL Server, if available, alternatively the VFL clients may not want to reveal Feature IDs or VFL Server may know the supported FeatureIDs for a VFL Client based on configuration.

3.	Each NWDAF VFL Client invokes Nnwdaf_ VFLTraining_Response or and each AF VFL Client invokes Nnwdaf_ VFLTrainingRequest_Response, possibly via NEF when the AF is untrusted, to indicate to the VFL Server whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process.

4.	The VFL server determines the final list of samples considering the samples that all selected VFL clients support, if used the Feature ID per VFL client and VFL Interoperability Information to be used for training and provide them to the selected VFL Clients at the start of the training phase, as described in clause 6.2H.2.3.1.

#### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Editor´s note: It is FFS whether trusted AF can act as VFL clients when an untrusted AF acts as VFL Server.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

Editor´s note:	For the VFL preparation procedure, whether and how NEF does pre-work of sample IDs intersection before the VFL server determines the final sample IDs is FFS.

  - The untrusted AF as VFL server sends VFL preparation requests to each of the candidate VFL client(s), using Nnef_VFLTrainingRequest_Request with the ML Preparation Flag to check if the VFL client(s) can meet the ML Model training requirement (which includes the Analytics ID, list of sample IDs, and optionally Feature ID, etc.). The external NWDAF IDs obtained in the discovery procedure (see Clause 6.2H.2.1.1) is included to indicate the target NWDAFs. The suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training.

  - The NEF maps the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s). The NEF send VFL preparation request to the candidate internal NWDAF ID using Nnwdaf_ VFLTraining_Request service with the same information as provided in step 1 in 6.2H.2.2.1.

  - Same as step 2 in 6.2H.2.2-1.

  - Same as step 3 in 6.2H.2.2-1.

  - The NEF maps the internal NWDAF and SUPI(s) to the external NWDAF and GPSI(s). The NEF sends the Nnef_ VFLTraining_Request Response to the VFL Server with the same information as provided in step 4.

  - Same as step 4 in 6.2H.2.2-1.

* * * Next Change * * *

### 6.2H.x	Contents of ML Model Training service for Vertical Federated Learning

The consumers of the ML Model training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnefe_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	VFL Interoperability Information that indicates the intermediate results that the VFL Server supports (e.g. activation function, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this release.

- The list of samples selected by Server.

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	VFL Interoperability information supported by each VFL Clients.

-	The list of samples selected by VFL client.

-	The Feature ID(s) that the VFL client supports. A Feature ID indicates that what features the VFL client can use for an Analytics ID, the values of the Feature ID are not standardized.

Editor's note:	Which parameters are optional, or mandatory is FFS, the list of parameters also needs to be extended.

* * * End of Changes * * *