SA WG2 Meeting #166	S2-2412455

# 14 - 18 November 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1269 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | High level description refinement of NWDAF initiated VFL training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds some ‘VFL training termination’ related description to clause 5.4 to resolve the EN below in the NWDAF initiated VFL training procedure: Editor’s Note: Possible procedures to be executed at the end of the VFL training (to terminate a training session and to store/handle trained models) are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refine clause 5.4 by adding some VFL termination description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning techinique without exchanging/sharing local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AF.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

Editor's note:	Support for AF acting as VFL server selecting multiple NWDAFs is FFS.

-	requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL model correlation ID, and it requests to report intermediate results.

-	aggregates intermediate results from VFL client(s) and computes intermediate training results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training results towards VFL clients involved in the joint VFL training process.

-	determines to terminate the VFL training process.

-	stores VFL correlation ID and locally trained ML Model after VFL training process.

-	It initiates the VFL inference process using VFL model correlation ID.

-	It aggregates local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

VFL client:

-	locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients or VFL server due to e.g. data privacy, data security, data access rights.

-	computes the intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	stores VFL correlation ID and locally trained ML model after VFL training process.

-	performs inference based on the local model and local data and provides inference results to VFL server.

Editor's note:	Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS.

Editor's note:	Accuracy monitoring in VFL is FFS.

Editor's note:	For an NWDAF impacts of the split into AnLF and MTLF are FFS.

* * * End of changes* * *