3GPP TSG SA WG2#166	            S2-2412530

Orlando, USA,18-22 November 2024 	(revision of S2-2412484)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5764 | Rev | 6 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Slice change based on AF request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, ZTE, [Ericsson, Toyota, Nokia, NEC, Lenovo, Huawei] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the October meeting, due to the direction of narrowing the scope of TEI19, this CR proposes a solution to update the allowed NSSAI regardless of the support for Rel-18 slice replacement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces text to enable to trigger a slice replacement within a set of subscribed network slices based on AF request. AF sends AF requested slice replacement to AMF via PCF, and when AMF receives it, it updates the allowed NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the support of AF-requested slice change in the 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.5.2.2a (new Clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The DUMMY WI code needs to be replaced by the TEI19_XX WI on "New WID on Network Controlled Network Slice Selection", once allocated after the WI approval |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.15.5.2.2a	AF Requested modification of the Set of Network Slice(s) for a UE

In non-roaming cases, an authorized AF or an AF subject to authorization via the NEF, may request to replace a certain S-NSSAI (Replaced S-NSSAI) for a UE with another S-NSSAI (Alternative S-NSSAI) which is part of the UE subscription. The request in either case is sent to the PCF for the UE handling the impacted subscriber. Similarly, an authorized AF or an AF subject to authorization via the NEF, may request to terminate the replacement then request to use the Replaced S-NSSAI instead of the Alternative S-NSSAI for a UE again. The AF may subscribe to Notification on the outcome of the Network Slice Replacement.

Only a single AF can request the modification of the Set of Network Slice(s) for one replaced S-NSSAI.

NOTE 1: 	How the AF obtains the S-NSSAI to be replaced and the Alternative S-NSSAI is out of scope of this specification.

The PCF for the UE based on operator policies considers the AF request. Consequently the PCF for the UE triggers the Network Slice Replacement at the AMF as defined in TS 23.503 [45].The PCF sends Slice replacement management in the access and mobility management policies to the AMF per implicitly subscription, as described in TS 23.503 [45]. The Slice replacement management includes the Replaced S-NSSAI and the corresponding Alternative S-NSSAI as provided in the AF request, PCRT to request Notification on outcome of Network Slice Replacement request, together with a Network Slice Replacement Type. The Network Slice Replacement Type indicates that the Network Slice Replacement is requested by AF. The AMF handles the procedure as following:

Editor’s Note:	The value of Network Slice Replacement Type is FFS.

Editor’s Note:	Which PCRT to be used to request Notification on outcome of Network Slice Replacement request is FFS.

-	The AMF shall not handle the Network Slice Replacement procedure as described in clause 5.15.19

-	If the Alternative S-NSSAI is not within the UE subscription, or is not supported in the UE Registration Area, the AMF shall notify the PCF that the Network Slice Replacement is not triggered with a cause value. The PCF further notify the AF.

-	If the Alternative S-NSSAI is within the UE subscription and is supported in the UE Registration Area, the AMF includes the Alternative S-NSSAI in the Allowed NSSAI (if not present).

-	The AMF removes the Replaced S-NSSAI from the Allowed NSSAI and add the Replaced S-NSSAI into the Rejected S-NSSAI by a UE Configuration Update procedure,

Editor’s Note:	It is FFS whether the Replaced S-NSSAI should be removed from the Configured NSSAI.

-	The AMF releases the PDU sessions that were using the Replaced S-NSSAI

-	The AMF stores in the UE context the access and mobility management policy information as received from PCF, and transfer to new AMF during inter AMF mobility

NOTE 3: Operator policies in the PCF ensures that the UE is configured with a URSP rule that contains the Alternative S-NSSAI to establish a new PDU Session.

The trusted AF or a NEF may send request to PCF for the UE to stop the Network Slice Replacement. In this case the PCF for the UE sends access and mobility management policies to the AMF, as described in TS 23.503 [45]. The AMF remove the Replaced S-NSSAI from the Rejected S-NSSAI by a UE Configuration Update procedure. The UE may request to register the Replaced S-NSSAIs again next time the UE sends a Requested NSSAI.

End of CHANGES