3GPP SA WG2 Meeting #166 	S2-2412563

# 18 - 22 Nov 2024, Orlando, USA									       		  (revision of S2-2412545)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5098 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UPF event exposure service at N4 session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, ETRI, ZTE, Samsung, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - UPF event exposure service handling at PDU session release was integrated including UPF relocation case and was accepted in TS 23.501 at the last meeting. However, the related procedure was not included in TS 23.502 yet. - According to clause 5.8.2.17 of TS 23.501, if the Analytics filter information includes the Area of Interest or DNAI, the consumer shall not use direct subscription to UPF event. The clause 4.15.4.5.7 needs to be corrected to align with 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - N4 session release is handled in general. A Subscription termination reporting indication parameter and a Remaining data reporting indicator instead of Relocation report indication are used. - Change the Figure for clarification and simplification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment within the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

##### 4.15.4.5.7	Information flow for subscription directly to UPF event exposure service during N4 session release

Figure 4.15.4.5.7-1: Subscription directly to UPF event exposure service during N4 session release

- If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), steps 1-2 as described in Figure 4.15.4.5.5-1 are performed.

2.	The UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE identified by the UE IP address. The consumer may include a Remaining data reporting indication, which indicates how to handle the data collected by the source UPF when the N4 session of the UPF is released (e.g., due to UPF relocation or PDU session release). The consumer may include a Subscription termination reporting indication, which triggers the UPF to report Subscription termination to the consumer when N4 session is released.

3.	When the notification event is triggered, the UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

4.	When the N4  session corresponding to the UE is released, the UPF determines that the subscription to the UPF event has been terminated.

5.	If step 2 included Subscription termination reporting indication, the UPF sends a last notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination indication to notify that the subscription to the UPF event(s) has been terminated, and may include a cause code specifying the reason for subscription termination, e.g.  N4 session release. If step 2 included Remaining data reporting indication, the UPF may report to the consumer any data for the subscribed event, that has been collected but not sent to the consumer yet.

NOTE: 	N4 session release may be part of UPF relocation within the same PDU Session (see clause 4.3.5.3). The UPF event consumer that knows the new UE IP address allocated to the UE could send a new subscription to the target UPF as described in clause 4.15.4.5.5.

* * * End of Change * * *