3GPP SA WG2 Meeting #166	S2-2412610

# Orlando, US, 18-22 November 2024	               										    (was S2-2412236)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0484 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the diffusion of DNS information via MANET |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 4.2.8A: There is a missing text between “MANET” and “on” in the following sentence: “In addition, the 5G ProSe UE-to-UE Relay shall support a dedicated MANET on the PC5 interfaces towards…” The intent was to refer to the dedicated MANET Discovery Info message that is referenced in the NOTE further below: NOTE: The MANET Discovery Info message will be specified in Stage 3. Clause 5.14.3.2: Clause 4.2.8A refers to clause 5.14.3.2 for description of the propagation of DNS information, but there is no such information in clause 5.14.3.2. Clause 6.7.5.2.1: contains an Editor’s note related to possible other enhancements for propagation of DNS information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.2.8A: Added the missing “Discovery Info message” after “MANET”. Clause 5.14.3.2: Added clarification on the propagation of DNS information. Clause 6.7.5.2.1: Added a reference to clause 5.14.3.2 and deleted the Editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and incomplete text remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 4.2.8A; 5.14.3.2; 6.7.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	Void.

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	Void.

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[22]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[23]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[24]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	Void.

[27]	Void.

[28]	3GPP TS 38.351: "NR; Sidelink Adaptation Layer Protocol".

[29]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[30]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[31]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[32]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[33]	3GPP TS 22.268: "Public Warning System (PWS) requirements".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	IETF RFC 7181: "The Optimized Link State Routing Protocol Version 2 (OLSRv2)".

[36]	IETF RFC 6130: "Mobile Ad Hoc Network (MANET) Neighborhood Discovery Protocol (NHDP)".

[xx]	IETF RFC 5444: "Generalized Mobile Ad Hoc Network (MANET) Packet/Message Format".

NEXT CHANGE

### 4.2.8A	5G ProSe Multi-hop UE-to-UE Relay reference architecture

Figure 4.2.8A-1 shows the Layer-3 5G ProSe UE-to-UE Relay reference architecture. The 5G ProSe End UEs communicate with each other via a set of interconnected 5G ProSe Multi-hop UE-to-UE Relays.

Figure 4.2.8A-1: Reference architecture for Layer-3 5G ProSe Multi-hop UE-to-UE Relay

Based on capability and configuration the 5G ProSe UE can act as a 5G ProSe End UE, a 5G ProSe Multi-hop UE-to-UE Relay or both. Each 5G ProSe End UE and the 5G ProSe UE-to-UE Relay may have subscriptions from the same PLMN or different PLMNs.

-	For IP PDU type, the 5G ProSe Multi-hop UE-to-UE Relays form a MANET network as defined in IETF RFC 7181 [35]. A 5G ProSe UE acting as 5G ProSe UE-to-UE Relay shall support the protocols defined in IETF RFC 7181 [35] and IETF RFC 6130 [36] on the PC5 interfaces towards other 5G ProSe Multi-hop UE-to-UE Relays. In addition, the 5G ProSe UE-to-UE Relay shall support a dedicated MANET Discovery Info message on the PC5 interfaces towards other 5G ProSe Multi-hop UE-to-UE Relays for propagation of DNS information, as described in clause 5.14.3.2.

NOTE:	The MANET Discovery Info message will be specified in Stage 3.

NEXT CHANGE

#### 5.14.3.2	IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The RSC configuration as defined in defined in 5.1.5.1 indicates if IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication is used. The 5G ProSe Multi-hop UE-to-UE Relays supporting the same RSC establishe IP based Layer-2 links with each other and form a relay cloud to provide IP connectivity for 5G ProSe End UEs connected to any of the 5G ProSe Multi-hop UE-to-UE Relays.

NOTE:	If a 5G ProSe Multi-hop UE-to-UE Relay support multiple RSCs, different Layer-2 links will be established, with each associated with a specific RSC. The 5G ProSe UE-to-UE Relay operates as separate logical entities in the separate relay clouds of different RSCs, i.e. no routing table sharing and no traffic forwarding across different relays clouds of different RSCs.

The 5G ProSe End UE may establish multiple Layer-2 links with different 5G ProSe UE-to-UE Relays supporting the same RSC, in order to maximize the reachability (e.g. in some cases, different 5G ProSe UE-to-UE Relays may serve different disjointed IP subnets).

Mobile Ad-hoc Network (MANET) based routing protocols are used over the IP connectivies in the relay cloud to provide multi-hop forwarding functionality between 5G ProSe End UEs. Each of the 5G ProSe Multi-hop UE-to-UE Relay acts as a MANET router. The exact MANET routing protocols to be supported, e.g. Neighborhood Discovery Protocol (NHDP) [36], OLSRv2 [35], are associated with the RSCs based on configuration.

Based on configuration associated with the RSC, the 5G ProSe End UE obtains the IP address/prefix in two options:

-	Each of the 5G ProSe End UE may be configured with a routable IP address/prefix associated with the RSC. This IP address/prefix does not change when the 5G ProSe End UE changes the connections with 5G ProSe Multi-hop UE-to-UE Relays.

-	If the 5G ProSe End UE does not have an IP address/prefix configured for the RSC, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay it connects to. In this case, the 5G ProSe End UE may need to change the IP address/prefix when it changes its connection to another 5G ProSe UE-to-UE Relay.

The 5G ProSe End UE obtains the IP address/prefix of the target 5G ProSe End UE via DNS queries. 5G ProSe Multi-hop UE-to-UE Relay needs to support DNS operations.

Once the 5G ProSe Multi-hop UE-to-UE Relay becomes aware of the 5G ProSe End UE’s IP address/prefix, the 5G ProSe Multi-hop UE-to-UE Relay propagates the related DNS information (i.e. User Info ID and associated IP address/prefix) to all other 5G ProSe Multi-hop UE-to-UE Relays connected to the same MANET using a dedicated MANET Discovery Info message. The MANET Discovery Info message is diffused throughout the MANET using Multipoint Relay flooding (MPR flooding) as defined in IETF RFC 7181 [35], if available. If MPR flooding is not available, then the MANET Discovery Info message is diffused using simple flooding i.e. by transmitting a copy of the MANET Discovery Info message on all interfaces except the one on which the message has been received.

The MANET Discovery Info message is based on the generalized MANET message format defined in IETF RFC 5444 [xx]. The 5G ProSe Multi-hop UE-to-UE Relay initiating the sending of the MANET Discovery Info message controls the propagation depth of the MANET Discovery Info message based on the configured "maximum number of hops per RSC" as specified in 5.1.5.1a, by using the built-in mechanism defined in IETF RFC 5444 [xx].

When MANET protocol determines the route for IP forwarding, layer-2 link QoS and hop counts can be refelected, e.g. with the Link Metrics value as defined in OLSRv2 [35]. The 5G ProSe Multi-hop UE-to-UE Relay can also enforce the configured hop limit based on the link metrics when building the routing tables, e.g. ignore the route that exceeds the threshold derived from hop limit.

If 5G 5G ProSe Multi-hop UE-to-UE Relay has traffic for its own, it can use any IP address it owns for the transmission.

NEXT CHANGE

#### 6.7.5.2	Communication via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays (IP type)

##### 6.7.5.2.1	Layer-2 link establishment via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays (IP type)

Figure 6.7.5.2.1-1 shows the Layer-2 link establshment procedure via 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays. This procedure is used by an 5G ProSe End UE to connect to the relay cloud, or for a 5G ProSe Layer-3 Multi-hop UE-to-UE-Relay to connect to other 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays.

Figure 6.7.5.2.1-1: 5G ProSe Communication via IP type of 5G ProSe Multi-hop UE-to-UE Relay

0.	The 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays and the other 5G ProSe End UEs have already established IP type of connections. The 5G ProSe Layer-3 Multi-hop UE-to-UE-Relays and 5G ProSe End UEs may have used the discovery procedure as defined in clause 6.3.2.6.2, and established the Layer-2 link as defined in clause 6.4.3.10.1.

1.	The 5G ProSe End UE performs the 5G Multi-hop UE-to-UE Relay discovery procedure as described in clause 6.3.2.6.2, and finds the 5G Multi-hop UE-to-UE Relay-1.

2.	The 5G ProSe End UE establishes an IP based Layer-2 link with the 5G ProSe Multi-hop UE-to-UE Relay-1 as described in clause 6.4.3.10.1.

Depends on the configuration, the 5G ProSe End UE may obtain an IP address/Prefix from the 5G ProSe Multi-hop UE-to-UE Relay-1. Otherwise, the 5G ProSe End UE informs the 5G ProSe Multi-hop UE-to-UE Relay-1 of its pre-configured IP address/prefix.

The 5G ProSe End UE may also configure the corresponding services and PC5 QoS Flows over the Layer-2 like with the 5G ProSe Multi-hop UE-to-UE Relay-1. The 5G ProSe End UE may use the Layer-2 link modification procedure as defined in clause 6.4.3.4 to add/modify/remove PC5 QoS Flows corresponding to the services and link status.

3.	The 5G ProSe Multi-hop UE-to-UE Relay-1 updates its records of the 5G ProSe End UEs and may trigger the MANET signalling, e.g. OLSRv2 signalling [35], to update other connected 5G ProSe Multi-hop UE-to-UE Relays. This will result in a routing table on the relays that allows other UEs to reach the newly connected 5G ProSe End UE.

The 5G ProSe Multi-hop UE-to-UE Relay may also trigger signalling to update the DNS entries in other connected 5G ProSe Multi-hop UE-to-UE Relays for the 5G ProSe End UE using the MANET Discovery Info message, as described in clause 5.14.3.2.

4.	The 5G ProSe End UE may perform a DNS query for a target 5G ProSe End UE, if it does not know the IP address/prefix of the target. The DNS query may stop at the 5G ProSe Multi-hop UE-to-UE Relays that has the entry for the target 5G ProSe End UE.

5G ProSe End UE starts to communicate with the target 5G ProSe End UE using IP based communications. The 5G ProSe Multi-hop UE-to-UE Relays forwards the packets based on the routing table established with the MANET protocols.

End of CHANGEs