3GPP TSG-SA2 Meeting #166	S2-2412611

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024					   was S2-2412268

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0518 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-hop UE-to-Network Model B Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V., Huawei, HiSilicon, Samsung, InterDigital, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedures to enable Multi-hop ProSe UE-to-Network Relay with Model B still contain ENs that need to be resolved, and a few missing aspects that need to be specified such as Layer-2 link management procedures. Additionally, the path information is required in the the 5G ProSe UE-to-Network Relay Discovery Response message for Model B since it is mandatory in the Direct Communication Request message which is sent by the 5G ProSe Remote UE as specified in clause 6.4.3.9: - 5G ProSe Remote UE sends a unicast Direct Communication Request message to the selected 5G ProSe Intermediate Relay. The Direct Communication Request message additionally includes: - Path information: an (ordered) list of User Info ID of 5G ProSe Intermediate Relays and the UE-to-Network Relay selected by the 5G ProSe Remote UE based on the path information provided to the 5G ProSe Remote UE during 5G ProSe UE-to-Network Relay Discovery procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Make path information mandatory and add discoverer information in the Discovery Response message for model B. - Complete Layer-2 link management procedures, resolving the related EN and including IP address management and relay reselection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications for supporting multi-hop UE-to-Network relay with Model B |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.1, 6.4.3.9, 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

#### 5.8.3.1	Common identifiers for 5G ProSe UE-to-Network Relay

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Announcer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Announcer Info: provides information (i.e. User Info ID) about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

The 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A) is extended and modified as follows when used for 5G ProSe multi-hop UE-to-Network Relay discovery:

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the 5G ProSe UE-to-Network Relay Discovery Announcement message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the 5G ProSe UE-to-Network Relay Discovery Announcement if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID when sending the discovery message.

-	Destination Layer-2 ID: the 5G ProSe Intermediate UE-to-Network Relay sets the Desination Layer-2 ID of the 5G ProSe UE-to-Network Relay Discovery Announcement message based on configuration described in clause 5.1.4.1.

-	Announcer Info: identify information (i.e. User Info ID) of the announcing 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay.

-	(optinal) Root Relay Info: this is the User Info ID of the 5G ProSe UE-to-Network Relay. A 5G ProSe Intermediate UE-to-Network Relay may stored it in the discovery entry and included it in the 5G ProSe UE-to-Network Relay Discovery Announcement message. This information can be used in the relay selection at the 5G ProSe Remote UE or other 5G ProSe Intermediate UE-to-Network Relays.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links to the Root Relay, i.e. the 5G ProSe UE-to-Network Relay, for this RSC.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

Editor's note:	Whether the same 5G ProSe UE-to-Network Relay Discovery Announcement message is reused for multi-hop operation or a new message type is to be defined will be decided by Stage 3.

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Solicitation message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoverer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe Remote-UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Discoverer Info: provides information (i.e. User Info ID) about the discoverer user.

-	Target Info: provides information (i.e. User Info ID) about the targeted discoveree user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the 5G ProSe Remote UEs interested in related connectivity services.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: a unmodified value that indicates the hop limit of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than the (pre)configured maximum number of hops.

-	Path information: an (ordered) list of User Info ID(s) of Intermediate UE-to-Network Relay(s) that indicates the transmitted path of the message.

The following parameters are used in the 5G ProSe UE-to-Network Relay Discovery Response message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoveree Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-Network Relay Discovery Solicitation message.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-Network Relay provides to 5G ProSe Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information (i.e. User Info ID) about the discoveree.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop count: indicates the number of hops between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay on the path selected by the 5G ProSe UE-to-Network Relay.

-	Path information: an (ordered) list of User Info ID(s) of the discoverer user and the Intermediate UE-to-Network Relay(s) on the path selected by the 5G ProSe UE-to-Network Relay.

The following parameters may be used in the Relay Discovery Additional Information message (using Model A) based on the procedure defined in clause 6.5.1.3 for 5G ProSe UE-to-Network Relay and clause 6.3.2.5.4 for 5G ProSe multi-hop UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID to send the Relay Discovery Additional Information message.

-	Destination Layer-2 ID: the Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user (i.e. User Info ID of the 5G ProSe UE-to-Network Relay).

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the Relay Discovery Additional Information message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the Relay Discovery Additional Information message if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identify information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay. This parameter is only applicable to the additional parameter announcement procedure over 5G ProSe multi-hop UE-to-Network Relay specified in clause 6.3.2.5.4.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

-	Additional parameters: the additional parameters for 5G ProSe Layer-3 UE-to-Network Relay (when applicable) are defined in clause 5.8.3.2.

NOTE 1:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication) in clause 6.4 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) with a different discovery model.

NOTE 2:	If a 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE from different PLMNs discover each other, it means that the Relay Service Code is associated with the same connectivity service, and the same Relay Service Code is provisioned based on Service Level Agreement among PLMNs.

NOTE 3:	The Hop count, Hop-Limit and Path information are used for multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B. The multi-hop UE-to-Network Relay Discovery message has no impact on 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE that only supports the single-hop UE-to-Network Relay. Single-hop and Multi-hop UE-to-Network Relays discoveries can be distinguished based on RSC.

* * * * Next Change * * * *

#### 6.4.3.9	Layer-2 link management over PC5 reference point for Multi-hop 5G ProSe UE-to-Network Relay (based on Model B Discovery)

##### 6.4.3.9.1	Layer-2 Link establishment and management

The Layer-2 link procedures over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.6 is used for the PC5 reference point among the 5G ProSe Remote UE, the 5G ProSe Intermediate UE-to-Network Relay and the UE-to-Network Relay after Model B Discovery, with the following differences and clarifications:

For the UE oriented Layer-2 link establishment as described in the clause 6.4.3.1,

-	The 5G ProSe Remote UE determines the destination Layer-2 ID for PC5 unicast link establishment based on the unicast source Layer-2 ID of the selected (neighbour) 5G ProSe Intermediate Relay (as specified in clause 5.8.3) during UE-to-Network Relay discovery as specified in clause 6.3.2.5.3.

-	The 5G ProSe Intermediate UE-to-Network Relay determines the destination Layer-2 ID for PC5 unicast link establishment based on the unicast source Layer-2 ID of the selected 5G ProSe Intermediate Relay or UE-to-Network Relay (as specified in clause 5.8.3) during UE-to-Network Relay discovery as specified in clause 6.3.2.5.3.

NOTE: 	It is up to implementation how long the 5G ProSe Intermediate UE-to-Network Relay keeps the Layer-2 ID information of other realys/Remote UE obtained in the discovery procedures. E.g., until it receives the a certain number of Response message(s) towards the Remote UE or based on an implementation timer.

-	5G ProSe Remote UE sends a unicast Direct Communication Request message to the selected  (neighbour) 5G ProSe Intermediate UE-to-Network Relay. The Direct Communication Request message additionally includes:

-	Path information: an (ordered) list of User Info ID of 5G ProSe Intermediate Relays and the UE-to-Network Relay selected by the 5G ProSe Remote UE based on the path information provided to the 5G ProSe Remote UE during 5G ProSe UE-to-Network Relay Discovery procedure.

-	QoS Info: indicates the End to End QoS Info.

-	5G ProSe Intermediate UE-to-Network Relay sends a unicast Direct Communication Request message to the next 5G ProSe Intermediate UE-to-Network Relay or the UE-to-Network Relay according to the path information in the received Direct Communication Request message. The Direct Communication Request message additionally includes:

-	Path information: an (ordered) list of User Info ID of 5G ProSe Intermediate UE-to-Network Relays and the UE-to-Network Relay selected by the 5G ProSe Remote UE based on the path information provided to the 5G ProSe Remote UE during 5G ProSe UE-to-Network Relay Discovery procedure.

-	QoS Info: End to End QoS Info and the remaining QoS Info of hops from the Intermediate UE-to-Network Relay to the network as specified in clause 5.6.2.

-	In step 4 and step 5, step 4a and step 5a are performed if the 5G ProSe Intermediate/UE-to-Network Relay's identity matches the Target Info (if any) and the Relay Service Code is one of the Relay Service Codes included during UE-to-Network Relay discovery as specified in clause 6.3.2.5.3.

For the Layer-2 link release as described in the clause 6.4.3.3,

-	If the Layer-2 link release procedure is initiated by the 5G ProSe Intermediate UE-to-Network Relay, the Disconnect Request message may indicate the 5G ProSe UE-to-Network Relay is temporarily not available as described in clause 5.12.

-	If the service authorization for acting as a 5G ProSe Intermediate UE-to-Network Relay is revoked, the 5G ProSe Intermediate UE-to-Network Relay should initiate the release of the Layer-2 link that the revoked authorization affects.

Each PC5 unicast link for 5G ProSe UE-to-Network Relay is associated with a Unicast Link Profile, which additionally includes:

-	Path Information: which contains the User Info ID of Intermediate UE-to-Network Relay at the next hop to the Remote UE.

Each PC5 unicast link for 5G ProSe Intermediate UE-to-Network Relay is associated with a Unicast Link Profile, which additionally includes:

-	Path Information: which contains the User Info ID of Intermediate UE-to-Network Relay or UE-to-Network Relay at the adjancent hop.

##### 6.4.3.9.2	IP address/prefix management for 5G ProSe Layer-3 Multi-hop UE-to-Network Relay

When IP based PDU session types are used by the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay acts as an IP Router.

The IP address allocation is performed by the 5G ProSe UE-to-Network Relay, as defined in clause 6.5.1.

DHCP based IP address configuration and IPv6 routing advertisement is performed as described in clause 6.4.3.8.2.

After IP address assignment, the Remote UE may initiate a Link Modification Procedure which includes the Remote UE ID and the Remote UE information. The 5G ProSe Intermediate UE-to-Network Relays builds their local IP routing table based on the Remote UE info.

##### 6.4.3.9.3	Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE and 5G ProSe Intermediate UE-to-Network Relay keep performing the measurement of the signal strength of PC5 unicast link with its parent Relay (Intermediate Relay or U2N relay) for relay reselection.

Relay reselection is triggered if a 5G ProSe UE-to-Network Relay or Intermediate UE-to-Network Relay is no longer available, if a connection with the previous or next relay is lost, or if the link quality drops below a configured threshold.

When a 5G ProSe Intermediate UE-to-Network Relay detects the link failure or link quality degradation, the 5G ProSe Intermediate UE-to-Network Relay informs the 5G ProSe Remote UE of the link failure or link quality degradation of the PC5 link so that the 5G ProSe Remote UE may initiate multihop UE-to-Network Relay reselection procedure.

The 5G ProSe Remote UE may perform the 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 to discover available 5G ProSe UE-to-Network Relays or 5G ProSe multi-hop UE-to-Network Relay Discovery procedures defined in clause 6.3.2.5.3 to discover available multihop paths to the 5G ProSe UE-to-Network Relays for 5G ProSe (multihop) UE-to-Network Relay reselection.

After the 5G ProSe multihop UE-to-Network relay reselection procedure, if the same 5G ProSe UE-to-Network Relay is used for connection with a 5G ProSe Remote UE, the existing IP address of the 5G ProSe Remote UE is reused. The IP address of the Remote UE may be included by the 5G ProSe UE-to-Network Relay and Intermediate UE-to-Network Relays in the DCA/LMA message.

After a preconfigured duration, the Relay that detected the need for reselection releases the link with the previous and next relays if applicable, unless it is reselected by the Remote UE. This will trigger them to perform the same procedure until the complete path has been released. Then, the 5G ProSe UE-to-Network Relay removes its association with the 5G ProSe Remote UE including the allocated IP address.

* * * * Next Change * * * *

### 6.5.3	5G ProSe UE-to-Network Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE keeps performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-Network Relay for relay reselection.

The 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 are used to discover available 5G ProSe UE-to-Network Relays for 5G ProSe UE-to-Network Relay reselection.

For 5G ProSe Multi-hop UE-to-Network Relay, 5G ProSe UE-to-Network Relay reselection is performed as described in clause 6.4.3.8.3 for model A discovery, and clause 6.4.3.9.3 for model B discovery.

* * * * End of Changes * * * *