3GPP TSG-WG SA2 Meeting #166	S2-2412623

# Orlando, USA, 18th Oct – 22nd Nov, 2024		(revision of S2-2411801)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5845 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB-gNB configuration during MWAB mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to R3-245840, the following descriptions are endorsed in RAN3: “The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location” This CR introduces the descriptions on MWAB-gNB configuration during MWAB mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description regarding MWAB-gNB configuration during MWAB mobility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MWAB feature on Configuration and Mobility are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.2.2, 5.49.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.49.2.2	Configuration of the MWAB-gNB

The MWAB-gNB's configuration is managed by the OAM of the MWAB Broadcasted PLMN/SNPN, and the address (or FQDN) of the OAM server per MWAB Broadcasted PLMN/SNPN may be configured at the MWAB-gNB. The OAM server of the MWAB Broadcasted PLMN/SNPN provides (re-)configuration parameters considering geographical information of MWAB as specified in TS 38.401 [42], e.g. for the purpose including the access stratum operation of the MWAB-gNB, the N2, N3 and Xn interface management, activating/deactivating the MWAB-gNB operation, and to assist the MWAB-gNB providing information used by MWAB-UE for the BH PDU Session(s) management via URSP processing.

NOTE: 	It is expected the OAM of the MWAB Broadcasted PLMN/SNPN is aware of location in which the MWAB operation can be authorized and also can obtain MWAB’s current location. Only when the MWAB is authorized successfully, the configuration information can be provisioned to the MWAB-gNB. Alternatively, the OAM can also provision configuration parameters related to different areas, as defined in TS 38.401 [42].

Editor's note:	Details of the configuration information needs the coordination with SA WG5 and RAN WG3.

Second CHANGE

5.49.5.3	Impact of MWAB mobility

The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect.

-	The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE.

-	During Mobility, a MWAB-gNB can obtain or apply a new configuration as specified in TS 38.401 [42] from the OAM server of MWAB Broadcasted PLMN and, if any new AMF was provided as part of the new configuration, the MWAB-gNB sets up the N2 connection with the new AMF and it releases the N2 connection with any old AMF removed from the MWAB-gNB configuration.

-	The MWAB-UE may request new or modify existing BH PDU Sessions for the corresponding N2, N3 and Xn interfaces, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new BH PDU Session(s) as needed.

-	SSC mode 3 may be applied to the BH PDU Sessions to minimize traffic interruption when UPF re-allocation for these BH PDU Sessions is required during the MWAB mobility.

-	When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change.

End of CHANGES