3GPP TSG-WG SA2 Meeting #166	S2-2412668

# Orlando,FL, USA 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | 0495 | rev | 11 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Monitoring event for S&F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Sateliot, China Telecom, NEC, CATT, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2024-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions agreed in the TR 23.700-29 for KI#2 this CR proposes to introduce monitoring events agreed as part of conclusions.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Monitoring event for S&F feature is implemented based on the conclusions reached for KI#2 in TR 23.700-29. This revision over agreed version in SA2#165 proposes to resolve this EN: Editor’s note: Whether feeder link availability period will be provided to SCS/AS is FFS. There are two choices: a) Introduce this feeder link availability period parameter (as proposed in this CR – highlighted with yellow); or b) Remove the EN with a conclusion that this parameter is not required. After the discussion, this CR can be updated in one of the above directions. As a compromise for companies who are not interested in feeder link availability period parameter, this parameter can be kept optional. This implies: a) If this parameter is included, AF can accurately know when to send the data and delay in sending data b) If this parameter is not included then AF will have to estimate that data will be delivered to the UE with the max time of “Estimated time to delivery” and this Estimated time to delivery network also considers the feeder link link availability time. For e.g.: Consider: a) Delivery time of data when feeder link available is 1 hour. b) Feeder link(FL) is available at 10 O clock and next FL availability time is after 6 hours.. a) If feeder link availability information is provided(i.e. at 10) to the AF, then AF is aware based on when it has sent the data(e.g. at 9) UE will receive the data(e.g. by 11). b) If feeder link availability information is not provided. Then estimated delivery time value considering FL availability is 7 hours(1 hour to send data + 6 hours maximum time for FL to establish depeding on when data is sent to CN) AF will assume data will be delivered in 7 hours without accurately knowing when it can be delivered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Monitoring event for S&F features is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)5.6.1.X, (new)5.6.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes *********************

#### 5.6.1.X	Specific Parameters for Store and Forward Satellite operation information

This monitoring event allows the SCS/AS to be notified of Store and Forward Satellite operation information, identified by EPS as per TS 23.401 [7] clause 4.13.X.

1.	The SCS/AS sets Monitoring Type to "Store and Forward Satellite operation information" prior to sending Monitoring Request to the SCEF as in step 1 of clause 5.6.1.1.

2.	The SCEF executes step 2 of clause 5.6.1.1.

3.	The SCEF executes step 3 of clause 5.6.1.1.

4. The HSS executes step 4 of clause 5.6.1.1.

5.	The HSS executes step 5 of clause 5.6.1.1.

6.	The MME executes step 6 of clause 5.6.1.1 and starts watching for Store and Forward Satellite operation information events.

7-9.	Steps 7-9 of clause 5.6.1.1 are executed.

*************************** Next Changes **********************

#### 5.6.3.X	Reporting Event: Store and Forward Satellite operation information

1a.	This monitoring event is detected as of step 1a of clause 5.6.3.1, which is when the MME detects that the UE is registered in S&F mode, moving from S&F mode to not registered in S&F mode(see TS 23.401 [7] clause 4.13.X) or there is change in previously reported values of Downlink S&F estimated Delivery Time and/or the feeder link availability period.

2a.	Step 2a of clause 5.6.3.1 is executed.

3.	Step 3 of clause 5.6.3.1 is executed. Depending on operator configuration, the MME sends to the SCS/AS that a UE is registered in S&F Mode or moving from S&F mode to not registered in S&F mode, a Downlink S&F estimated Delivery Time and the feeder link availability period.

If feeder link availability period is not included, the Downlink S&F estimated Delivery time is the estimated/expected time required to deliver the data to the UE from the time data has been received in the network. If feeder link availability period is included, the Downlink S&F estimated Delivery time is the estimated/expected time required to deliver the data to the UE after feeder link is established.

*************************** End of Changes *********************