3GPP TSG-SA WG2 Meeting #166	S2-2412673

# Orlando, USA, November 18-22, 2024										(revision of S2-2412450)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1522 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility procedure for UE-Satellite-UE communication in IMS – ground fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO[, Nokia] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3_ARCH work item and the conclusion of FS_5GSAT_Ph3_ARCH captured in clause 8.3 of TR 23.700-29, this CR adds description on Mobility procedure of UE-Satellite-UE communication in IMS focusing on ground fallback. This CR is written based on the following principle as well as the above-mentioned conclusion. - Use of the N5 interface between IMS and 5GC is assumed. - IMS AS, requested by P-CSCF via a message, sends SIP re-INVITE to complete IMS AGW relocation. (P-CSCF does not need to act as SIP B2B UA.) - No UE impact. - No new SDP attribute is introduced. (A new SIP header is introduced for conveying satellite ID.) - Subscription data is not considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Mobility procedure of UE-Satellite-UE communication in IMS focusing on ground fallback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.5.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428 and CR 1521. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

AX.5.2.2	Ground fallback procedure

Editor's note:	Subject to the TSG SA decision, SA2 considers a P-CSCF solution (i.e., a solution that relies on P-CSCF initiating SIP re-INITE) during the initial exception phase to finalize the mobility procedure. How to conclude this consideration is FFS.

Figure AX.5.2.2-1 depicts a signalling flow diagram for the case that optimized media routing is abandoned and the routing with the media transiting through the ground segment is selected after change of satellites that serve a UE. The figure is written in such a way that change of satellite occurs in the originating network for the purpose of the explanation, while change of satellite can occur in the terminating network as well.

NOTE 1:	IMS entities not relevant for the procedure are omitted below for brevity of the description.

NOTE 2:	In this Release of the specification, the originating network and the terminating network in clause AX.5.2.2 are the same PLMN.

Use of the N5 interface between IMS and 5GC is assumed.

Based on the procedure described in clause AX.5.1, P-CSCF is expected to subscribe from 5GC for the early and the late notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic by using a scheme defined in TS 23.501 [93] and TS 23.502 [94].

Figure AX.5.2.2-1:	Ground fallback procedure

The steps in the call flow are as follows:

1.	Step 1 of clause AX.5.2.1 applies.

2.	P-CSCF receives the early notification of the satellite user plane management events associated with UE-Satellite-UE communication media traffic from PCF as defined in TS 23.502 [94]. This early notification contains an indication being set "EARLY". When this early notification contains satellite ID of a target satellite that has gNB to which the UE gets connected, this indication indicates that 5GC is prepared to change the user plane paths for optimized media routing to the one through this target satellite.

3.	P-CSCF determines that optimized media routing cannot continue if there is no target satellite ID included in the early notification as received in step 2, or the two satellites identified by the satellite ID received in step 2 for the originating network and the satellite ID stored for the terminating network has no ISLs. The P-CSCF determines to activate ground fallback routing.

NOTE 3:	How P-CSCF uses the satellite IDs to derive whether the two satellites are connected and whether optimized media routing is possible are not specified.

4.	P-CSCF requests IMS AGW on ground to configure the IP address allocated in UE, which the P-CSCF has stored, to be used by the IMS AGW on ground as the destination of media traffic towards the UE and to reserve an IP address in the IMS AGW on ground to be used by the UE as the destination of media traffic. In addition, the P-CSCF requests the IMS AGW on ground to configure context information other than IP addresses of the connection point towards the UE based on the corresponding context in the IMS AGW on satellite. This step 4 is performed according to clause 8.2 of TS 23.334 [74].

5.	P-CSCF requests IMS AGW on ground to reserve an IP address in the IMS AGW on ground to be used by the terminating network as the destination of media traffic towards the originating network. In addition, the P-CSCF requests the IMS AGW on ground to configure context information other than IP addresses of the connection point towards the terminating network based on the corresponding context in the IMS AGW on satellite. This step 5 is performed according to clause 8.2 of TS 23.334 [74].

6.	P-CSCF replies to PCF as defined in TS 23.502 [94] to the early notification received in step 2 by indicating that the change of the user plane paths for optimized media routing to the one through the target satellite should not be performed. P-CSCF also sends to PCF the IP address allocated in IMS AGW on ground to be used by UE as the destination of media traffic, so that 5GC updates the packet filter list of the QoS rule in UE for media traffic to additionally contain this IP address according to clause 4.3.3.2 of TS 23.502 [94].

7.	P-CSCF sends a SIP message to IMS AS to request it to send SIP re-INVITE to the terminating network. This SIP message contains the IP address allocated in IMS AGW on ground to be used by the terminating network as the destination of media traffic. This SIP message does not contain any satellite ID.

8.	IMS AS sends SIP re-INVITE to the terminating network. This SIP re-INVITE contains an SDP offer that has the IP address allocated in IMS AGW on ground to be used by the terminating network as the destination of media traffic. This SIP re-INVITE does not contain any satellite ID.

9.	P-CSCF receives the SIP re-INVITE.

10.	P-CSCF requests IMS AGW on ground to configure the IP address allocated in UE, which the P-CSCF has stored, to be used by the IMS AGW on ground as the destination of media traffic towards the UE and to reserve an IP address in the IMS AGW on ground to be used by the UE as the destination of media traffic. In addition, the P-CSCF requests the IMS AGW on ground to configure context information other than IP addresses of the connection point towards the UE based on the corresponding context in the IMS AGW on satellite. This step 10 is performed according to clause 8.2 of TS 23.334 [74].

11.	P-CSCF requests IMS AGW on ground to configure the IP address received in step 9 to be used by the IMS AGW on ground as the destination of media traffic towards the originating network and to reserve an IP address in the IMS AGW on ground to be used by the originating network as the destination of media traffic towards the terminating network. In addition, the P-CSCF requests the IMS AGW on ground to configure context information other than IP addresses of the connection point towards the originating network based on the corresponding context in the IMS AGW on satellite. This step 11 is performed according to clause 8.2 of TS 23.334 [74].

NOTE 4:	It is assumed in general that the newly established IMS AGW (e.g., IMS AGW on the target satellite or on ground) allows voice/video media to flow immediately after the reservation and configuration are completed (e.g., without waiting for the response from the remote end if the reservation and configuration are made triggered by SIP re-INVITE).

12.	P-CSCF sends to PCF the IP address allocated in IMS AGW on ground to be used by UE as the destination of media traffic, so that 5GC updates the packet filter list of the QoS rule in UE for media traffic to additionally contain this IP address according to clause 4.3.3.2 of TS 23.502 [94].

13.	P-CSCF sends SIP re-INVITE to UE. This SIP re-INVITE contains an SDP offer that has the IP address allocated in IMS AGW on ground to be used by UE as the destination of media traffic.

NOTE 5:	RTP/RTCP is not symmetric between step 13 and step 20.

14.	The media path from the originating network to the terminating network remains the same. The media path from the terminating network to the originating network is via ULCL on satellite, PSA on ground, IMS AGW on ground in the terminating network and further via IMS AGW on ground, PSA on ground, and ULCL on satellite in the originating network.

15. UE sends SIP 200 OK containing an SDP answer to P-CSCF. P-CSCF sets an implementation-specific timer for releasing IMS AGW on satellite that expires sometime after UE in the originating network receives SIP re-INVITE in step 20.

16.	P-CSCF sends SIP 200 OK to the originating network. This SIP 200 OK contains an SDP answer that has the IP address allocated in IMS AGW on ground to be used by the originating network as the destination of media traffic towards the terminating network. This SIP 200 OK does not contain any satellite ID.

17. IMS AS receives the SIP 200 OK.

18.	IMS AS sends SIP re-INVITE to P-CSCF. This SIP re-INVITE contains an SDP offer that has the IP address allocated in IMS AGW on ground in the terminating network to be used by the originating network as the destination of media traffic towards the terminating network. This SIP re-INVITE does not contain any satellite ID.

19. P-CSCF requests IMS AGW on ground to configure the IP address received in step 18 to be used by the IMS AGW on ground as the destination of media traffic towards the terminating network. This step 19 is performed according to clause 8.4 of TS 23.334 [74].

20.	P-CSCF sends SIP re-INVITE to UE. This SIP re-INVITE contains an SDP offer that has the IP address allocated in IMS AGW on ground in the originating network to be used by UE as the destination of media traffic.

21.	The media path in both directions is via ULCLs on satellite, PSAs on ground, and IMS AGWs on ground.

22.	UE sends SIP 200 OK containing an SDP answer to P-CSCF.

23.	P-CSCF sends SIP 200 OK containing an SDP answer to IMS AS. This SIP 200 OK does not contain any satellite ID.

24. Both in the originating network and in the terminating network, SMF in 5GC releases ULCL and L-PSA on the source satellite according to NOTEs 7 and 8 and steps 11 and 12 in clause 4.3.5.7 of TS 23.502 [94].

25. P-CSCF in the originating network releases IMS AGW on satellite in the originating network sometime after receiving SIP 200 OK in step 22. P-CSCF in the terminating network releases IMS AGW on satellite in the terminating network after the timer being set in step 15 expires. This step 25 is performed according to clause 8.5 of TS 23.334 [74].

26.	The media path in both directions is via PSAs on ground and IMS AGWs on ground.

## ---End of the Change---