3GPP TSG-WG2 Meeting #166	S2-2412756

# Orlando, USA, November 18-22, 2024										   revision of S2-2411660

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0150 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Instructing a UAV to perform its altitude reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order for 5G network to perform in-flight UAV monitoring (clause 5.12.3) and assistance with USS changeover (clause 5.13), NFs are required to know UAV’s altitude information (for instance, to detect and report UE’s deviation from the assigned trajectory, which includes TAs information and altitude values for each of the TAs). However, there is no mechanism that (1) configures aerial UEs (UAVs) for altitude reporting to NG-RAN nodes, (2) reports UAV’s altitude information to a core network NF (e.g. AMF); moreover, the CN needs to be informed when a UAV crosses the certain altitudes that are of relevance for a UAV’s flight path. This CR addresses these two aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #---r02--- -removing “NAS” alternative from Option B with node-level signalling -removing no-UE-associating signalling altervative from Option B. #---r00--- -This CR proposes two options on how the CN can configure aerial UEs for altitude reporting, considering a flight path information received from a USS/UTM. Option A: configuration (altitude threshold values, periodicity information) is sent to an aerial UE over the application layer from the USS/UTM after the UAS NF determines these parameters. Option B: node-level signalling using either UE-associtated signalling or non-UE-associated signalling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functionality required to support the in-flight UAV monitoring and USS changeover procedures is missing, which impedes the aforementioned functionalities of the 5G network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5141 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[16]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[17]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[18]	3GPP TS 26.502: "5G multicast-broadcast services; User Service architecture".

[19]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[20]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[XY1]	3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".

[XY2]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

>>>> Next Change <<<<

## 5.X	Altitude reporting for aerial UEs

## 5.x.1	General

In order to assist a USS/UTM with aerial UE’s flight planning and monitoring, the 5GS may enable configuration of aerial UEs (UAVs) to perform altitude reporting. This may include providing aerial UEs with threshold values for altitude reporting and/or request reporting periodicity (e.g., threshold-based, event-based, periodic). For that, the 5GS may determine altitude thresholds and periodicity with which an aerial UE should report its altitude. Altitude thresholds can be derived, for instance, based on the flight path information provided by the serving USS/UTM during a pre-flight planning request or a USS changeover request. Delivery of the altitude thresholds and reporting periodicity can be done via application layer or via node-level signalling. The 5GS should forward the altitude measurements results to the AMF and NEF/UAS NF.

### 5.X.2	Instructing aerial UEs to perform altitude reporting

Procedure shown in Figure 5.X.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV’s flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring, and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively.

UAS NF/ NEF

USS

NFs

AMF

NG-RAN

UAV

1. Obtaining the 5GC assistance for pre-flight planning or in-flight UAV monitoring as specified in clause 5.12.2 and 5.12.3, respectively.

2. Based on the candidate flight paths information and/or the provided flight path information, the UAS NF/NEF determines minimum and maximum values of altitudes (i.e., altitude thresholds) for UAV’s flight

3a-1. Nnef_RetrieveInfoUAVFlight_Get response

3a. Delivering the UAS NF/NEF derived altitude threshold values to the USS/UTM

Option A: Application layer

3a-2. Nnef_UAVFlightAssistance_Notify request

4a. Delivering the altitude thresholds for UAV’s altitude reporting over the application layer for the UAV’s consumption

3b. Namf_EventExposure_Subscribe (with altitude thresholds, periodicity indication)

Option B: Node-level signalling

4b. N2 message

5b. Instructing/configuring UEs for altitude reporting

Figure 5.X.2-1: Procedure for instructing aerial UEs for altitude reporting

1.	UAS NF/NEF assists the USS/UTM with pre-fligh planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13.

2.	Based on the flight path information included in Step 1 and other information obtained as a result of either of the procedures (i.e., NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e., altitude range) for UAV’s flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [XY1] can be used for altitude thresholds and event-based reporting.

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically (e.g. every 5s, 15s, 30s, 1min, 2min etc.), threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

3.	Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e., threshold-based, event-based, or periodic) for UE’s altitude reporting to the aerial UE(s).

Option A: Application layer.

3a.	The UAS NF/NEF sends the derived altitude threshold values and provides indication about the required periodicity of the reporting (on demand, event-triggered, periodic, or per change of the altitude, or change of a specific geographical area) to the serving USS/UTM either in a Nnef_RetrieveInfoUAVFlight_Get response message or in a Nnef_UAVFlightAssistance_Notify message depending on the procedure that has triggered the UAS NF/NEF to derive the altitude thresholds for the aerial UE.

4a.	The USS/UTM sends the altitude reporting thresholds and, optionally, indication about reporting periodicity (and any additional information required to perform the reporting, e.g., a list of TA(s)/NG-RAN node identifier(s) for event-based reporting, periodicity value (e.g. 5s, 15s, 30s, 1min, 2 min etc.) for periodic reporting) to the UAV over the application layer for the UAV’s consumption.

NOTE 2:	Application layer reporting itself is outside of 3GPP scope.

Option B: Node-level signalling.

Editor's Note: This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV’s deviation from the assigned trajectory (i.e., Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE’s altitude reporting.

Additionally, the UAS NF/NEF may also include an indication to perform UE’s altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s).

4b.	The AMF sends the corresponding N2 messages to all applicable NG-RAN node(s).

5b.	The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE 3:	Aspects related to NG-RAN instructing UEs to perform altitude reporting are specified by RAN in TS 38.413 [XY2].

>>>> Next Change <<<<

### 5.X.3	Reporting UE’s altitude information

A procedure for reporting aerial UE’s altitude information to the core network is shown in Figure 5.X.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.X.2.

UAS NF/ NEF

USS

NFs

AMF

NG-RAN

UAV

0. Instructing a UE to perform altitude reporting as specified in clause 5.X.2

1. UE’s altitude reporting conditions are met

Option A: Application layer

2a. Reporting UE’s altitude information over the application layer to the serving USS

3a. Nnef_UAVFlightAssistance_Update  request

Option B: Node-level signalling

1a. UE performs altitude information reporting

2b. N2 message

3b. Namf_EventExposure_Notify (with information about UE’s current altitude)

4. Determining the impact on the UAV’s flight path

Figure 5.X.3-1: Procedure for reporting of UE’s altitude information

0.	An aerial UE receives request to perform its altitude information reporting as specified in clause 5.X.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity.

1.	UE’s flight altitude meets the previously provided altitude reporting conditions (i.e., altitude thresholds and, optionally, reporting periodicity).

Option A: Application layer.

2a.	The UE may report the altitude information over the application layer to the serving USS/UTM in case the USS/UTM has provided altitude reporting threshold and, optionally, indication about reporting periodicity in Step 4a in clause 5.X.2.

NOTE 1:	Application layer reporting itself is outside of 3GPP scope.

3a.	The USS may invoke the Nnef_UAVFlightAssistance_Update to inform the UAS NF/NEF about UE’s altitude.

Option B: Node-level signalling.

1a - 2b.	The UE may provide a report with the altitude information to the NG-RAN. The report may contain the altitude information and, for instance, a time stamp or a report ID indicating when the UE has measured the altitude information that is being reported.

NOTE 2:	Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [XY1].

The NG-RAN node includes the received UE’s altitude information in an N2 message to the AMF.

3b.	The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request.

4.	The UAS NF/NEF together with the USS determines the impact on UAV’s flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

>>>> End of Changes <<<<