3GPP TSG-SA2 Meeting #166	S2-2412759

# Orlando, USA, November 18- 22, 2024          			                 		 (revision of S2-2412589)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0152 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Nnef_UAVFlightAssistance_Create service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes to update the Nnef_UAVFlightAssistance_Create service operation from the following two aspects: - Based on the approved CR (S2-2411160) in SA2#165 meeting, the USS/UTM can send a Nnef_UAVFlightAssistance_Create request to request in-flight monitoring of a target UAV. This function adds new input (optional) parameters to the service operation. - Based on the approved CR (S2-2411159) in SA2#165 meeting, the flight path information is clarified which includes a set of 4-point polygons, height information, and corresponding timestamps/time schedule for the flight, it is proposed to update the fligh path parameter in the Nnef_UAVFlightAssistance_Create service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the input: optional part of Nnef_UAVFlightAssistance_Create service operation for USS changeover, pre-flight planning, and for in-flight monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a destination point (even when the destination point is located in geographical area served by different USS).

Editor's note:	Representation of the destination point and border crossing point format is FFS.

Input, Required: UAV's identifier (e.g. GPSI).

Input, Optional: For USS changeover: flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells; For pre-flight planning: flight path information using a set of 4-point polygons, height information (optional), and (if available) corresponding timestamps/time schedule for the flight, time schedule for crossing/spending at each of the TAs/cells; For in-flight monitoring: information about the flight path(s), monitoring mode, reporting requirements, assistance information, the planned flight path(s), a list of UAVs in the area of interest if the flight environment is set for the monitoring mode, present value for distance if the monitoring mode is set to Ranging/Sidelink positioning; Notification Target Address.

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

* * * End of Change * * *