3GPP TSG-SA2 Meeting #166	S2-2412947

# Orlando, United States, 18th-22nd Nov 2024	 (revision of 2411187, 2328, 2380, 2656, 2722, 2900)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5746 | rev | 10 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of UDP-Option carrying XRM metadata over UDP tunnel for end-to-end encrypted XRM flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Futurewei, InterDigital Inc., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA plenary approved conclusions in 23.700-70 clause 8.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new feature that supports UDP-Option carrying media related information over N6 which is established using connect-udp as On-path N6 signaling between the AS proxy and the PSA UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conclusions in 23.700-70 clause 8.2 for KI#2 are not supported by normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X.4 (new), 5.8.5.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR should be implemented after CR 5745 |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL New Text)

##### 5.37.X.4 Usage of UDP Option transferring media related information over N6

The mechanism described in this clause uses connect-udp in tunnell mode for On-path N6 signaling to establish a UDP tunnel between the UPF as HTTP/3 client and the AS as HTTP/3 proxy according to clause 5.37.X.2 to securely transfer media related information. This clause defines the differences with the mechanisms defined in clause 5.37.X.2 when media related information is carried over UDP Option.

The media related information is encoded into UDP-Option as defined in [xx3] and transferred through the UDP tunnel over N6, in which the inner UDP datagram contains end-to-end encrypted XRM media packet between AS proxy and UE, and the outer UDP datagram between AS proxy and UPF contains UDP-Option carrying media related information (see Annex X).

Editor’s Note:  The format of the media related information carried by the specific UDP-Option and exchanged between the UPF and the AS proxy will be further defined by 3GPP CT working groups.

Editor’s Note: The details for documenting and assigning a UDP option kind value for carrying media related information in IETF are pending collaboration with IETF.

The usage of UDP-Option transferring media related information over N6 is supported as defined in clause 5.37.X.2 with following differencess:

-	When the AF provides On-path N6 signaling information for connect-udp, it may also provide an indication of UDP-Option in Protocol Description for transfering media related information. This is to instruct the PSA UPF that the media related information is carried in UDP-Option in UDP datagram over N6.

-	The inner UDP datagrams contain the unmodified payload for both UL and DL traffic and the media related information is included in the UDP option of the DL outer UDP datagram.

-	The UPF supports the HTTP/3 Client functionality for negotiation of security keys used for encrypting media related information carried in the UDP-Option. The UPF shall remove the UDP-Option carrying media related information before forwarding the XRM media packets over right PDU Session towards the (R)AN and the UE.

Editor’s Note: Security keys for UDP-Option can be negotiated using connect-udp upgrade token in tunnelled mode between the UPF and AS proxy. The details are up to SA3.

NEXT CHANGE (2)

#### 5.8.5.3          Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.5.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application identifier, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application identifier |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
|  | FQDN Filter for DNS Query | Contains one or more FQDN, FQDN range, and/or any FQDN. | The FQDN or FQDN range only used for detection of plain DNS Query message (i.e. not subject to ciphering). The usage is described in TS 23.548 [130]. |
|  | Protocol Description | Indicates service protocol used by the flow (NOTE 8) (NOTE 9) |  |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e. source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e. FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e. FAR, QER, URR). |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e. lookup of the other PDRs. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1:Needed e.g. if: -UPF supports multiple DNN with overlapping IP addresses; -UPF is connected to other UPF or AN node in different IP domains. -UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. -UPF "local switch" may be used for DNN/S-NSSAI dedicated for PIN. NOTE 2:Either a FAR ID or a MAR ID is included, not both. NOTE 3:The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4:When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that CN tunnel info is to be locally determined. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in the case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. NOTE 8: Not for PDR matching. In the case of unencrypted traffic, it may be provided to assist PDU Set identification when PDU Set Identification and marking applies to the PDR and/or to assist identification of the last packet of the Data burst in downlink when End of Data Burst identification and marking in downlink applies to the PDR. See clause 5.8.2.4.2 and TS 26.522 [179]. NOTE 9: Not for PDR matching. In the case of end-to-end encrypted traffic, it may be provided to indicate how to retrieve media related information as specified in clauses in 5.37.X. |  |  |  |

NEXT CHANGE (all new text) (3)

# Annex X (normative):

# UDP-Option carrying media related information over UDP tunnel

When the DL traffic is end-to end encrypted and transported from the AS proxy through a UDP tunnel established using connect-udp between the AS proxy and the PSA UPF as described in 5.37.X.2, the AS proxy generates a XRM packet which inner UDP datagram contains the DL UDP media packets, i.e. QUIC packet between AS proxy and UE, and the outer UDP datagram between AS proxy and UPF contains UDP-Option carrying encrypted media related information. Figure X-1 shows XRM packet structure.

Figure X-1: XRM packet structure

After the UPF establishes a QUIC connection to the AS proxy using connect-udp according to clause 5.37.X.2, the AS proxy and the PSA UPF negotiates security keys using connect-udp upgrate token in tunneled mode. Then the AS proxy encodes media related information into the UDP-Option in outer UDP datagram and uses security keys for encrypting media related information carried in the UDP-Option.

END OF CHANGES