3GPP TSG-WG SA2 Meeting #166	S2-2413022

# 18 - 22 November, 2024, Orlando, USA			                                 (revision of S2-2411404)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5628 | rev | 15 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Energy Efficiency and Energy Saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Rakuten Mobile, SK Telecom, China Unicom, NEC, Tencent, TOYOTA MOTOR CORPORATION, Samsung, ZTE, LG Electronics, ETRI, KDDI, Orange, Interdigital, NTT DOCOMO, KPN, Deutsche Telekom, vivo, Huawei, HiSilicon, Telefonica, Nokia, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This paper provides the basic framework and function design to support energy related information collection, energy consumption information calculation, and exposure. New NF EIF is introduced to support the above functionality. The methods of calculation for the Energy consumption information of different granularities are provided. The Energy Consumption information of the different granularities are derived from user plane traffic transmission, i.e. the combination of Energy Consumption information in NG-RAN, I-UPF(s) and PSA-UPF(s), and the data volume is the main contributor. The input data for Energy Consumption information calculation include: Node-level energy consumption information, Node-level data volume, and data volume of the required granularities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NF EIF for architecture and function design to support energy consumption information collection and calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy saving features cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 3.2; 5.X;6.2.5; Annex XX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

skip

[195]	3GPP TS 38.414: "NG-RAN; NG data transport".

[196]	3GPP TS 38.799: "Study on Additional Topological Enhancements for NR".

[Y]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

# * * * * Second change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AI/ML	Artificial Intelligence/Machine Learning

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AoI	Area of Interest

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

CQRCI	Clock Quality Reporting Control Information

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DetNet	Deterministic Networking

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

EIF	Energy Information Function

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FL	Federated Learning

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

L4S	Low Latency, Low Loss and Scalable Throughput

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPQUIC	Multi-Path QUIC

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

MWAB	Mobile gNB with wireless access backhauling

N3IWF	Non-3GPP InterWorking Function

N3QAI		Non-3GPP QoS Assistance Information

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NAT	Network Address Translation

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NS-AoS	Network Slice Area of Service

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PDV	Packet Delay Variation

PEGC	PIN Element with Gateway Capability

PEI	Permanent Equipment Identifier

PEMC	PIN Element with Management Capability

PER	Packet Error Rate

PFD	Packet Flow Description

PIN	Personal IoT Network

PINE	PIN Element

PLR	Packet Loss Rate

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSIHI	PDU Set Integrated Handling Information

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QMC	QoE Measurement Collection

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

RTT	Round Trip Time

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SF	Service Function

SFC	Service Function Chain

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAC	TSC Assistance Container

TSCAI	Traffic Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TSS	Timing Synchronization Status

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * *Third change (all new text) * * * *

5.X	Support of Energy Efficiency and Energy Saving

5.X.1	General

The 5GS supports some features aimed at Energy saving described in the following clauses.

5.X.2	Energy Consumption Information collection, calculation and exposure

5.X.2.1	General

The Energy Information Function (EIF) is defined to collect the UE related Energy Consumption information, calculate the Energy Consumption information at UE, PDU Session and/or QoS flow granularity, and expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF) subject to operator’s policy.

Node level energy consumption is reported PLMN-wide at a configurable period T.

5.X.2.2	Energy Consumption information collection

The Energy Information Function (EIF) collects the UE related Energy Consumption information include Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e., PDU Session and/or QoS flow) from UPF (via SMF).

The serving SMFs are retrieved from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

EIF provides UE ID, DNN/S-NSSAI, IP 5-tuple to retrieve the information from SMF. And the information collected from SMF by EIF, is shown in Table 5.X.2.2-1.

The data volume reporting period from the SMFs is the period T with PLMN-wide configuration.

Table 5.X.2.2-1: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session |
| IP 5-Tuple | IP-5-tuple |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time period. |
| > UL/DL Data Volume of UPF | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > gNB serving the UE | The gNB which serving the UE. |
| Time period | Time period of the collected information. |

Table 5.X.2.2-2 provides the list of information received from OAM. EIF requests the OAM information with providing the serving gNB ID(s) and (I-)UPF ID(s), which provided by SMFs. The reporting period from the OAM is the period T which is the same as data volume reporting period from SMFs.

Table 5.X.2.2-2: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time period T based on clause 6.7.3.4.2 of TS 28.554[10]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time period T based on clause 6.7.1.1 of TS 28.554[10]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time period T based on clause 6.7.3.1 of TS 28.554[10]. |
| UPF data volume | Data volume consumed at a UPF. |

When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1: The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 2: In this release, only the energy related information of user plane communication is supported, and that of control plane signalling is not supported.

5.X.2.3	 Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularies (UE, PDU Session and/or QoS flow), based on input parameters from table 5.X.2.2-1 and 5.X.2.2-2 and gets the results. Some example formulas to support the above calculation are described in Annex XX.

5.x.2.4	Energy Consumption information exposure

AF/NEF or 5GC NF subscribes the EIF for Energy consumption information of required granularities (UE, PDU session and/or QoS flow).

- For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

- For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

- For QoS flow level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and Flow description(s).

- The consumer NF may also subscribe the above information exposure with providing reporting period, reporting frequency.

Editor’s NOTE: Whether there are other required parameters for energy consumption information exposure are FFS.

Editor’s NOTE:	 Whether and how the renewable energy is introduced is FFS and depending on SA5.

Editor’s NOTE: whether and how the per application energy consumption information can be exposed are FFS.

# * * * * Forth change  * * * *

6.2.5	NEF

6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, time synchronization service information and PDU Set handling service specific information. In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	Redirecting the AF to a more suitable NEF/L-NEF e.g. when serving an AF request for local information exposure and detecting there is a more appropriate NEF instance to serve the AF's request.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Support management of ECS Address Information.

-	Support management of relationship between DNAI and EAS Address Information.

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

-	Charging data collection and support of charging interfaces.

-	Support of Member UE selection assistance functionality:

-	NEF may provide one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information to the AF based on the parameters contained in the request from the AF as described in clause 5.46.2. NEF supports the translation of the member UE selection filtering criteria parameters received from the AF to the corresponding event or analytics filters that can be understood by the 5GC NFs for events or analytics related data collection. NEF interacts with 5GC NFs using existing services in order to collect the corresponding data and then derive the list(s) of candidate UE(s) and other assistance information as described in clause 4.15.13 of TS 23.502 [3].

-	Support of Multi-member AF session with required QoS for a set of UEs identified by a list of UE addresses:

-	Details are specified in clause 4.15.6.13 of TS 23.502 [3].

-	Support of UAS NF functionality:

Details are defined in TS 23.256 [136].

-	Support of EAS deployment functionality:

Details are defined in TS 23.548 [130].

-	Support of SBI-based MO SM transmit for MSISDN-less MO SMS:

Details are defined in TS 23.540 [142].

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Support management of common EAS and common DNAI:

Details are defined in TS 23.548 [130].

-	Support request and exposure of energy related information as defined in clause 5.x.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

6.2.5.1	Support for CAPIF

When an NEF is used for external exposure, the CAPIF may be supported. When CAPIF is supported, an NEF that is used for external exposure supports the CAPIF API provider domain functions. The CAPIF and associated API provider domain functions are specified in TS 23.222 [64].

# * * * * Eighth change * * * *

Annex XX (informative):
Network Energy consumption information calculation

# XX.1	General

This Annex provides examples on how EIF calculate the energy consumption for required granularities.

XX.2	Calculation of Energy Consumption parameters

The energy consumed at a UPF (EUPF)or a gNB (EgNB) is known over a time window T. And the EIF is aware of the data volume (DV) consumed at a UPF by a UE (DVUE,UPF), UE PDU session (DVSession,UPF) and UE QoS flow (DVFlow,UPF) over the time interval T. The EIF is aware of the overall data volume at a gNB (DVgNB), as well as at a UPF (DVUPF) which serving the UE. Then based on the below example fomular, the required granularities Energy consumption information can be obtained.

To derive the of energy consumed at a gNB over a period of time T by a UE, PDU session, QoS flow, the formulas are:

EUE,gNB=EgNBDVUE,gNBDVgNB

ESession,gNB=EgNBDVSession,gNBDVgNB

EFlow,gNB=EgNBDVFlow,gNBDVgNB

To derive the energy consumed at a UPF over a time period T by a UE, session, QoS flow the formulas are:

EUE,UPF=EUPFDVUE,UPFDVUPF

ESession, UPF=EUPFDVUE,UPFDVUPF

EFlow,UPF=EUPFDVFlow,UPFDVUPF

To derive the energy consumed in the network in the time interval T by a UE (EUE), Session (ESession), QoS flow (EFlow) the formula is:

EUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi=Esession

where gNBUET are all gNBs used by the UE in an interval T, UPFUET are all UPFs used by the UE in an interval T.

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi

where gNBSessionT are all gNBs used by the PDU Session in an interval T, UPFSessionT are all UPFs used by the PDU Session in an interval T.

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi

where gNBFlowT are all gNBs used by the QoS Flow in an interval T, UPFFlowT are all UPFs used by the QoS Flow in an interval T.