3GPP TSG-SA3 Meeting #119	     S3-244658

Orlando, Florida, 11 – 15 Nov 2024

Source:	Google

Title:	Conclusion for KI#5 (Certificate Renewal)

Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve the pCR to TR 33.776

# 2	References

[1]	3GPP TR 33.776: “Study of ACME for Automated Certificate Management in SBA”

[2]	IETF RFC 8555: “Automatic Certificate Management Environment (ACME)”, 2019

[3]	3GPP TS 33.310: “Network Domain Security (NDS); Authentication Framework (AF)”

# 3	Rationale

This contribution proposes a conclusion to KI#5 in TR 33.776.

# ******************   START OF CHANGES   ************************

# 7 Conclusion

## 7.5. 	KI#5: Certificate renewal

### 7.5.1	Analysis

This key issue is addressed by two solutions, namely Solution #4 (Reuse solution about policy-based certificate renewal) and Solution #9 (Using ACME protocol for certificate renewal), which proposes to reuse the solution adopted in the study of FS_ACM during Release 18. Solution #4 depends on the preconfigured policy. Similarly, in solution #9 the NF initiates certificate renewal procedures based on pre-configured certificate renewal policies. In other words, the NF receives a trigger for certificate renewal from the pre-configured policies. As the CA/RA receives the request, it checks if an NF is pre-authorized for certificate renewal. If an NF/ACME client is pre-authorized for certificate renewal, the NF need not go through the challenge-response mechanism.

### 7.5.1 Conclusion

It is proposed to adopt the pre-configured policies from Solution #4 and Solution #9 as the basis for certificate renewal. Additionally, it is proposed that Solution #9 be used for detailed steps including pre-authorization aspects of certificate renewal for the normative work.

******************    END OF CHANGES   ************************