3GPP TSG-SA3 Meeting #119	S3-244802

Orlando, US  11 - 15 November 2024

# Source:	Nokia

# Title:	Updates and EN Removal for solution #7

# Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

It is required to approve this contribution.

# 2	References

[1]	3GPP TR 33.745 “Study on security aspects of 5G NR Femto”

# 3	Rationale

The contribution proposes updates and EN removal for solution#7.

# 4	Detailed proposal

# * * * * First change* * * *

### 6.7.1		Solution details

Figure 1 shows the message sequence where Core network (AMF) initiated location verification.

Figure 1: Core network (AMF) initiated location verification.

Note: For the figure 1, 3GPP TS 23.273 Figure 6.11.1-1 is used as a base.

- 5G NR Femto sends NG Setup Request by including the 5G NR Femto ID as per TS 38.413.

- AMF responds with NG Setup Response indicating the NG connection is successful.

- UE registers with network and is in the RRC connected state.

- If 5G NR Femto ID is included in the NG Setup request, AMF decides to initiates  location verification of 5G NR Femto.

- AMF obtains UE location using NAS TRANSPORT messages for requesting the UE position, or, using MDT reports provided by the UE. For MDT reports, prior user-consent is verified before seeking the report from UEs connected with the Femto.

- NOTE: If the payload container type is set to "Location services message container" and is included in the UL NAS TRANSPORT, DL NAS TRANSPORT or CONTROL PLANE SERVICE REQUEST message, the payload container contents include location services message payload. This is as per existing 3GPP specification TS 24.501.

- After obtaining UE location, AMF requests LMF to verify the location information by sending new message Nlmf_DetermineFemtoLocation Request by including 5G NR Femto ID, Location information, optionally 5G NR Femto serial number may be sent to AMF (during Stage 3 standardization API name may be decided). Any of the existing equivalent message may also be used.

  - Considering that Femto device is not expected to move beyond a certain range, AMF sends this request only to the LMF of the same serving network. If LMF fails to find the Femto ID and/or its location, failure case is handled as explained below.

- LMF verifies Location information received from AMF with operator configured location (optionally 5G NR Femto serial number may be considered). Here, LMF uses the 5G NR Femto location information and Femto range to verify whether the UE is within the configured range (coverage area) and checks the relative location to confirm that the 5G NR Femto is at the location where it is supposed to be.

- LMF responds to AMF with location verification of 5G NR Femto is success.

  - If the verification is successful Femto services can be continued as normal.

- If the received location information from AMF (in step 6) is not within the range (coverage area) of the  configured/stored Femto location in the LMF then LMF sends Nlmf_DetermineFemtoLocation Response message to AMF with failure cause = “Femto/5G NR Femto device location has been changed as compared to assigned location”.

  - This failure message is also sent if the femto device location is not found in that LMF. In such a scenario, a different failure cause indicating “Femto/5G NR Femto device location not found in LMF” can be sent to AMF.

- AMF notifies operator to take corrective measures because the 5G NR Femto has moved from its location. Also, this can be an indication of a false/compromised 5G NR Femto. Operator can use this information to further apply analytics and derive patterns if any. In some implementations, this information can be sent to an automation function (like NWDAF/MDAF) which can take automated decisions based on analytics.

- Operator/automation function disables the services to 5G NR Femto due to the change of 5G NR Femto location. In some implementations, the operator/automation function can download factory software with a range of 0 meters, or, disable the broadcast/pilot channel, or any other such methods to ensure that the compromised 5G NR Femto cannot be used.

- Operator/automation function informs the Femto owner about the disabled services via SMS or other means. This can enable any genuine Femto owners to approach the operator and resolve the problem. Also, Operator/automation function may inform the LEA about this finding of false/compromised 5G NR Femto, if needed based on local regulations.

### 6.7.2		Solution Evaluation

In this solution, AMF contacts only the LMF which belongs to the serving network for verifying the NR femto’s location.

This solution provides an additional location security mechanism for NR Femto. This solution proposes to use UE location, which can be obtained transparently and securely by AMF, to verify the Femto’s actual location and ensure that it has not moved beyond a certain range (e.g., outside the home for residential femtos).

This solution addresses key issue #3.

# * * * * End of changes * * * *