3GPP TSG-SA3 Meeting #119	S3-244842

Orlando, US, 11 -15 November 2024

# Source:	Huawei, HiSilicon

# Title:	Updating solusion #7

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR proposal for TR 33.759

# 2	References

[1]	3GPP TS 33.759

# 3	Rationale

This contribution updates the solution #7 to address ENs in TR 33.759 [1]

# 4	Detailed proposal

pCR

***  START OF CHANGES  ***

## 6.7	Solution #7: UUAA for USS changeover

### 6.7.1	Introduction

This solution addresses the "Key Issue #1: security enhancements to NEF services in support of multiple USSs".

In order to accomplish the USS changeover procedure from the serving USS to the target USS specified in the clause 5.13 of 3GPP TS 23.256 [3]. The enhancements to UUAA is required, e.g., to trigger the UUAA procedure from the serving USS and determine the UUAA procedure for the candidate USS. The details are described in the following clause.

### 6.7.2	Solution details

The steps 1-21 of the procedure of USS changeover as specified in the clause 5.13.2 of the TS 23.256 [3] are amended as follows (only affected steps are described):

Step 4.	The request sent from the USS to the NEF includes an indicator. This indicator indicates to the NEF to exclude some USSes for flight assistance. These USSes are not listed in the UUAA context.

Step 5 – the NEF checks whether its UUAA context contains the UUAA results for candidate USSs.

Step 6 – the NEF performs the flight assistance procedure as in the clause 5.13.2 of the TS 23.256 [3]. The NEF may skip those USSs without UUAA results in the UUAA context (step 5)

NOTE: flight assistance is in SA2 scope.

Step 8 – In the response message, the NEF includes info about whether the candidate USSs require UUAA.

Step 9 – the serving USS (e.g., USS1) may choose the target USS not requiring UUAA.

Step 20 - If the serving USS selectes a target USS (e.g., USS2) requires UUAA, the serving USS1 informs the UAV or the target USS to perform UUAA as in the clause 5.13.2 of the TS 23.256 [3].

The UAV triggers the UUAA as in TS 33.256 [4] with the target USS address received from the serving USS.

Alternatively, the selected target USS2 sends an authentication request to the NEF and the NEF sends a UUAA request to the UAV through the SMF.

NOTE: the USS2 needs to establish a security link between the USS2 and the NEF before performing the UUAA procedure.

The NEF stores the USS2’s UUAA results in the UUAA context as in the TS 33.256 [4] if successful.

Editor’s Note: target USS determination is to be aligned with SA2.

### 6.7.3	Evaluation

The solution adresses the requirement of Key Issue #1.

The USS changeover procedure defined in the TS 23.256 [3] is reused, with additional details on the amended UUAA procedures if required.

***  END OF CHANGES  ***