3GPP TSG-SA3 Meeting #119	S3-244846

Orlando, US, 11 -15 November 2024

# Source:	Huawei, HiSilicon

# Title:	Inventory of protocols and algorithms for PQC migration

# Document for:	Discussion

Agenda Item:	3.2

# 1	Decision/action requested

For discussion and endorsement if agreed

# 2	References

[1]	3GPP S3-242377

[2]	3GPP S3-242378

[3] 	3GPP TS 33.501

# 3	Rationale

In responses to LSs received from ETSI and GSMA, SA3 has answered their questions based on the current status in SA3 in the LS replies [1] and [2] respectively. Given the activities and progress related to Post-Quantum Cryptography (PQC) in other standards organizations, e.g., NIST and IETF, it is desirable for SA3 to have a clear stand on issues related to PQC and a concrete plan for PQC migration.

As in the SA3’s LS replies [1][2], the current agreement on PQC in SA3 can be summarized briefly as follows:

- All the 128-bit symmetric algorithms used in 5G systems are secure against quantum computing in years ahead. There is no urgency to consider PQC migration as of now.

- Quantum computing will weaken the security of all asymmetric algorithms used in the 3GPP profiles and protocols in the 5G systems. SA3 will update the use of these algorithms, protocols, or profiles with quantum-safe versions in due course after they are made available by the respective standards bodies. As profiles of these protocols apply to 4G and 3G too, SA3 will update the 3GPP profiles of these protocols as well.

Based on the above agreement, further analysis and observations can be made as follows, focusing on asymmetric algorithm related procedures and profiles:

## 3.1	Primary Authentication

The two primary authentication methods, i.e., 5G AKA or EAP-AKA', are based on asymmetric algorithms (shared secret keys). Whereas the additional primary authentication methods specified in the Annex B of TS 33.501 [3] in private networks using the 5G system are based on the EAP framework, which allows authentication methods based on asymmetric algorithm to be used. For example, the procedures based on the EAP-TLS is illustrated in the Annex B of TS 33.501. Although the Annex B is informative, the EAP-TLS procedure should be migrated to a quantum-safe version in the future.

## 3.2	NAS Security, RRC security, UP Security

The NAS/RRC/UP security is based on symmetric algorithms.

## 3.3	SUPI Privacy

The Elliptic Curve Integrated Encryption Scheme (ECIES) specified in SECG is used to encrypt a SUPI. A quantum-safe version needs to be used in the future.

## 3.4	Service Based Interfaces (SBI) Security including SEPPs Security

All NFs based on SBI support mutually authenticated TLS and HTTPS as specified in RFC 9113 and RFC 2818. The SBI authorization is based on the OAuth 2.0 framework as specified in RFC 6749. These protocols need to be migrated to the quantum-safe versions in the future.

## 3.5	Security procedures for non-SBI

There are various interfaces in 5G systems are non-SBI, e.g., N2, N3, Xn, N6, and other interfaces, e.g., Rx based on DIAMETER or GTP. In general, the protection of these interfaces is based on IPsec ESP and IKEv2 as defined in TS 33.210. In addition to IPsec, DTLS as specified in RFC 6083 shall also be supported. These protocols need to be migrated to the quantum-safe versions in the future.

## 3.6	Secondary Authentication, Network slice NSSAA procedures

These procedures are essentially based on the EAP framework. Since the supported EAP methods, which may include asymmetric algorithms though, are not specified in 3GPP, there is no need update the procedure in 3GPP, with respect to PQC.

## 3.7	NEF security, Network Slice Management security

The mutual authentication and security protection between the NEF and AF interface and the management interface between the slice management service consumer and producer are based on TLS. Authorization of requests from AF or slice management service consumers is based on OAuth 2.0. Both protocols need to be migrated to the quantum-safe versions in the future.

## 3.8	Non-3GPP access

## The IKEv2 defined in RFC 7296 to set up one or more IPsec ESP security associations. In addition, DTLS may also be involved. These protocols should be miraged to a quantum safe version in the future.

## 3.9	NPN

The two primary authentication methods, i.e., 5G AKA or EAP-AKA', can be reused for NPN. In addition, EAP framework is also supported. Like in the NSSAA procedure, EAP method details are not specified in 3GPP and there is no impact to the NPN specifications.

## 3.10	Edge computing

The DNS over TLS specified in IETF RFC 7858 and RFC 8310 are used by the UE and the EASDF for the authentication and encryption of the DNS connection. The protocols cited should be miraged to a quantum safe version in the future.

## 3.11	Security profiles and implementations

In addition to the procedures specified, there are also security profiles that involve asymmetric algorithms affected by quantum computing. As specified in TS 33.210 and TS 33.310, there are additional specifications required to consider for PQC migration:

Certificate related protocols/profiles: X.509v3, CRL (RFC 5280), OCSP (RFC 6960), PKCS #10 (RFC 2986), CMPv2 (RFC 4210/6712/9480/9481), JWE and JWS (RFC 7515, 7516, 7518)

All these protocols should be miraged to quantum safe versions in the future.

## 3.12	Observations

The following observations can be made based on above analysis:

Observation 1: Many procedures/protocols/profiles in 5G systems are related to asymmetric algorithms. Their security will be weakened by Quantum computing. They will be subject to migration to quantum safe versions.

Observation 2: The migration works in some SDO, e.g., IETF, have started based on available PQC algorithms/protocols/certificates, but not expected to be completed soon. PQC migration work in other SDO, e.g., SECG, has yet to start.

Observation 3: It is possible that the quantum-resistant protocols/specifications to be adopted in 3GPP may not be available in time. For example, it is not clear whether there will be a quantum resistance version of ECIES to be developed in SECG.  Another example, there are sheer number of protocols in IETF subject to migration, it will not be surprised to see completing migration work may take years or decades.

# 4	Detailed proposal

Based on the analysis and observations above, the following proposal are presented for discussions and agreement/endorsement:

Proposal 1: An inventory of 3GPP protocols and algorithms should be made available within 3GPP (e.g., in Rel-20) and can be updated from time to time if needed.

Proposal 2: SA3 liaisons with other SDOs, i.e., the IETF or ETSI etc., to make sure quantum-resistant protocols/specifications adopted in 3GPP are made available before 3GPP specifications. For example, requesting SECG/ETSI to generate quantum-resistance version of ECIES if needed or inform the IETF the 3GPP protocols subject to PQC migration in the IETF.