3GPP TSG-SA3 Meeting #119	S3-234964

# Orlando, US, 11th - 15th November 2024

# Source:	Qualcomm Incorporated

# Title:	Conclusion of multi-hop U2N relay communication security in KI#1

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a conclusion of multi-hop U2N relay communication security in KI #1.

# 2	References

[1]	 TR 33.743

# 3	Rationale

This contribution proposes to add a conclusion of Key Issue #1, particularly for multi-hop UE-to-Network (U2N) relay communication security aspects.

Particularly, it is proposed to reuse the existing security procedure for PC5 security establishment for each hop as described in solution #5,#10, and #16.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 7.x	Key issue #1: Security for multi-hop UE-to-Network Relay

The following statements are agreed as a basis for normative work:

For multi-hop UE-to-Network Relay communication:

- The security procedure over User Plane for PC5 security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay is based on solution #10 and solution #16.

- The security procedure over Control Plane for PC5 security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay is based on solution #5.

NOTE 1: For the security procedure over User Plane, it is assumed that an Intermediate UE-to-Network Relay is able to access to the 5G PKMF in its HPLMN (i.e., PC8 establishment via Intermediate UE-to-Network Relay(s) or UE-to-Network Relay).

NOTE 2: For PC5 security establishment using the security procedure over Control Plane following discovery with Model A, PC5 security establishment for each hop is performed during discovery as specified in clause 6.4.3.8.1 of TS 23.304 [4].

***** END OF CHANGES *****