# 3GPP TSG-SA3 Meeting #119	S3-244990

# Orlando, US, 11 -15 November 2024

Title:	[draft] Reply LS on Issues related to Analytics context transfer between AnLF(s)

Response to:	S3-244638/S2-2409441 LS Reply to Issues related to Analytics context transfer between AnLF(s)

Release:	Rel-18

Work Item:	eNA_Ph3_SEC

Source:	SA3

To:	SA2

Cc:	-

Contact Person:	Wei Zhou

E-mail Address:	zhouwei@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 thanks SA2 for the LS reply to Issues related to Analytics context transfer between AnLF(s).

SA3 would like to provide the following answers to the questions and request from SA2:

“The source NWDAF can provide the file address(es) of the trained ML Model(s) only if the ML Model(s) is produced by this source NWDAF (i.e. the MTLF if contained in the source NWDAF) and the target NWDAF is allowed to get the ML model file address(es) based on local information (e.g., the ID of the target NWDAF is locally configured in the source NWDAF or stored as part of allowed NF instance list for the ML model(s) in the source NWDAF). Question 1: Can SA3 indicate if there are security issues in the scenario above?”

Answer to Question 1: In the above scenario, i.e. the ML Model(s) is produced by the source NWDAF containing MTLF and the target NWDAF is allowed to get the ML model file address(es) (e.g., the ID of the target NWDAF is locally configured or stored as part of allowed NF instance list for the ML model(s) (as specified in Annex X.10 of TS 33.501) in the source NWDAF), SA3 see no security issue for the source NWDAF to provide ML model file address to the target NWDAF.

“Question 2: For scenarios other than the above, when the source NWDAF contains both AnLF and MTLF, is there any security issue for the source NWDAF to provide the file address(es) of the trained ML Model(s) to the target NWDAF?”

Answer to Question 2: In scenarios other than the above (i.e. the ML model is not produced by the source NWDAF), the source NWDAF should not provide ML model file address to the target NWDAF if the target NWDAF is not authorized as allowed NF instance for the ML model.

“SA2 kindly asks SA3 to address security issues, if any, that may be raised by the transfer of the ML model file address from the source to the target NWDAF in future release.”

Answer: In scenarios that the ML model is not produced by the source NWDAF, SA3 considers that the existing procedure as defined in Annex X.10 of TS 33.501 can be adopted to address the security issue, as follows:

- The source NWDAF (as NF Service Consumer) may request ML model on behalf of the target NWDAF (i.e. ML model Consumer) to the NF Service Producer as specified in Annex X.10 of TS 33.501. If the target NWDAF is authorized for the ML model by the NF Service Producer, the source NWDAF can provide the ML model file address to the target NWDAF; otherwise, the source NWDAF should not provide the ML model file address to the target NWDAF.

Then there is no future work required for SA3 to address the security issue.

2. Actions:

To SA2

ACTION:  SA3 kindly asks SA2 to take the above answers into account.

3. Date of Next SA3 Meeting:

SA3#120			February 17th – 21st, 2025			Athens, GR

SA3#121			April 07st – 11th, 2025				Goteborg, SE