3GPP TSG-SA3 Meeting #119	S3-245062

Orlando, US, 11 -15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove normative language from test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Normative language shall not be used in test redaction |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the normative language in 2 test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.2, 4.2.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | draftCR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### ********** 1st Change *************

##### 4.2.2.2.2	Protection at the transport layer

Requirement Name: Protection at the transport layer

Requirement Reference: TS 33.501 [10], clause 5.9.2.1, clause 13.1, clause 13.3.2

Requirement Description:

NF Service Request and Response procedure supports mutual authentication between NF consumer and NF producer as specified in TS 33.501 [10], clause 5.9.2.1 .

All network functions support TLS. Network functions support both server-side and client-side certificates. The TLS profile follows the profile given in Annex E of TS 33.310 [9] with the restriction to be compliant with the profile given by HTTP/2 as defined in RFC 7540 [11] as specified in TS 33.501 [10], clause 13.1.

Authentication between network functions within one PLMN uses one of the following methods:

-	If the PLMN uses protection at the transport layer as described in clause 13.1, authentication provided by the transport layer protection solution is used for authentication between NFs as specified in TS 33.501 [10], clause 13.3.2.

NOTE 1:	This test case only applies to service-based interfaces.

Threat References: TR 33.926 [4], clause 5.3.6.3, Weak cryptographic algorithms

Test case:

Test Name: TC_PROTECT_TRANSPORT_LAYER

Purpose:

Verify that TLS protocol for NF mutual authentication and NF transport layer protection is implemented in the network products based on the profile required.

Procedure and execution steps:

Pre-Conditions:

Network product documentation containing information about supported TLS protocol and certificates is provided by the vendor.

A peer implementing the TLS protocol configured by the vendor is available.

The tester bases the tests on the profile defined by 3GPP in Annex E of TS 33.310 [9] with the restriction that it is compliant with the profile given by HTTP/2 as defined in RFC 7540 [11].

Execution Steps

1.	The tester checks that compliance with the TLS profile can be inferred from detailed provisions in the network product documentation.

2.	The tester establishes a secure connection between the network product under test and the peer and verify that all TLS protocol versions and combinations of cryptographic algorithms that are mandated by the TLS profile are supported by the network product under test.

3.	The tester tries to establish a secure connection between the network product under test and the peer and verify that this is not possible when the peer only offers a feature, including protocol version and combination of cryptographic algorithms, that is forbidden by the TLS profile.

Expected Results:

-	The network product under test and the peer establishes TLS if the TLS profiles used by the peer are compliant with the profile requirements in TS 33.310 [9] Annex E and RFC 7540 [11].

- 	The network product under test and the peer fail to establish TLS if the TLS profiles used by the peer are forbidden in TS 33.310 [9] Annex E or RFC 7540 [11].

Expected format of evidence:

Provide evidence of the check of the product documentation in plain text. Save the logs and the communication flow in a .pcap file.

##### 4.2.2.2.3	Authorization of NF service access

###### 4.2.2.2.3.1	Authorization token verification failure handling within one PLMN

Requirement Name: Authorization token verification failure handling within one PLMN

Requirement Reference: TS 33.501 [10], clause 13.4.1.1

Requirement Description:

According to TS 33.501 [10], clause 13.4.1.1, the NF Service producer verifies the access token as follows:

-	The NF Service producer ensures the integrity of the access token by verifying the signature using NRF’s public key or checking the MAC value using the shared secret. If integrity check is successful, the NF Service producer verifies the claims in the access token as follows:

NOTE: Void.

-	It checks that the audience claim in the access token matches its own identity or the type of NF service producer. If a list of NSSAIs or list of NSI IDs is present, the NF service producer checks that it serves the corresponding slice(s).

-	If an NF Set ID present, the NF Service Producer checks the NF Set ID in the claim matches its own NF Set ID.

- 	If the access token contains "additional scope" information (i.e. allowed resources and allowed actions (service operations) on the resources), it checks that the additional scope matches the requested service operation.

-	If scope is present, it checks that the scope matches the requested service operation.

-	It checks that the access token has not expired by verifying the expiration time in the access token against the current data/time.

-	If the verification is successful, the NF Service producer executes the requested service and responds back to the NF Service consumer. Otherwise it replies based on Oauth 2.0 error response defined in RFC 6749 [12]. The NF service consumer optionally stores the received token(s). Stored tokens may be re-used for accessing service(s) from producer NF type listed in claims (scope, audience) during their validity time.

Threat References: TR 33.926 [4], clause 6.3.3.1, Incorrect Verification of Access Tokens

Test Case:

Test Name: TC_AUTHORIZATION_TOKEN_VERIFICATION_FAILURE_ONE_PLMN

Purpose:

Verify that the NF service producer does not grant service access if the verification of authorization token from a NF service consumer in the same PLMN fails.

Procedure and execution steps:

Pre-Conditions:

-	Test environment with a NF service consumer.

-	The NF service consumer may be simulated.

-	The network product under test has already mutually authenticated with the NF service consumer.

-	The tester has access to the interface between the NF service consumer and the network product under test.

-	The tester has the NRF’s private key or the shared key.

-	The network product under test is preconfigured with the NRF’s public key or the shared key.

Execution Steps

The network product under test receives the access token sent from the NF service consumer, verifies the access token based on Oauth 2.0.

Test Cases 1~4 are tests on failure handling by the network product under test when the mandatory claims in access token failed verification.

Test Case 1: Verification failure of the access token integrity

1)	The tester computes an access token correctly, except that the signature or the MAC is incorrect, e.g., the signature or the MAC is randomly selected, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test.

2)	The integrity verification of the access token by the network product under test fails.

Test Case 2: Incorrect audience claim in the access token

1)	The tester computes an access token correctly, except that the audience claim is incorrect, i.e., the audience claim in the access token does not match the identity or the type of the network product under test, and then includes the access token in the NF Service Request sent from NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token is valid. However, the audience claim in the access token does not match its identity or type.

Test Case 3: Incorrect scope claim in the access token

1)	The tester computes an access token correctly, except that the scope is incorrect, i.e., the scope does not match the requested service operation, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token and the audience claim are  valid. However, the scope does not match the requested service operation.

Test Case 4: Expired access token

1)	The tester computes an access token correctly, except that the expiration time has expired against the current data/time, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token, the audience and scope claims are all valid. However, the expiration time in the access token has expired against the current data/time.

Test Cases 5~8 are tests on failure handling by the network product under test when the optional claims in access token failed verification.

NOTE:	The test cases below only apply to the NFs which support identifying and understanding the optioanl claims in the received access token.

Test Case 5: Incorrect list of S-NSSAIs in the access token

1)	The tester computes an access token correctly, except that the list of S-NSSAIs is incorrect, i.e., the network product under test does not serve the slices indicated in the list of S-NSSAIs, and then includes the access token in the NF Service Request sent from NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token, the audience, scope and expiration time claims are all valid. Then it further checks the list of S--NSSAIs included in the access token.

Test Case 6: Incorrect list of NSIs in the access token

1)	The tester computes an access token correctly, except that the list of NSIs is incorrect, i.e., the network product under test does not serve the slices indicated in the list of NSIs, and then includes the access token in the NF Service Request sent from NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token, the audience, scope and expiration time claims are all valid. Then it further checks the list of NSIs included in the access token.

Test Case 7: Incorrect NF Set ID in the access token

1)	The tester computes an access token correctly, except that the NF Set ID is incorrect, i.e. the NF Set ID in the claim does not match the NF Set ID of the network product under test, and then includes the access token in the NF Service Request sent from NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token, the audience, scope and expiration time claims are all valid. Then it further checks the NF Set ID included in the access token.

Test Case 8: Incorrect additional scope in the access token

1)	The tester computes an access token correctly, except that the additional scope information is incorrect, i.e. the allowed resources and allowed actions on the resources do not match the requested service operations, and then includes the access token in the NF Service Request sent from the NF service consumer to the network product under test.

2)	The network product under test verifies that the integrity of the access token, the audience, scope and expiration time claims are all valid. Then it further checks the additional scope included in the access token.

Expected Results:

For test cases 1~4 on verification failure of mandatory claims in the access token, the network product under test rejects the NF service consumer’s service request based on Oauth 2.0 error response defined in RFC 6749 [12].

For test cases 5~8 on verification failure of optional claims in the access token, if the network product under test understands these optional claims (list of S-NSSAIs, list of NSIs, NF Set ID, additional scope), it rejects the NF service consumer’s service request based on Oauth 2.0 error response defined in RFC 6749 [12].

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

##### ********** End 1st Change *************