3GPP TSG-SA3 Meeting #119	S3-245074

Orlando, US, 11 -15 November 2024

Title:	[Draft] Reply LS on enhancement to the protocol stack of IMS Data Channel

Response to:	LS S3-244646/S4-241373 on LS on enhancement to the protocol stack of IMS Data Channel from CT4

Release:	Rel-18

Work Item:	NG_RTC_SEC

Source:	SA3

To:	SA4

Cc:	SA3-LI

Contact person:	Vlasios Tsiatsis

vlasios.tsiatsis@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 would like to thank SA4 for the LS on their LS on enhancement to the protocol stack of IMS Data Channel.

SA3 would like to point out that the attached discussion paper in the incoming LS proposes security-related solutions in SA4. Security is in the remit of SA3 and it would be better that security related issues are addressed in SA3.

SA3 would to like to respond as follows to the questions posed by SA4.

Q1:	Would SA3 see any requirement of mandating the usage of IMS media plane encryption?

A1:	The existing specifications do not mandate the use of IMS media plane encryption. According to some local regulations, it is required to use encryption when applicable. However this would also need to consider any applicable LI requirements.

Q2: Could you confirm that the MNO can choose whether to encrypt IMS audio and video media during the media negotiation with UE, in IMS?

A2:	For the case of legacy IMS media the MNO can choose whether to encrypt IMS audio and video media during the UE registration in IMS.

Q3: Could the MNO choose whether to encrypt DC media during the media negotiation with UE, similar to how they can choose to encrypt IMS audio and video media?

A3:	The MNO could have the capability to choose whether to encrypt DC media during the media negotiations with the UE but DC media stack includes the DTLS layer which cannot be enabled/disabled on demand.

However, SA3 would like to observe that the question of whether the MNO can enable/disable DC media encryption is not the right one. Although SA3-LI has responded to the LS from SA4 (in s3i240475) in a similar question that "The capability to activate/deactivate IMS media plane confidentiality protection is mandatory from a lawful interception perspective." later on in the reply SA3-LI clarifies that " From a general sense, SA3-LI maintains the requirement that, when present, media shall be delivered either in unencrypted format, or with the means and instructions to decrypt the encrypted media and communications related information."

Therefore the right question to answer is whether the LI functions have access to the plaintext Content of Communication (CC). SA3 has discussed this issue and has concluded that the current specification in TS 33.328 is incomplete with respect to the necessary architecture and procedures to allow P2P communication between two UEs to fulfil the LI requirements. As a result SA3 will study this further and get back to SA4.

# 2	Actions

To SA4

ACTION: SA3 kindly requests SA4 to take the information into account and allow SA3 to study the IMS DC media security that fulfils LI requirements.

# 3	Dates of next TSG SA WG 3 meetings

SA3#120	17 – 21 February 2025		Athens (Greece)

SA3#121	7 – 11 April 2025			Goteborg, Sweden