3GPP TSG-SA3 Meeting #119	S3-245077

Orlando, US, 11 -15 November 2024

Title:	[Draft] Reply LS on clarification on home network triggered re-authentication

Response to:	LS S3-244632/C4-244496 on clarification on home network triggered re-authentication from CT4

Release:	Rel-18

Work Item:	HN_Auth

Source:	Ericsson (to be SA3)

To:	CT4

Cc:

Contact person:	Vlasios Tsiatsis

vlasios.tsiatsis@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Agreed CR in SA3

# 1	Overall description

SA3 would like to thank CT4 for their reply and observations on stage 2 specification of the Home Network initiated Authentication procedure.

As clarified in an earlier LS response, SA3 clarified that the AMF can provide an unsuccessful response to notification to trigger a re-authentication for a UE if the AMF/SEAF determines that it cannot run a primary authentication as described in step 4 (e.g., due to local policy), the AMF/SEAF does not have a valid context for the UE or the UE is under handover to another AMF.

Otherwise, the AMF shall respond with a successful response to the UDM. A basic principle followed in current stage 2 specification of the Home Network initiated Authentication procedure is that the successful response from the AMF to the UDM is to be considered as a pure acknowledgment of the reception of the notification request and an indication to the UDM that AMF will try to initiate the primary authentication procedure for the UE as soon as possible. In any case, the acknowledgment was meant to be considered in UDM as that the AMF has already initiated the procedure and ensure the UDM that the UE will be authenticated. This is the same principle followed in other similar procedures e.g. AAA-S initiated NSSAA procedure.

The situation where the AMF is not capable of triggering the authentication procedure (e.g. the user is unreachable or in power saving mode) can be detected by the UDM (e.g. if the UDM does not receive a subsequent Nudm_UEAU_ResultConfirmation operation after sending the notification for triggering primary authentication to an AMF after a e.g. pre-defined period of time or the UE registers from another AMF) and if needed UDM may send the notification request to another AMF (in the same or different access), if any.

SA3 would like to inform CT4 that the attached agreed CR clarifies the UDM and AMF behaviour upon both successful and unsuccessful situations.

Therefore, SA3 does not consider the need to provide any additional indication in the successful response from the AMF as proposed by CT4 in their LS and kindly requests CT4 to refrain from further specifying that in stage 3 specifications.

# 2	Actions

To CT4

ACTION: SA3 would like to request CT4 to take the information above into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#120	17 – 21 February 2025		Athens (Greece)

SA3#121	7 – 11 April 2025			Goteborg, Sweden