3GPP TSG-SA3 Meeting #119	S3-245209

Orlando, US, 11 - 15 November 2024

# Source:	Cisco Systems

# Title:	Conclusion for KI#1: ACME initial trust framework

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approval of proposed conclusion for KI#1 of TR 33.776

# 2	References

[1]	3GPP TR 33.776 (V0.5.0), "Study of Automated Certificate Management Environment (ACME) for the Service Based Architecture (SBA)".

[2]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF) ".

# 3	Rationale

This contribution proposes a conclusion for KI#1 in TR 33.776 [1].

# 4	Detailed proposal

* * * Start of change - all new text * * *

## 7.X		Key issue #1: ACME initial trust framework

### 7.X.1		Analysis

This key issue identifies the need to assert the certificate requesting client’s identity before issuing a security credential to the client. Solution #2 addresses this key issue. The solution uses the existing initial trust schema, as defined in TS 33.310 [3], and illustrates how the components of the initial trust schema map to the corresponding components of ACME [2].

Solution #2 minimises the impact of adding support for ACME [2] to 5G SBA. It relies on support for the third initial trust option in 33.310 [3], i.e., OAM issued signature of certain NF profile parameters. It does not support the other two options, i.e., OAM issued certificate or Initial Authentication Key (IAK); however, support for these options may be added as part of the normative phase.

However, there are messages exchanged prior to requesting and obtaining a certificate. If authentication is delayed until a challenge is issued and satisfied, an unauthenticated ACME client can potentially create account and authorization resources. It is preferable to move initial trust forward to prevent this.

Solution #Y: ACME Account Key Initial Trust Establishment, establishes initial trust at ACME account creation via ACME account key pair provisioning. This can be taken forward as the basis for work in the normative phase.

### 7.X.2		Conclusion

Normative phase can begin based on Solution #2.

Editor's Note:	Inclusion of Solution #Y, and other solutions for KI #1, are FFS.

* * * End of change * * * *