3GPP TSG-SA3 Meeting #119	S3-245349

# Orlando, USA  11 - 15 November 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Conclusion for KI#1.1

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve the conclusion proposed for KI#1.1 in this contribution.

# 2	References

[1]	3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

# 3	Rationale

SA2 in SA2#165 agreed the CR S2-2410711 to TS 23.548, defining the relevant N6 delay measurement assistance information to be provisioned via existing EDI (EAS Deployment Information) management procedure. I.e., AF provides the protocol specific details, and SMF uses that information for making decision and configuring accordingly the UPF:

| N6 delay measurement assistance information | - Includes the following parameters per DNAI and/or per EAS to enable N6 delay monitoring: measurement endpoint address information (consisting of IP address and optionally port number). - measurement protocol(s) supported by the application measurement endpoint, - protocol-specific configuration parameters. [optional]. NOTE X, NOTE Y. |
| --- | --- |

SA2 in SA2#165 also agreed the CR S2-2410937 to TS 23.502, to enable via PFCP protocol:

- as part of N4 association procedure, the SMF to configure the above information to UPF

- as part of the N4 report procedure, the UPF to report the requested info (N6 delay measurement) to SMF

Accordingly, the security requirement of KI#1.1 explicitly indicates that “5G system should provide capability to *enable* security mechanism, e.g., integrity, anti-replay protection, authentication, optionally confidentiality protection, provided by N6 delay measurement protocol(s).”

The concrete proposal is to address the issue on how to enable in 5G system the security mechanisms in the selected protocols used for N6 delay measurement between UPF and EAS. More specifically, based on SA2 agreed procedures for this topic, the enablement of the security mechanism include the configuration of security parameters for different active measurement protocols based on operator policies. Those are required to support authentication, integrity, confidentiality and anti-replay protection.

# 4	Detailed proposal

******************START of CHANGES****************************************

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.1	KI#1.1: Security aspects related to enhancements of EAS and local UPF (re)selection.

In conclusion of KI#2 of TR 23.700-49 (see 8.2 of [2]):

-	N6 delay between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) is measured by leveraging existing mechanisms (e.g. ICMP, TWAMP, OWAMP or other protocols defined by IETF).

The following guidelines from the security point of view for the N6 delay measurement between UPF PSA and EAS will be documented:

- It is recommended to use of the measurement protocols supporting authentication, integrity, and anti-replay protection.

- It is recommended not to use a delay measurement mechanism that requires sensitive information exchange but does not support confidentiality by the measurement protocols.

- It is not recommended to use a measurement protocol without authentication, integrity and anti-replay protection in place.

- It is not recommended to use a measurement protocol with disclosed vulnerabilities which may be exploited for attack or blocked by the operators.

NOTE: The enablement of the security mechanisms available in the selected measurement protocol, i.e., the configuration and provision of corresponding security parameters, makes use of existing procedures to enable N6 delay measurement defined in TS 23.548 [7] and TS 23.502 [8].

******************END of CHANGES****************************************